# 3GPP TSG-RAN WG4 Meeting #114bis                             R4-2503789

# Wuhan, China, 17th – 21st April, 2025

Source:  ZTE Corporation, Sanechips

Title:  Discussion on the RRM requirements of AI/ML Beam management

Agenda Item:	7.19.3

Document for:	Approval

- Introduction

In RAN plenary #94 meeting, a new SID[1] for Rel-18 was approved to study the AI/ML for NR air interface. The objectives for RAN4 of this SID are listed as follows.

- Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this document, we will provide some initial views on RRM requirements for beam management from the general requirements framework perspective.

- Discussion

Based on the progresses in the previous meeting, there were several typical use cases incorporated in [2], which are listed below.

Agreement: - Beam management - Spatial-domain DL beam prediction - Temporal DL beam prediction

Beam management case 1 is the spatial-domain DL beam prediction for set A of beams based on measurement result of set B beams which the set B beams is the subset beam of set A, the best beams shall be estimated through AI/ML based BM methods with L1 measurements on a beam subset as the figure shown:

Figure 1 AI beam prediction (Spatial domain)

Another sub use case is beam management case 2 which is the time-domain DL beam prediction for set A of beams based on the historic measurement results of set B of beams. The best beam in the future can be predicted through AI/ML based BM methods based on the beam quality at the current and historical time as the figure shown:

Figure 2 AI beam prediction (Time domain)

In this paper, we mainly provide the analysis on the RRM requirements for performance metrics, prediction delay and TCI state activation based on RAN1 and RAN2’s progress and agreements for BM case1 and case 2.

2.1 Performance metrics

Different metrics/KPIs have been discussed and were captured in [2], RAN4 has discussed the related performance metrics for several meetings, the following test metrics are identified and could be considered:

- Option 1: RSRP accuracy - Option 2: Beam prediction accuracy -Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" -Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" -Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, -Related measurement accuracy can be considered to determine x - Option 4: combinations of above options

We find that performance metrics mainly include two parts, which are RSRP accuracy and beam prediction accuracy. In this section, we shall analyze these two different KPIs respectively.

2.1.1 RSRP Accuracy (option 1)

RAN4 has discussed several meetings on how to define the RSRP accuracy, including the absolute RSRP accuracy and the relative RSRP accuracy. In RAN4 #112bis meeting, the absolute RSRP accuracy has already been defined as below:

Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

Based on the above agreement, we shall distinguish what is the benchmark of top-K beams. Since the formula only tells us the index i may be any beam index and not sure the index of top-K beams shall be predicted beams or groundtruth beams. There are two choices which are the top-K beams based on groundtruth L1-RSRP and the top-K beams based on predicted L1-RSRP. In my opinion, the top-K beams shall be the predicted beams instead of the groundtruth beams. Since the purpose of the absolute RSRP accuracy is to verify the performance of AI model based on the predicted beams or UE reported several optimal predicted beams. Thus, RAN4 shall check the reported predicted beams with groundtruth beams whether the reported top-K beams are predicted better or worse. For instance, the top-4 reported beam indices are beam 1, beam 2, beam 3 and beam 4, the L1-RSRP sequence of groundtruth beams indices are beam 1, beam 4, beam 5, beam 6, beam 3, beam 2 and others. If the index equals 1, the absolute RSRP accuracy may be small, however, if the index equals 2, the absolute RSRP accuracy may be large. Then, it means that the performance of AI model is not good enough. On the contrary, for the top-K beams are groundtruth beams but not the predicted beams, may be some UE reported beams could not be verified or why should we require the RSRP accuracy of the beam that the UE doesn’t even report.

On the other hand, if the index equals 1, the absolute RSRP accuracy may be small, however, if the index equals 2, the absolute RSRP accuracy may be large. Then, it means that the performance of AI model is not good enough. What is the standard to verify the performance of AI system based on the absolute RSRP accuracy shall be considered or how to quantify the the small or large L1-RSRP difference shall be studied. From my perspective, a margin could be considered, that is, if the L1-RSRP difference of same beam exceeds the margin, the difference is large. Otherwise, the difference is small. However, the absolute RSRP accuracy in legacy can be seen as below:

Table 1: SSB based L1-RSRP absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| ±6.5 | ±9.5 | ≥-3 | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 8.5 | 11.5 | ≥-3 | N/A |  | -70 | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

One question is that the proposed margin shall be maintained or smaller than legacy value. There are two options to consider the change of proposed margin:

- Option 1: Maintain the legacy measurement accuracy. In legacy, UE shall do the full beam sweeping in order to confirm the best Rx beam to receive the downlink RS and get the better communication with network. The AI system has already improve the overhead and reduce the complexity than legacy. Thus, maintain the legacy measurement accuracy could be understood.

- Option 2: Smaller than legacy measurement accuracy. This is a more direct understanding since the AI performance shall have the better performance than legacy and the measurement accuracy shall be tightened. In this way, the performance of AI method could be displayed.

To my understanding, option 1 and option 2 are fine for us, the proposed margin could be equal to and smaller than the legacy measurement accuracy requirements.

- Observation 1: The top-K beams are reported or predicted beams or top-K beams are groundtruth beams.

- Observation 2: For the top-K beams are groundtruth beams but not the predicted beams, may be some UE reported beams could not be verified or why should we require the RSRP accuracy of the beam that the UE doesn’t even report.

- Observation 3: What is the standard to verify the performance of AI system based on the absolute RSRP accuracy shall be considered or how to quantify the the small or large L1-RSRP difference shall be studied.

Observation 4: One question is that the proposed margin shall be maintained or smaller than legacy value. There are two options to consider the change of proposed margin:

- Option 1: Maintain the legacy measurement accuracy. In legacy, UE shall do the full beam sweeping in order to confirm the best Rx beam to receive the downlink RS and get the better communication with network. The AI system has already improve the overhead and reduce the complexity than legacy. Thus, maintain the legacy measurement accuracy could be understood.

- Option 2: Smaller than legacy measurement accuracy. This is a more direct understanding since the AI performance shall have the better performance than legacy and the measurement accuracy shall be tightened. In this way, the performance of AI method could be displayed.

- Proposal 1: For the absolute RSRP accuracy, the index i shall be the top-K beams based on the predicted beams or UE reported beams instead of the groundtruth beams.

- Proposal 2: One margin for the absolute RSRP accuracy shall be studied in order to verify the performance of AI system. The proposed margin could be equal to and smaller than the legacy measurement accuracy requirements.

However, the relative RSRP accuracy has not be defined during several meetings. First of all, we shall clarify the necessity of using relative RSRP accuracy to determining the optimal reported beams. Secondly, the formula of relative RSRP accuracy shall be defined.

Before the above analysis, we shall specify what is the relative RSRP accuracy. The legacy description can be seen as below:

Legacy including ICBM related: The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the cell (serving cell or cell with different PCI from serving cell) on which UE performs L1-RSRP measurements. LTM related: The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured on one SSB configured in LTM-CSI-ResourceConfig compared to the L1-RSRP measured from another SSB configured in LTM-CSI-ResourceConfig from the cell(s) on the same frequency in FR2.

We know that the benchmark is the measured largest value of L1-RSRP among all SSBs of the serving cell or one neighbour cell in legacy. However, the LTM related relative RSRP has no benchmark, only two different beams do the comparison but in two different cells. We deem that these two kinds of definition could be considered when we discuss the formulation of the relative RSRP accuracy.

Necessity of using relative RSRP accuracy

- Remove the bias. RSRP measurement may be impacted by path loss, multi-path effect and LOS/NLOS etc., which the RSRP measurement results will contain the random bias. The relative RSRP accuracy refers to difference between two different SSBs (including the largest value), that is, the relative RSRP accuracy gives the differential results and the bias between different beams could be cancel out since the different beams may have the similar bias impact unlike the absolute RSRP accuracy.

- Choose the optimal reported beam. The relative RSRP accuracy allows the network to compare the received signal power from different beams more accurately by comparing the relative differences among them. It aids the UE and the network in determining the optimal reported beam.

Based on the above, we think the relative RSRP accuracy shall be considered.

Observation 5: The relative RSRP accuracy could remove the bias and choose the optimal reported beam.

Proposal 3: The relative RSRP accuracy shall be considered for RSRP accuracy.

Formula of the relative RSRP accuracy

From my perspective, there is no neighbour cell impact for AI beam management, only serving cell downlink Tx beam prediction is in the scope. Thus, the LTM and ICBM related relative RSRP accuracy shall be precluded and only legacy definition of relative RSRP accuracy with serving cell shall be baseline.

Some companies deem that the relative RSRP accuracy shall between the best predicted beam and one of the rest of predicted beams. To my understanding, the extreme case is that L1-RSRP of the best predicted beam (benchmark) is not good enough, that is, the performance of AI system has already been degraded. Thus, the predicted beam could not be the benchmark. On the contrary, the groundtruth beams are based on the UE measurement which is more reliable than predicted beams in the initial prediction stage. Thus, the best groundtruth beams shall be the benchmark. The formula of the relative RSRP accuracy could be :

Relative RSRP accuracy=predicted L1-RSRP of beam index i - ground truth of the beam index n ,

where the beam index n owns the largest value

Observation 6: For AI beam management, only serving cell downlink Tx beam prediction is in the scope.

Observation 7: The groundtruth beams are based on the UE measurement which is more reliable than predicted beams.

Observation 8: The largest ground truth shall be as the benchmark for definition of relative RSRP accuracy.

Proposal 4: The formula of relative RSRP accuracy could be:

Relative RSRP accuracy=predicted L1-RSRP of beam index i - ground truth of the beam index n ,

where the beam index n owns the largest value

2.1.2 Beam Prediction Accuracy (option 2)

Option 2 has three sub-options including Top-1(%), Top-K/1(%) and Top-1/K(%), we shall study these three KPIs one by one and decide whether to use them or not.

- Top-1(%): the predicted best beam is the best ground truth beam. From my perspective, this metric is a little bit strict. Since some companies provide the evaluation that the value between the best ground truth beam and the second best beam is very close that UE may not distinguish both of them especially considering the BB and RF error. For instance, the best predicted beam ID is beam 1, the best groundtruth beam ID is beam 1 and the second best groundtruth beam ID is beam 2, the RSRP difference between both of them is extremely small, UE will wrongly recognize the best groundtruth beam as beam 2, then the UE fails the test. Some companies will ask if the RSRP difference is very closed, it has little impact on the prediction accuracy. We also agree with it, but this metric is related to beam ID and the description show us Top-1 strongest beam ID not the best RSRP accuracy. During the test, the test may be failed because of smallest prediction errors. And the other unreported beams could not be guaranteed. To my understanding, this metric shall be set as low priority.

- Top-K/1(%): the best ground truth beam is one of the Top-K predicted beams. The same situation may be occurred as Top-1(%), the value between the best ground truth beam and the second best beam is very close that UE may not distinguish both of them especially considering the BB and RF error. For example, the Top-2 predicted beams are 1, 2, the best and the second best ground truth beams are 1, 5 and the value difference is 0.2dB. UE may wrongly chooses the best ground truth beam as beam 5, beam 5 is not in the Top-2 beams, then the UE fail the test and the AI model will be updated. This will cause the mistake because of the small difference value between the best and the second best ground truth beams. Thus, this metric could not be used alone, which shall combine with the other metrics like RSRP accuracy. First of all, UE would pass the test if the best ground truth beam is in Top-K predicted beams. If not, the relative RSRP accuracy metric shall be considered or used, which the worst reported beam shall compared to the best ground truth beam. If the RSRP difference is small than the legacy margin or new proposed margin, it means that the UE could pass the test. Also, if the number of K is larger and larger, the probability of the best ground truth beam is in the top-K beams is larger and larger.

- Top-1/K(%): the best predicted beam is one of the Top-K ground truth beam. One issue we may face is that this metric only consider the predicted best beam, but the other predicted beams are not guaranteed. For instance, if the reported Top-3 predicted beams are beam 1, beam 7 and beam 8, the Top-4 ground truth best beams are beam 1, beam 2, beam 3 and beam 4, where the beam 7 and beam 8 are the last two beams in groundtruth beams, we can see that the UE could pass this test based on this performance metric. The best beam only guarantee the current communication, switching to the wrong alternative reported beam may cause communication interruption when the optimal beam quality decreases or is lost (such as due to occlusion or interference). Although the main purpose for AI/ML beam is to select the best beam, the whole system may not work in the field and the whole system will not be ensured. This metric shall be set as low priority.

Actually, the most companies point out that Top-1/1 is too strict to pass the test. For that metric, we deem that K=1 and N=1. Then, we could relax that metric to Top-K/1, where K>1 and N=1. if K is larger and larger, the prediction accuracy is more and more accurate. We deem that the K could be relaxed from K=1 to K≥3. Next way is to relax N, that is, Top-1/N which is the same as Top-1/K. We deem that Top-1/K has its own limitation.

Observation 9: For Top-1(%), the test may be failed because of smallest prediction errors.

Observation 10: For Top-K/1(%), this metric will cause the mistake because of the small difference value between the best and the second ground truth beams. If the number of K is larger and larger, the probability of the best ground truth beam is in top-K predicted beams is larger and larger.

Observation 11: For Top-1/K(%), this metric only consider the predicted best beam, but the other predicted beams are not guaranteed, the whole system may be degraded.

Proposal 5: For beam prediction accuracy, Top-1(%) and Top-1/K(%) shall be set as low priority.

Proposal 6: For Top-K/1(%), the K shall be relaxed to K≥3.

Proposal 7: The Top-K/1(%) could be used together with the RSRP accuracy to verify the performance of AI system.

2.1.3 Beam Prediction Accuracy (option 3)

Option 3 is defined as ,”The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam x dB.”

Figure 3  Option 3 for beam management

Assuming the left one in figure 2 is the Top-5 predicted beams and the maximum RSRP among top-5 predicted beams is beam 4. The right one in figure is the Top-5 legacy/measured beams and the strongest RSRP is the red one which is the beam 1. The blue one is the reference RSRP value of beam 4. The understanding for option 3 is that the AI/ML model predicted beam 4 which has the strongest RSRP prediction value and corresponding reference RSRP value shall be larger than the strongest reference RSRP value (beam 1) minus x dB:

Beam 4 in top-5 legacy/measured beams > Beam 1 in top-5reference beams – x dB,

then the test will pass.

The following instance can be as reference, assuming:

RSRP of predicted beams: beam 1 = -79dBm, beam 2 = -81dBm, beam 3 = -90dBm,

RSRP of reference beams: beam 1 = -80dBm, beam 2 = -79dBm, beam 3 = -91dBm,

if Top-K = Top-1, the UE would like to report beam 1 which has the largest RSRP predicted value. As this interpretation, the RSRP difference is between measured beam 1 and the measured beam 2, that is, beam 2-beam 1<x dBm. If the RSRP difference is assumed as 3dBm, the test will pass since the RSRP difference between reference beam 1 and beam 2 is 1dBm which is smaller than the threshold. One issue we may face is that this metric only considers the predicted best beam, but the other predicted beams are not guaranteed. Although the main purpose for AI/ML beam is to select the best beam, the whole system may not work in the field if the other predicted beams are not verified and the whole system will not be ensured same as Top-1/K(%).

- Observation 12: Option 3 only considers the predicted best beam, but the other predicted beams are not guaranteed. Although the main purpose for AI/ML beam is to select the best beam, the whole system may not work in the field and the whole system will not be ensured.

- Proposal 8: At least the option 3 could not be precluded which could be set as low priority.

2.2 Prediction delay

In the last meeting, prediction delay requirements have been discussed without any agreements as below:

Agreement: - Prediction delay includes: - Measurement for prediction - FFS on whether the legacy measurement delay can be reused - Inference delay - Other components are FFS

Before we RAN4 defining the prediction delay requirements, the components of prediction delay shall be firstly decided, and then the specific requirements could be considered.

Potential components in prediction delay

To my understanding, the following figure could show us the concrete components in prediction delay:

Figure 4 Prediction delay

Based on figure 3, we think that the whole processing delay contains three parts, the starting point is collection of input data and the ending point is UE reporting the predicted results to gNB:

- Measurement delay: The time for measurement of SetB which is the input for inference.

- Inference delay: The time for inference from starting performing inference to generate the prediction results.

- Reporting delay: The time between the report triggering and the point when the UE starts to transmit the outputs over the air..

Proposal 9: The potential components in prediction delay contains three parts:

- Measurement delay: The time for measurement of SetB which is the input for inference.

- Inference delay: The time for inference from starting performing inference to generate the prediction results.

- Reporting delay: The time between the report triggering and the point when the UE starts to transmit the outputs over the air.

How to define the prediction delay requirements

We have studied the whole AI beam prediction delay which contains three parts. Actually, we shall define the separate delay requirements and then combine with them to confirm the whole AI beam prediction delay.

However, there is an agreement related to inference delay in RAN4 #114 meeting as below:

Agreement: - Use the following as the starting point - Inference latency can be implicitly captured in the delay of corresponding procedures (e.g. reporting delay) and/or performance requirements. - Inference latency is part of delay of the corresponding procedure and no separate requirement will be specified for inference latency. - Check the above bullets based on per use case, and if other WGs define the inference latency, RAN4 can revisit the above bullets.

It is observed that inference latency will be implicitly captured in the delay of corresponding procedures and RAN4 shall not define the separate requirements for inference latency. RAN1 has not define the inference latency so far, so RAN4 shall define other requirements including inference latency implicitly. The above agreement suggests that the inference latency could be captured in reporting delay, so figure 3 shall be modified as below:

Figure 5 Modified prediction delay

For measurement delay, the legacy requirements is below:

Table 2 Measurement period for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Based on the current spec, the legacy L1-RSRP measurement period is based on one-shot measurement. One shot measurement means the number of measured sample is one. For L1-RSRP measurement period, the number of M equals 1 or 3, where M=3 includes two samples to do the AGC and only one sample is used for measurement. The L1-RSRP measurement period owns sharing factor ”P”, number of sample “M” and Rx beam sweeping factor “N” in FR2. This measurement is about SetB measurement which is legacy L1-RSRP measurement, we propose to use the legacy measurement period as baseline or starting point.

For reporting delay (including inference latency), RAN4 shall wait for other WGs’ discussion on the concrete reporting mechanism.

Proposal 10: The legacy L1-RSRP measurement period shall be as baseline or starting point.

Proposal 11: Postpone discussion on reporting requirements until other WGs reach the conclusion on reporting mechanism.

2.3 TCI state activation

For AI beam prediction, UE firstly performs the L1-RSRP measurement for Set B as the input of AI model. The AI model generates the output which is the predicted Set A, and the AI model is the UE-side model. UE would report the final output to network, then network configures TCI state activation. RAN4 actually defines the active TCI state switching delay based on known TCI state or unknown TCI state in legacy, that is, different TCI state conditions mean the different TCI state switching delay. The legacy known conditions for TCI state could be seen as below:

The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state -TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement -The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command -The TCI state remains detectable during the TCI state switching period -The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3dB Otherwise, the TCI state is unknown.

There are two situation for considering the TCI state activation:

- Case 1: The best predicted Tx beam ID/RSRP belongs to Set B.

- Case 2: The best predicted Tx beam ID/RSRP doesn’t not belong to Set B.

To my understanding, the known and unknown conditions for Rx beam to UE will have an impact on TCI state switching delay. When the UE is aware of the current RX beam it is utilizing, it can more swiftly provide feedback on CSI, particularly when it can promptly report the optimal TCI state when channel conditions change. In this scenario, the network can immediately apply the recommended TCI state for beamforming adjustments, thereby reducing the TCI state activation delay. In contrast, if the UE does not precisely know its RX beam or if the channel state has changed, it may require additional time to measure various beam configurations to determine which TCI state best suits the current conditions. This results in increased TCI state activation latency because the system must wait for the UE to complete measurements and report the optimal selection.

Thus, we deem that the known condition on TCI state switching delay shall be based on the legacy delay and the measured Rx beam or not. For case 1, the best predicted Tx beam belongs to Set B and UE performs the full Rx beam sweeping for Set B, the TCI state is known. For case 2, the best predicted Tx beam doesn’t belong to Set B and the corresponding Rx beam of the best Tx beam may be unknown to UE, the TCI state is unknown.

Observation 13: Different TCI state conditions mean the different TCI state switching delay.

Observation 14: When the UE is aware of the current RX beam it is utilizing, TCI state activation delay is reduced. Otherwise, TCI state activation delay is not reduced.

Observation 15: There are two situation for considering the TCI state activation:

- Case 1: The best predicted Tx beam ID/RSRP belongs to Set B.

- Case 2: The best predicted Tx beam ID/RSRP doesn’t not belong to Set B.

Proposal 12: UE knows the Rx beam or the RS is in the Set B and the RS is also the L1-RSRP report by UE within 1280ms before TCI state activation, the TCI state is known. Otherwise, the TCI state is unknown.

2.4 Test Procedure

When we study the testability issues, the test procedure shall be clarified and unified firstly. In this section, we would like to state the test procedure as below:

| Training phase | 1.TE configure resource sets for measurement of Set B and beam report of Set A |
| --- | --- |
|  | 2.UE measures Set B as the model input. |
| Inference phase | 3.Generates the test setup environment for Set B. |
|  | 4.UE predicts best Tx beam/ L1-RSRP in Set A. |
|  | 5.UE reports predicted best Tx beam/ L1-RSRP to TE in Set A. |
| Groundtruth phase (measurement of RSRP with high SNR) | 6.TE configures resource sets for measurement of Set A. |
|  | 7.Generates the test setup environment for Set A (including high SNR). |
|  | 8.UE measures Set A under high SNR condition as ground truth. |
|  | 9.UE reports groundtruth (measurement on Set A) to TE. |
| TE evaluation phase | 10.Evaluates the predicted results and groundtruth and judge the performance degradation |

- Proposal 13: The test procedure shall be clarified and unified firstly:

| Training phase | 1.TE configure resource sets for measurement of Set B and beam report of Set A |
| --- | --- |
|  | 2.UE measures Set B as the model input. |
| Inference phase | 3.Generates the test setup environment for Set B. |
|  | 4.UE predicts best Tx beam/ L1-RSRP in Set A. |
|  | 5.UE reports predicted best Tx beam/ L1-RSRP to TE in Set A. |
| Groundtruth phase (measurement of RSRP with high SNR) | 6.TE configures resource sets for measurement of Set A. |
|  | 7.Generates the test setup environment for Set A (including high SNR). |
|  | 8.UE measures Set A under high SNR condition as ground truth. |
|  | 9.UE reports groundtruth (measurement on Set A) to TE. |
| TE evaluation phase | 10.Evaluates the predicted results and groundtruth and judge the performance degradation |

2.5 Measurement Error

In the last meeting, RAN4 has agreed the simulation assumption for studying the impact of measurement error on beam prediction accuracy. There are two steps for beam simulation:

- Measurement error for different SNR

- Prediction accuracy without measurement error and without measurement error

For the first part, the measurement error for different SNR is simulated as below. Here, we only simulate for AWGN channel. For fading channel, the measurement error may be larger. Besides, measurement error is dependent on baseband algorithm.

Measurement error for AWGN channel

| SNR | -5dB | -3dB | 0dB | 5dB |
| --- | --- | --- | --- | --- |
| L1-RSRP measurement error (dB) | 1.3 | 1 | 0.6 | 0.3 |

- Observation 16: The smaller SNR conditions, the larger measurement error.

- Secondly, we would apply the above measurement error to all Tx beams, where measurement error is modeled as Gaussian distribution. The following figure[3] provides us the concrete method to add the measurement error:

- Figure 6 Add measurement error

- We main provide the performance metrics Top-1 and Top-K/1 with ME and without ME as below:

- Top-1:

| - Measurement error (dB) | - Prediction accuracy (%) |
| --- | --- |
| - No error | - 85.48 |
| - 0.3 | - 77.22 |
| - 0.6 | - 74.04 |
| - 1 | - 68.08 |
| - 1.3 | - 65.10 |

- Top-K/1:

| Measurement error (dB) | Prediction accuracy % |  |  |  |
| --- | --- | --- | --- | --- |
|  | K=2 | K=3 | K=4 | K=5 |
| No error | 94.02 | 96.96 | 98.20 | 98.66 |
| 0.3 | 92.18 | 95.58 | 97.14 | 98.14 |
| 0.6 | 90.10 | 94.50 | 96.76 | 97.66 |
| 1 | 87.34 | 92.98 | 95.58 | 96.82 |
| 1.3 | 85.42 | 91.96 | 95.04 | 96.62 |

- Based on the above prediction accuracy results on Top-1 and Top-K/1, we find that there will be performance degradation due to measurement error. For Top-1, the prediction accuracy is below 86% even if there is no error on test data. When measurement error is 1.3dB, the prediction accuracy of Top-1 is 65.10% which has severe degradation. Thus, this metric could not be considered. On the contrary, for the metric Top-K/1 when K>1, the prediction is getting larger and larger when K is larger and larger whatever the measurement error is added or not. To my understanding, this metric shall be considered as baseline based on simulation results.

- Observation 17: The performance degradation will be occurred due to added measurement error.

- Observation 18: For Top-1, the prediction accuracy is below 86% even if there is no error on test data.

- Observation 19: The prediction is getting larger and larger when K is larger and larger whatever the measurement error is added or not.

- Proposal 14: RAN4 shall further study the measurement error impact on different performance metrics if new metrics are introduced.

- Proposal 15: Except measurement error impact, RF error influence shall also be considered in RAN4.

- Conclusion

In this contribution, we have the following observations and proposals for the AI/ML beam management:

- Observation 1: The top-K beams are reported or predicted beams or top-K beams are groundtruth beams.

- Observation 2: For the top-K beams are groundtruth beams but not the predicted beams, may be some UE reported beams could not be verified or why should we require the RSRP accuracy of the beam that the UE doesn’t even report.

- Observation 3: What is the standard to verify the performance of AI system based on the absolute RSRP accuracy shall be considered or how to quantify the the small or large L1-RSRP difference shall be studied.

Observation 4: One question is that the proposed margin shall be maintained or smaller than legacy value. There are two options to consider the change of proposed margin:

- Option 1: Maintain the legacy measurement accuracy. In legacy, UE shall do the full beam sweeping in order to confirm the best Rx beam to receive the downlink RS and get the better communication with network. The AI system has already improve the overhead and reduce the complexity than legacy. Thus, maintain the legacy measurement accuracy could be understood.

- Option 2: Smaller than legacy measurement accuracy. This is a more direct understanding since the AI performance shall have the better performance than legacy and the measurement accuracy shall be tightened. In this way, the performance of AI method could be displayed.

- Proposal 1: For the absolute RSRP accuracy, the index i shall be the top-K beams based on the predicted beams or UE reported beams instead of the groundtruth beams.

Proposal 2: One margin for the absolute RSRP accuracy shall be studied in order to verify the performance of AI system. The proposed margin could be equal to and smaller than the legacy measurement accuracy requirements.

Observation 5: The relative RSRP accuracy could remove the bias and choose the optimal reported beam.

Proposal 3: The relative RSRP accuracy shall be considered for RSRP accuracy.

Observation 6: For AI beam management, only serving cell downlink Tx beam prediction is in the scope.

Observation 7: The groundtruth beams are based on the UE measurement which is more reliable than predicted beams.

Observation 8: The largest ground truth shall be as the benchmark for definition of relative RSRP accuracy.

Proposal 4: The formula of relative RSRP accuracy could be:

Relative RSRP accuracy=predicted L1-RSRP of beam index i - ground truth of the beam index n ,

where the beam index n owns the largest value

Observation 9: For Top-1(%), the test may be failed because of smallest prediction errors.

Observation 10: For Top-K/1(%), this metric will cause the mistake because of the small difference value between the best and the second ground truth beams. If the number of K is larger and larger, the probability of the best ground truth beam is in top-K predicted beams is larger and larger.

Observation 11: For Top-1/K(%), this metric only consider the predicted best beam, but the other predicted beams are not guaranteed, the whole system may be degraded.

Proposal 5: For beam prediction accuracy, Top-1(%) and Top-1/K(%) shall be set as low priority.

Proposal 6: For Top-K/1(%), the K shall be relaxed to K≥3.

Proposal 7: The Top-K/1(%) could be used together with the RSRP accuracy to verify the performance of AI system.

- Observation 12: Option 3 only considers the predicted best beam, but the other predicted beams are not guaranteed. Although the main purpose for AI/ML beam is to select the best beam, the whole system may not work in the field and the whole system will not be ensured.

Proposal 8: At least the option 3 could not be precluded which could be set as low priority.

Proposal 9: The potential components in prediction delay contains three parts:

- Measurement delay: The time for measurement of SetB which is the input for inference.

- Inference delay: The time for inference from starting performing inference to generate the prediction results.

- Reporting delay: The time between the report triggering and the point when the UE starts to transmit the outputs over the air..

Proposal 10: The legacy L1-RSRP measurement period shall be as baseline or starting point.

Proposal 11: Postpone discussion on reporting requirements until other WGs reach the conclusion on reporting mechanism.

Observation 13: Different TCI state conditions mean the different TCI state switching delay.

Observation 14: When the UE is aware of the current RX beam it is utilizing, TCI state activation delay is reduced. Otherwise, TCI state activation delay is not reduced.

Observation 15: There are two situation for considering the TCI state activation:

- Case 1: The best predicted Tx beam ID/RSRP belongs to Set B.

- Case 2: The best predicted Tx beam ID/RSRP doesn’t not belong to Set B.

Proposal 12: UE knows the Rx beam or the RS is in the Set B and the RS is also the L1-RSRP report by UE within 1280ms before TCI state activation, the TCI state is known. Otherwise, the TCI state is unknown.

- Proposal 13: The test procedure shall be clarified and unified firstly:

| Training phase | 1.TE configure resource sets for measurement of Set B and beam report of Set A |
| --- | --- |
|  | 2.UE measures Set B as the model input. |
| Inference phase | 3.Generates the test setup environment for Set B. |
|  | 4.UE predicts best Tx beam/ L1-RSRP in Set A. |
|  | 5.UE reports predicted best Tx beam/ L1-RSRP to TE in Set A. |
| Groundtruth phase (measurement of RSRP with high SNR) | 6.TE configures resource sets for measurement of Set A. |
|  | 7.Generates the test setup environment for Set A (including high SNR). |
|  | 8.UE measures Set A under high SNR condition as ground truth. |
|  | 9.UE reports groundtruth (measurement on Set A) to TE. |
| TE evaluation phase | 10.Evaluates the predicted results and groundtruth and judge the performance degradation |

- Observation 16: The smaller SNR conditions, the larger measurement error.

- Observation 17: The performance degradation will be occurred due to added measurement error.

- Observation 18: For Top-1, the prediction accuracy is below 86% even if there is no error on test data.

- Observation 19: The prediction is getting larger and larger when K is larger and larger whatever the measurement error is added or not.

- Proposal 14: RAN4 shall further study the measurement error impact on different performance metrics if new metrics are introduced.

- Proposal 15: Except measurement error impact, RF error influence shall also be considered in RAN4.

- Reference

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP RAN Plenary

- R4-2420513, “WF on RAN4 requirements for NR AI/ML”, Qualcomm Incorporated

- R4-2417890, “Discussion on core requirements of AI/ML BM use case”, Media Tek Inc.