# 3GPP TSG-RAN WG4 Meeting #114 bis                              R4-2503791

# Wuhan, China, 7th – 11st April, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM requirements for Ambient-IoT

Agenda item:	7.24.3

Document for:	Approval

Introduction

At the RAN#106 meeting, the revised WID on Ambient-IoT [1] has been approved. In this meeting, we will discuss the scenarios and provide our initial views on potential RRM impact. We could see the following description of the latest WID:

Ambient-IoT RAN4 scope: - RAN4 scope: - Specify RF requirements for Ambient-IoT BS, device 1, and CW - RF requirements for Type 1-C Ambient-IoT BS - RF requirements for device 1 - RF requirements for CW - Specify RRM core requirements for device 1, if necessary - Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

Discussion

RAN1 and RAN2 have already discussed Ambient-IoT with several meetings. Compared to legacy NR functionalities, the supported and not supported features could be seen as below:

As to the A-IoT required functionalities, the following functionalities are supported: - A-IoT paging (see clause 6.3.3) - A-IoT random access procedure (see clause 6.3.4) - A-IoT data transmission (see clause 6.3.5) As to the A-IoT required functionalities, at least the following functionalities are NOT supported (see TS 38.300 [153] for references for any legacy NR functionality): - AS security (The AS layer design assumes no support of AS security, unless the study in [152] further concludes differently.) - RRC states - RRC connection management - RRM L3 measurement reporting - Mobility - ASN.1 encoding/decoding - Periodical system information and MIB - Tracking/RAN area update procedure - Per-packet QoS and per-QoS flow at AS level - HARQ - RLC ARQ/AM - AS-layer (above physical layer) RLC-like/ARQ-like retransmission - AS-layer (above physical layer) repetition NOTE 1: It is not precluded that the reader and A-IoT device send the "payload" again as new transmission from A-IoT MAC perspective. - Multiple A-IoT logical channels for upper layer data - NR SR, as in TS 38.300 [153] - NR BSR, as in TS 38.300 [153]

Based on the table above, we conclude that the legacy RRC layer is not supported, including RRC states, RRM L3 measurement reporting, and mobility operations (such as cell re-selection/handover).etc. Regarding the Ambient-IoT required functionalities, only A-IoT paging, A-IoT random access procedure, and A-IoT data transmission are supported. Hence, it can be stated that the functionalities supported at this stage are as follows:

Figure 1 Overall AS procedures between A-IoT device and reader

For another point, the timing relationship between R2D transmission and D2R transmission has already been studied in  RAN1 during SI phase and some basic definition has been captured in [2].

A-IoT processing time aspects are studied in terms of the following timing relationships: TR2D_min: Minimum time between a R2D transmission and the corresponding D2R transmission following it. TD2R_min: Minimum time between a D2R transmission and the corresponding R2D transmission following it. TD2R_max: Maximum time between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max], when a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device. See clause 6.3 for message descriptions. TR2D_R2D_min: Minimum time between two different consecutive R2D transmissions to the same A-IoT device. TD2R_D2R_min: Minimum time between two different consecutive D2R transmissions from the same A-IoT device. For the time interval between a R2D transmission and the corresponding D2R transmission following it, there are two options studied: Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D ≥ TR2D_min.

To our understanding, looking at the whole procedure, the timing issue should be studied, including both downlink timing and uplink timing synchronization or transmission latency between R2D and D2R transmission.

Observation 1: The legacy RRC layer is not supported, including RRC states, RRM L3 measurement reporting and mobility etc.

Proposal 1: Only the RRM impact on supported A-IoT functionalities shall be discussed.

Proposal 2: The timing issue should be studied, including both downlink timing and uplink timing synchronization or latency between R2D and D2R transmission.

In this paper, we would like to mainly study the RRM impact on A-IoT random access procedure and timing issues.

2.1 A-IoT Random Access Procedure

Based on RAN1 and RAN2’s discussion, the A-IoT random access procedure is used for the Ambient IoT devices to access the network for subsequent data transmission, and the A-IoT random access procedure is triggered by the reader. Similar to the legacy NR random access procedure, A-IoT random access includes both contention-free and contention-based access mechanisms, but the exact procedures are different from legacy NR random access procedure. We would like to analyze the potential RRM impact on random access in [3] based on the existing A-IoT random access procedure in [2].

2.1.1 Contention based random access procedure (CBRA)

In [2], there are two kinds of CBRA procedure are supported, and RAN2 has the agreements as below:

RAN2 #129 meeting: - Both CFRA and CBRA will be supported. - Only 3-step RA will be supported for CBRA in Rel-19.

Consequently, 3-step CBRA procedure shall be only considered in this stage. The main description on 3-step CBRA could be seen as below:

Solution 1: A-IoT Msg1 without data - A-IoT Msg1: When the A-IoT device identifies the start of its own access occasion, it sends one 16-bit random ID generated by the A-IoT device to the reader. - A-IoT Msg2: The reader responds with the successfully received random ID. If the A-IoT device receives the A-IoT Msg2 including a random ID, which is the same as the previously transmitted one in A-IoT Msg1, it considers the contention resolution as successful.

RAN4 shall study RRM impact on random access based on 3-step CBRA and CFRA. The following figure primarily refer to Ambient-IoT 3-step CBRA:

Figure 2  Ambient-IoT 3-step CBRA procedure

We find that the Ambient-IoT 3-step CBRA procedure is entirely distinct from NR legacy CBRA. For Ambient-IoT random access, contention resolution occurs during the process of Msg2, and Msg1 solely carries random ID without data.

Observation 2: Only 3-step CBRA procedure is agreed for Ambient-IoT.

Observation 3: Ambient-IoT 3-step CBRA procedure is distinct from NR legacy 4-step and 2-step CBRA procedure.

Furthermore, device behaviour is totally distinct from legacy UE behaviour due to varying random access procedure. Consequently, defining the correct behaviour is essential. In current spec [3], we outline the requirements for the UE regarding random access procedure. By analogy with the defined format in clause 6.2.2 [3], it can be seen that at least the following correct behaviour shall be studied:

Ambient-IoT CBRA: Correct behaviour when transmitting Msg1 (Random ID) Correct behaviour when receiving Msg2 (Response Msg1) Correct behaviour when not receiving Msg2 (Response Msg1)

Correct behaviour when transmitting Msg1

All devices would monitor paging message broadcasted by reader. Then, target devices will send Msg1 with random ID to reader. In [2], some notes can be seen as below:

NOTE 1: The random ID is randomly generated by the A-IoT device. NOTE 2: The A-IoT Msg2 is used for contention resolution, since it is assumed that the size of random ID in A-IoT Msg1 should be sufficient for contention resolution purpose. It is assumed that, with the sufficient range for random ID, the probability of multiple A-IoT devices selecting the same access occasion and the same random ID for A-IoT Msg1 transmission will be sufficiently low.

We conclude that the random ID is randomly generated by the target A-IoT devices instead of reader’s configuration. For correct behaviour when transmitting Msg1, target devices only generate random ID randomly during (TR2D_min, [TR2D_max]) which is unlike the legacy NR UE behaviour.

Observation 4: The random ID is randomly generated by the A-IoT devices.

Observation 5: The probability of multiple A-IoT devices selecting the same access occasion and the same random ID for A-IoT Msg1 transmission will be sufficiently low.

Correct behaviour when receiving Msg2

Directly after Msg1, target devices receive Msg2 from reader, random ID may match with previously transmitted one in Msg1 or may not match. Target devices have different behaviour because of those two scenarios.

If the random ID in Msg2 matches with previously transmitted one in Msg1, target devices may stop monitoring for Msg2 and transmit the Msg3 with the device ID and/or any other upper layer data during (TR2D_min, [TR2D_max]). To our understanding, this correct behaviour is similar to legacy UE correct behaviour.

In [2], the following description can be seen:

It is supported for the A-IoT device to re-access in another opportunity controlled/provided by the reader (i.e., to retry the random access above), in case of D2R data transmission failure and contention resolution failure of contention-based random access.

We know that if D2R data transmission failure (Msg1)and contention resolution failure of contention-based random access (Msg2), it is supported for A-IoT device to re-access controlled by reader.

Consequently, if the random ID in Msg2 doesn’t match with previously transmitted one in Msg1, A-IoT device shall perform re-access procedure and the A-IoT devices are not expected to autonomously re-access. The re-access is always controlled by reader. However, the details on how reader control the re-access procedure is still under discussion by other working group.

Observation 6: If the random ID in Msg2 matches with previously transmitted one in Msg1, the correct behaviour is similar to legacy UE correct behaviour.

Observation 7: If the random ID in Msg2 doesn’t match with previously transmitted one in Msg1, A-IoT device shall perform re-access procedure and the A-IoT device is not expected to autonomously re-access. The re-access is always controlled by reader.

Correct behaviour when not receiving Msg2

If A-IoT devices do not receive the response from reader (Msg2), one description in [2] could be referred as below:

If the A-IoT device receives the A-IoT Msg2 including a random ID, which is the same as the previously transmitted one in A-IoT Msg1, it considers the contention resolution as successful. If the A-IoT Msg2 is not received by the device, the re-access is not autonomously performed i.e., the re-access is always controlled by the reader.

It says if Msg2 is not received by the device, the re-access is not autonomously performed which is similar to when receiving Msg2 but random ID is not matched. Consequently, A-IoT device shall perform re-access procedure and the A-IoT device is not expected to autonomously re-access if no response is received within the time duration. The re-access is always controlled by reader.

Observation 8: If target devices do not receive Msg2 from reader, the re-access will controlled/provided by reader.

Proposal 3: The correct behaviour of Ambient IoT CBRA shall be captured into TS38.133, the following correct behaviour shall be considered:

Ambient-IoT CBRA: Correct behaviour when transmitting Msg1 (Random ID) Correct behaviour when receiving Msg2 (Response Msg1) Correct behaviour when not receiving Msg2 (Response Msg1)

2.1.2 Contention free random access procedure (CFRA)

A reader can configure either contention based access or contention free random access, it needs to be further discussed on whether it is explicitly or implicitly by other working groups. If the random access is contention free access, the following two steps apply:

- Target device selects the indicated D2R occasion/resource and transmit Msg1 with random ID;

- Target device skips the contention resolution and performs the data transmission.

From our perspective, A-IoT CFRA procedure is similar to legacy NR CFRA procedure although there is no RRC signaling operation during A-IoT process, that is, reader indicates dedicated D2R occasion or resource to device. The correct behaviour of A-IoT CFRA procedure could refer to CBRA procedure with two specific steps mentioned above.

Proposal 4: RAN4 shall capture the correct behaviour of Ambient IoT CFRA into TS38.133, the following correct behaviour shall be considered:

Ambient-IoT CFRA: Correct behaviour when transmitting Msg1 (Random ID) Correct behaviour when receiving Msg2 (Response Msg1) Correct behaviour when not receiving Msg2 (Response Msg1)

2.2 Timing relationship

In the latest WID, only deployment scenario 1 with topology 1 and device 1 are considered. The scenario is illustrated in [4] as below:

Figure 3 Topology 1

In topology 1, the Ambient IoT device directly and bidirectionally communicates with a BS. The communication between the BS and the Ambient IoT device includes Ambient IoT data and/or signaling. R2D means BS transmits data or indication to device, D2R means device transmits data to BS.

RAN1 has already defined some timing relationship parameters, but there are still some parameters under discussion, such as [TR2D_max]. Based on the timing parameters defined by RAN1, we can generate the following figure with random access:

- Figure 4 Timing relationship with random access procedure

- Since the concrete range of timing relationship will be discussed in RAN1, the main task in RAN4 is to discuss the timing accuracy requirements for TR2D_min, TD2R_max, TD2R_min. For TR2D_max, TR2D_TR2D_min, TD2R_D2R_min, RAN1 is still discussing whether to define them and identifying the specific use cases for them. Thus, RAN4 shall wait for RAN1’s conclusion on the concrete range of timing relationship and define the specific timing accuracy requirements.

Observation 9: RAN1 has already defined some timing relationship parameters, but there are still some parameters under discussion, such as [TR2D_max, TR2D_TR2D_min, TD2R_D2R_min].

Observation 10: The concrete range of timing relationship will be discussed in RAN1, the main task in RAN4 is to discuss the timing accuracy requirements.

Proposal 5: RAN4 shall wait for RAN1’s conclusion on the range of timing relationship and further define the specific timing accuracy requirements.

Conclusion

In this paper we provided our views on RRM impact on R19 Ambient-IoT.

Observation 1: The legacy RRC layer is not supported, including RRC states, RRM L3 measurement reporting and mobility etc.

Proposal 1: Only the RRM impact on supported A-IoT functionalities shall be discussed.

Proposal 2: The timing issue should be studied, including both downlink timing and uplink timing synchronization or latency between R2D and D2R transmission.

Observation 2: Only 3-step CBRA procedure is agreed for Ambient-IoT.

Observation 3: Ambient-IoT 3-step CBRA procedure is distinct from NR legacy 4-step and 2-step CBRA procedure.

Observation 4: The random ID is randomly generated by the A-IoT devices.

Observation 5: The probability of multiple A-IoT devices selecting the same access occasion and the same random ID for A-IoT Msg1 transmission will be sufficiently low.

Observation 6: If the random ID in Msg2 matches with previously transmitted one in Msg1, the correct behaviour is similar to legacy UE correct behaviour.

Observation 7: If the random ID in Msg2 doesn’t match with previously transmitted one in Msg1, A-IoT device shall perform re-access procedure and the A-IoT device is not expected to autonomously re-access. The re-access is always controlled by reader.

Observation 8: If target devices do not receive Msg2 from reader, the re-access will controlled/provided by reader.

Proposal 3: The correct behaviour of Ambient IoT CBRA shall be captured into TS38.133, the following correct behaviour shall be considered:

Ambient-IoT CBRA:

Ambient-IoT CBRA: Correct behaviour when transmitting Msg1 (Random ID) Correct behaviour when receiving Msg2 (Response Msg1) Correct behaviour when not receiving Msg2 (Response Msg1)

Proposal 4: RAN4 shall capture the correct behaviour of Ambient IoT CFRA into TS38.133, the following correct behaviour shall be considered:

Ambient-IoT CFRA:

Ambient-IoT CFRA: Correct behaviour when transmitting Msg1 (Random ID) Correct behaviour when receiving Msg2 (Response Msg1) Correct behaviour when not receiving Msg2 (Response Msg1)

Observation 9: RAN1 has already defined some timing relationship parameters, but there are still some parameters under discussion, such as [TR2D_max, TR2D_TR2D_min, TD2R_D2R_min].

Observation 10: The concrete range of timing relationship will be discussed in RAN1, the main task in RAN4 is to discuss the timing accuracy requirements.

Proposal 5: RAN4 shall wait for RAN1’s conclusion on the range of timing relationship and define the specific timing accuracy requirements.

- 4 References

- RP-243326, New WID: Solutions for Ambient IoT (Internet of Things) in NR.

- TR 38.769

- TS 38.133

- TR 38.848