3GPP TSG-RAN4 Meeting #114bis	R4-2503835

Wuhan, China, April 7 – 11, 2025

Agenda item:			7.5.2

Source:	vivo

Title:	Discussion on methods for reducing the number of UE RX chains

Document for:	Discussion

- Introduction

In RAN4#114 meeting, the WF[1] has been agreed as follows, and this contribution will discuss about the remaining open issues.

#### Issue 3-1: Discussion on maximum input level for fragmented carriers - Maximum power level including all RF signals and interference at the UE input does not exceed -25 dBm #### Issue 3-2: Discussion of impacts on DL performance for UE enabling one Rx RF chain mode - Relaxation on REFSENS for both PCC and SCC will be evaluated and captured in the study outcome report. - DL performance degradation can be considered based on configurations - Configurations leading to no relaxation are not precluded pending on the evaluation results. The exact values and applicable conditions shall be decided in the WI phase if there is corresponding WI - Maintain the table format agreed in previous RAN4 meeting. Detailed evaluation assumptions such as imbalanced PRx/DRx noise level and other assumptions are encouraged to be provided from companies evaluation TPs #### Issue 2-6: Baseband assumption We do not need new agreements. Refer to WF R4-24020519 #### Issue 3-3: Discussion on in-gap interference requirements - For image of in-gap interference not overlap with wanted CC: Evaluate whether relaxation on ACS/IBB is needed based on evaluated ΔRIBNC on PCC and SCC. - For image of in-gap interference(ACS/IBB) overlap with wanted CC: companies are encouraged to provide the impact of interference to wanted signal power ratio of [15-26]dB to the ΔRIBNC for PCC and SCC. #### Issue 3-4: Discussion on DL fragmented carriers in TDD bands - Option 2: Separate LOs are assumed for UL and DL for TDD band Open issues: ### How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers Option 1: Legacy band combination reporting scheme + Rx sharing capability indication - FFS on whether the capability is specified as ”per-band per-BC” or “per CC per band per BC”. Other options are not precluded ### Discussion on semi-statically switch hardware resources procedure and fallback considerations ### Triggering condition to enable ”One RF Rx chain” FFS: - FFS on whether the UE indication solution is needed or not, with understanding that enabling one Rx chain is under the control of network. - Triggering condition for UE working in one Rx RF chain is needed - Triggering condition for UE working in legacy Rx RF chains is needed - The switching should be not be dynamic and be under network control with UE indication of condition. ### Discussion on applicability for PCell and SCell ### Discussion on the number of Rx and DL MIMO layer configuration for the UE supporting ”one Rx RF chain” mode

- Discussion

## Triggering condition to enable “One RF Rx chain”

As has been specified in fully separate and partially shared architecture, the absolute power level of the in-gap interferer and the relative power imbalance between it and the wanted signal is the test parameter and condition of ACS and IBB applying. With the corresponding test parameters, when the throughput is ≥ 95 % of the maximum throughput of the reference measurement channels then the ACS/IBB requirements are considered to be fulfilled. With RX sharing these requirements should be relaxed to different levels according to the frequency range and bandwidth of the in-gap interferer.

However, there should be some trigger conditions on whether RX sharing is allowed. When the condition is met then RX sharing could be performed and then the receive performance and demodulation effect could be verified further. If there is no trigger condition the UE would not know when to assess the receiving performance based on the relaxed set of requirements. RX chain sharing should be an optional capability and not mandatory, nor is it the default state for the UE.

Therefore, it is illogical for the UE defaults to rely on the fully shared architecture for signal reception with the relaxed DL requirements. Instead, the UE should first determine some basic information, such as when the interference signal’s absolute power and/or relative power density difference satisfy the corresponding requirement then the RX chain could switch to shared state and the received signal could be evaluated by a different set of requirements.

Observation1: It is unreasonable for the UE defaults to rely on one RX RF architecture for signal reception with the relaxed DL requirements.

Proposal 1: RX chain sharing should be an optional capability and not mandatory, nor is it the default state for the UE.

Besides, the context of the current discussion in SI stage is restricted in co-located prerequisite, which is an ideal environment. With the collocated precondition, the blocker’s relative power compared to the target CCs could be predicted in a certain range and the PSD difference would be no more than 20dB from the observation and analysis of [2]. As a result, it is not necessary to measure the in-gap environment frequently, except for the image overlapping case: When the blocker falls in the frequency range that its image would overlap the target CC then the blocker’s power should be controlled to a rather low level to guarantee the correct demodulation of the target CC. On the preceding condition if the blocker remains its frequency position unchanged and could not satisfy the corresponding power requirement then the blindly indication of RX chain sharing would be meaningless as the UE needs to fallback to legacy CA comb again and again, which is a waste of signaling. In total, the triggering condition for UE working in one RX RF chain is needed.

At the same time, if the related discussion would be expanded to the non-collocated assumption or inter-band combination, which is also kind of the direction of 6G study in the future, then inside the sharing chain the interfere signal would become more complex and uncontrollable. Therefore, the trigger condition to enable “one RX RF chain” should be specified and the backward compatibility also needs to be considered when getting into the details.

Observation 2: Considering the image overlapping issues and backward compatibility to non-collocated scenarios in the future, the measurement of the in-gap environment before switching to one RX RF chain mode is needed to pre-judge the receiving environment and prevent frequent fallbacks.

From the other side, when demodulation fails in one RX RF chain mode, the NW needs to accurately determine whether the cause stems from increased TX leakage and/or excessively high in-gap interference introduced by RX chain sharing, or other environmental factors not related to the state of the RX chain. Therefore, the measurement procedure is necessary, particularly for evaluating the in-gap environment. The measurement results will guide the NW in determining the next steps to avoid blindly switching link states without resolving the underlying demodulation performance degradation issues.

Observation 3: When demodulation fails in one RX RF chain mode, the UE needs to do some measurements and report the results to the NW to help it determine whether falling back will be efficient or not.

The conditions for switching to a shared RX chain can include: The PSD difference between the interference signal and the target signal does not exceed a certain value, the frequency separation between two DL CCs does not exceed a certain value, and the difference between the reception times of the two DL CCs does not exceed a certain value, etc. As can also be seen in [3], the blocker’s position should also be one of the vital factors to decide whether to perform sharing RX chain: If under one RX RF chain mode and the blocker’s image overlaps with the target CC then the procedure of image suppression should be triggered, or else the image product would become rather high to prevent effective demodulation of target CCs.

Proposal 2: Triggering conditions for UE working in one RX RF chain or falling back to legacy Rx RF chains is needed, including:

- Whether the PSD difference between the in-gap interferer and target CC exceeds a certain value.

- Whether the blocker is located in a position where its image would interfere with the target CC.

- Whether the frequency separation between two configured DL CCs exceeds a certain value.

- Whether the difference between the reception times of the two DL CCs exceeds a certain value.

## Detecting & reporting related issues

Take the measurement of in-gap blocker as an example. As the blocker is from another operator and its power could not be restricted by the deployment, the NW could not directly know the power strength either. Hence, the corresponding information could only be detected and measured by the UE.

The measurement method is diverse, and some existing measurement& report process could be a reference, such as RSSI measurement procedure. As to the measurement details the guidance from RAN2&RRM is needed, e.g., measurement gap, configuration of in-gap information and so on.

Observation 4: The measurement details need guidance from RAN2& RRM.

## Decision maker of switching & Fall-back configuration & Applicability for PCell and SCell

From one side, when the sharing condition is satisfied, the UE with sharing capability could decide to switch to the shared RX chain by itself and the relaxed set of requirements would take place of the legacy set. The switching response is faster and the time for the UE to report to the NW is omitted. When the state of the blocker changes, the switching response will be relatively quick, but the UE still needs to report the receiving mode for the NW to determine whether to indicate a new CA configuration.

From the other side, it could also be up to NW to decide whether to perform RX chain switching, i.e., with the UE’s report on the capability of RX chain sharing and the measurement results of the in-gap signal, the NW could determine and indicate whether to switch to one RX RF chain and simultaneously indicate a higher CA configuration. It would allow the UE to transmit data more efficiently using more CCs or even the freed MIMO layers. From this perspective, the later mechanism seems more reasonable. However, another extreme case should also be considered: When the blocker’s power suddenly increases to a rather high level or the target CC’s power drops to a rather low level for some uncertain reason, the NW may lose connection with the UE and the UE also cannot report the poor measurement results to the NW immediately. In this case, the UE has to decide to fall back on its own. Hence, the UE decision-making mechanism should be considered as a supplement.

Proposal 3: It is up to the NW to decide when to perform RX chain switching, and the UE decision-making mechanism could be considered as a supplement for some extreme cases.

With the RX chains switching to being shared, the RF requirements should be adjusted and some RF chains could be released, potentially allowing for the aggregation of more CCs. This would result in a wider variety of CA configurations supported by the UE. However, these new CA configurations could only be indicated with shared RX chains. Also, when the RX chains fall back to separate architecture, the CA configuration should also switch to the default group with less CCs combined. The released CC could be one of the non-contiguous intra-band CCs or the other inter-band CC. For example, the CA configuration in original separate architecture is CA_nX(2A), after RX chain sharing it may change to CA_nX(2A)-nY as the two CCs in band nX could be received by only one RX RF chain. However, when the fall back mechanism is triggered and the CA config would fall back to CA_nX(2A) or CA_nX-nY, the decision is up to the network.

Proposal 4: When fallback is triggered, the change of CA combination should still be determined by NW.

In the extreme cases mentioned above, when the UE has to decide to fall back on its own, and there should be a default process for the UE to follow, e.g., For single band operation, there must be either one of the two-fragmented carriers to be a PCELL, and when the UE needs to fall back then it would switch to the non-CA mode. It could also be extended to one band with more than 2CCs. However, for the higher band combination with more than one band, generally not all the bands would receive in one RX RF mode, so fallback may not be a big issue.

Proposal 5: For a mechanism in which the UE determines the switching of reception modes, a default fall-back process is necessary, e.g., one of the two fragmented carriers is configured as PCELL for the single band operation.

## How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers

Related to the UE capability for supporting one RX RF chain, it should be combined with the specific CA combination, as has been proposed in [4] there are three options to be considered:

Alt. 1: Legacy band combination reporting scheme + Rx sharing capability indication.

Alt. 2: Band combination capability update with the supported number of CCs.

Alt. 3: New band combination definition e.g. CA_n25(f_2A) + new set of signaling.

From our perspective, Alt2 should be deprioritized as it doesn’t match the current CA comb reporting scheme. The UE capability should follow the legacy reporting procedure, i.e., the new CA combination supported in one RX RF chain mode should be reported with other default-supported CA configurations, as they are all related to the stable hardware capabilities. Alt2 also contains an assumption that the UE could default work at one RX RF chain mode regardless the in-gap environment, which is unreasonable.

For Alt1 and Alt3, both are acceptable to us. However, for Alt1, it should introduce more indication details about which CCs can perform RX chain sharing. For example, in a combination of CA_nX(3A)-nY (2A), if only two CCs in the nX band can share the receive chain together due to the total bandwidth of 100M, then it would be very confusing for NW which two CCs can share the receive link if the ability to support RX chain sharing is reported by ‘per BC per band’. Considering the granularity of the capability, it seems that when the capability is specified as ‘per CC per band per BC’, it would be clearer.

Observation 5: ‘Legacy band combination reporting scheme + Rx sharing capability indication’ should consider the granularity of the capability because which CCs could be shared needs to be clarified.

Proposal 6: For ‘Legacy band combination reporting scheme + Rx sharing capability indication’, the granularity of the capability should be set as ‘per CC per band per BC’.

However, as discussed in the last meeting, some companies also have some concerns on whether the details of capability could be decided by RAN4 and maybe we should consult RAN2 for some guidance, so we could select options 1& 3 first and provide the pros and cons to RAN2 for further study.

Observation 6: For the details of UE capability for supporting ‘one RX RF chain’, RAN4 should provide the pros and cons of Alt1&3 to RAN2 for further study.

## Semi-statically switch hardware resources procedure and fallback considerations

In the co-located scenario, the power difference of the blocker and the target CCs is rather stable and divinable. So, the switching of the two receiving mode may not be too frequent. From the other side, the hardware capability will also limit the possibility of dynamic switching. From the implementation and signaling workload side, semi-statically switch is a better choice.

In addition, when there are multiple UEs locating nearby and operating in the same band combination, once one UE reports the gap information of two CCs in one band to NW, repeated detection of the other UEs can be ignored in the short term because the NW can make the same receiving state switch decision for all the UEs. From this perspective, it can also save a lot of evaluation and reporting process.

Observation 7: In the co-located scenario, the power difference of the blocker and the target CCs is rather stable, the switching of the two receiving modes may not be too frequent.

Proposal 7: The switching should not be dynamic and should be under network control with UE indication of condition.

## Identification of the issue that triggers discussion of RAN2 and/or RRM

According to the discussion above, there are some issues that need to trigger the discussion of RAN2 and/or RRM, and the related details are as follows:

Proposal 8: RAN4 RF needs to identify the issues to trigger the discussion of RAN2 & RRM side, for example:

- The details of UE capabilities for supporting higher CA combination with shared chain, RAN4 should provide the pros and cons of Alt1&3 to RAN2 for further study.

- The design of the measurement signal and reporting procedure.

- Conclusion

Observation 1: It is unreasonable for the UE defaults to rely on one RX RF architecture for signal reception with the relaxed DL requirements.

Proposal 1: RX chain sharing should be an optional capability and not mandatory, nor is it the default state for the UE.

Observation 2: Considering the image overlapping issues and backward compatibility to non-collocated scenarios in the future, the measurement of the in-gap environment before switching to one RX RF chain mode is needed to pre-judge the receiving environment and prevent frequent fallbacks.

Observation 3: When demodulation fails in one RX RF chain mode, the UE needs to do some measurements and report the results to the NW to help it determine whether falling back will be efficient or not.

Proposal 2: Triggering conditions for UE working in one RX RF chain or falling back to legacy Rx RF chains is needed, including:

- Whether the PSD difference between the in-gap interferer and target CC exceeds a certain value.

- Whether the blocker is located in a position where its image would interfere with the target CC.

- Whether the frequency separation between two configured DL CCs exceeds a certain value.

- Whether the difference between the reception times of the two DL CCs exceeds a certain value.

Observation 4: The measurement details need guidance from RAN2& RRM.

Proposal 3: It is up to the NW to decide when to perform RX chain switching, and the UE decision-making mechanism could be considered as a supplement for some extreme cases.

Proposal 4: When fallback is triggered, the change of CA combination should still be determined by NW.

Proposal 5: For a mechanism in which the UE determines the switching of reception modes, a default fall-back process is necessary, e.g., one of the two fragmented carriers is configured as PCELL for the single band operation.

Observation 5: ‘Legacy band combination reporting scheme + Rx sharing capability indication’ should consider the granularity of the capability because which CCs could be shared needs to be clarified.

Proposal 6: For ‘Legacy band combination reporting scheme + Rx sharing capability indication’, the granularity of the capability should be set as ‘per CC per band per BC’.

Observation 6: For the details of UE capability for supporting ‘one RX RF chain’, RAN4 should provide the pros and cons of Alt1&3 to RAN2 for further study.

Observation 7: In the co-located scenario, the power difference of the blocker and the target CCs is rather stable, the switching of the two receiving modes may not be too frequent.

Proposal 7: The switching should not be dynamic and should be under network control with UE indication of condition.

Proposal 8: RAN4 RF needs to identify the issues to trigger the discussion of RAN2 & RRM side, for example:

- The details of UE capabilities for supporting higher CA combination with shared chain, RAN4 should provide the pros and cons of Alt1&3 to RAN2 for further study.

- The design of the measurement signal and reporting procedure.

- References

- R4-2502873, WF on NR FR1 DL fragmented carriers study, MediaTek Inc. RAN4#114.

- R4-2501803, On RF requirements of fragmented carriers, Huawei, HiSilicon, RAN4#114.

- R4-2503836, Impacts on UE RF requirements and DL performance, vivo, RAN4#114bis.

- R4-2500243, On methods for reducing the number of UE Rx chains for fragmented carriers, Apple, RAN4#114.