3GPP TSG-RAN WG4 Meeting #114bis	R4-2503881

Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	Discussion on RRM requirements for supporting intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s)

Source: 	Huawei, HiSilicon

Agenda item:	7.4.2

Document for:	Discussion

Introduction

Supporting of intra-band non-collocated EN-DC/NR-CA deployment for Type 2 UE has been completed in Rel-18. The requirements for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and FR1 intra-band non-contiguous NR-CA for Type 2 UE were specified.

The new R19 WI is to further support intra-band non-collocated EN-DC/NR-CA deployment scenarios for Type 4 FWA UE. The objectives of the WID are duplicated as below [1].

- 4.1 Objective of SI or Core part WI or Testing part WI The core part of the work item includes: - Specify the core requirements for type 4a/4b capable FWA UE that supports non-co-located scenarios for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and/or FR1 intra-band non-contiguous NR-CA for up to 4-layer DL MIMO per CC, where MRTD>CP is assumed. - RF power imbalance requirements of up to 25dB - RRM requirements including MTTD and MRTD requirements and other identified requirement such as scheduling restriction/availability, SCell operation related delay and interruption. - Work is limited to EN-DC/NR-CA for bands 42, n77/n78 - Discuss and if needed, specify the UE behavior when the UE supports type 4a/b but operates in co-located scenarios depending on whether network signaling is provided. R18 handling of UE behavior can be used as a reference.

The contribution provides some analysis for supporting intra-band non-collocated EN-DC/NR-CA deployment scenarios from RRM requirements perspective.

Discussion

RF progress

In the following, the progress so far on UE capability and BS signalling in RF session for type 4 UE are summarized as below.

UE capability < Issue 2-2-2: Whether to support Type 2 capability by UE having Type 4 capabilities > Agreement: - If UE reports the Type 4 capability, Type 2 capability shall be deemed as support by default regardless of whether UE indicates Type-2 capability or not. - If UE reports the Type 4 capability, it is not necessary for UE to report Type 2 capabilities. If the issue is identified for these bullets, RAN4 will revisit them. < Issue 2-1: How to differentiate UE Capability Type 4a(6Rx/UE) and 4b(8Rx/UE) for EN-DC > Agreement: - Legacy capabilities, such as MIMO-CapabilityDL-r10 on E-UTRA carriers < Issue 2-2: UE Capability Type 4a(6Rx/UE) for NR-CA > Agreement: - Do not consider Type 4a for NR-CA

BS signalling No new NW signaling needed from Type 4 to single carrier fallback. 1）When maxMIMO-layers > 4, Type 1(collocated). No new BS signaling is needed. 2）When maxMIMO-layers=4, it is still under discussion 3）When maxMIMO-layers=2, - If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present, UE is Type 1. - If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent, UE is Type 2. NOTE: This is specified Rel-18 behavior. No need to further discuss.

RRM progress

In RRM session, in RAN4#112bis meeting, the impacted RRM core requirements were identified in [R4-2416850], and in RAN4#113 meeting, CR split among companies was approved in [R4-2420090]. The technical issues are almost settled in RRM. But it seems that the progress in RF session is not so sufficient to complete the formal CRs for RRM requirements. So we suggest companies can provide the draft CRs for the identified RRM requirements, and the UE capability regarding Type 4 UE and the description of “R19 type 4 capable UE is in non-collocated/collocated condition” with regard to BS signalling can be updated, once the UE capability and BS signalling are concluded in RF session.

Proposal: Companies can provide the draft CRs for the identified RRM requirements, and the Type 4 UE capability and the description of “R19 type 4 capable UE is in non-collocated/collocated condition” with regard to BS signalling can be updated, once the UE capability and BS signalling are concluded in RF session.

Conclusion

The contribution provides some analysis for supporting intra-band non-collocated EN-DC/NR-CA deployment scenarios from RRM requirements perspective.

Proposal: Companies can provide the draft CRs for the identified RRM requirements, and the Type 4 UE capability and the description of “R19 type 4 capable UE is in non-collocated/collocated condition” with regard to BS signalling can be updated, once the UE capability and BS signalling are concluded in RF session.

Reference

- RP-240838, New WID on Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s)