3GPP TSG-RAN4 Meeting # 114bis	R4-2503971

Wuhan, China, Apr.7th – 11th, 2025

# Title: 	Discussion on UE Capability UE behavior and network signaling for 	Type 4 NR CA/ENDC

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.4.1.2

# Document for:	Approval

# 1 Introduction

- In this contribution, we continue sharing our views about UE Capability/UE behavior and NW signaling for type 4 non-collocated intra-band NR CA and inter-band ENDC.

- Discussion

Regarding the UE Capability/UE behavior (i.e. Type 1, Type 2 and Type 4a/4b) and NW signaling for type 4 non-collocated intra-band NR CA and inter-band ENDC, it can be splitted into three cases which relies on the maxMIMO-layers per CC, which are:

1. maxMIMO-layers = 2

2. maxMIMO-layers = 4

3. maxMIMO-layers =>4

For the former two, there are agreements in the WF [1][2], we capture here for convenience.

- <Issue 2-2-3: UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b> Way forward: - When maxMIMO-layers = 2 - If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present, UE is Type 1. - If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent, UE is Type 2. NOTE: This is specified Rel-18 behavior. No need to further discuss.

And:

- <Issue 2-2-3: UE behavior or maxMIMO-layers>4/CC> Agreement: When maxMIMO-layers > 4, Type 1(collocated). No new BS signaling is needed.

For the last one, there are several options in last meeting:

- Option 1 (Samsung/Apple/KDDI) maxMIMO-layers=4, if Rel-19 new BS signaling IE is present, it could be either collocated(Type-1) or non-collocated(Type-4) pending on value is 0 or 1; If Rel-19 new BS signaling IE is absent, UE follows Rel-18 specification. - Option 2 (Nokia) - UE is Type 1 (2/CC), Type 1 (4/CC) or Type 4 depending on present Rel-19 BS signaling element value. - When Rel-19 BS signaling is absent, UE follows Rel-18 BS signaling, i.e. is Type 4. - Option 3 (Qualcomm) When maxMIMO-layers = 4 - Evaluate following options - Type 1 when rel-19 BS signaling is absent, and respective Rel-19 BS signaling - Type 4 when rel-19 BS signaling is absent, and respective Rel-19 BS signaling - Option 4 (ZTE) If maxMIMO-layer = 4 and Rel-19 NW signaling is provided, the NW signaling needs to specify Type 4 or Type 1. If maxMIMO-layer = 4 and NW signaling is not provided, Type 1 is assumed. - Option 5 (Huawei) When maxMIMO-layers = 4 - When BS signaling is absent, Type 4 is considered. (in accordance to the way Type 2 was specified in Rel-18) UE is Type 1. - When BS signalling is present, Type 1 is considered. (in accordance to the way Type 1 was specified in Rel-18).

In type 2 discussion, it is indicated as Type 2 UE when the Rel-18 NW signaling is absent, otherwise Type 1 UE is indicated. The Rel-18 NW signaling is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells (i.e. maxMIMO-layers ≤ 2 per CC). In other words, when maxMIMO-Layers with value is more than 2 each CC, then the Rel-18 NW signaling can be seen as absent and UE is collocated.

Unlike type 2 discussion, for type 4, when maxMIMO-layers =4 per CC which means the maxMIMO-Layers with value is more than 2 each CC, both Rel-18 type 1(collocated) and Rel-19 type 4(non-collocated) can be re-configured by NW. So we think explicit indication from NW for UE type is needed.

Figure 2-1 shows the situations for UE type determination when maxMIMO-layers =4.

Figure 2-1 UE type determination for the case when UE type 4 is reported

For figure 2-1(a), when the network is Rel-18 and maxMIMO-layers =4, if the signaling scheme of Rel-18 is followed, there is a contradiction between NW signaling and maxMIMO-layers =4. For this situation, UE is type1.

For figure 2-1(b), when the network is Rel-19 and maxMIMO-layers =4, NW signaling needs to indicate Rel-18 type 1(collocated) and Rel-19 type 4(non-collocated).

Considering the above, Rel-19 NW signaling design may be a bit different with Rel-18 NW signaling design. But of course, how to reflect the details can be final decided by RAN2.

Regarding the above options, in our understanding, option 4 is the similar direction as in option 1 and option 5, which aims to allow NW can explicitly indicate collocated and non-collocated behavior for UE. For sake of progress, we slight prefer to option 1 since the situation is clear especially for collocated considering collocated may always be possible no matter Rel-19 NW signaling is present or absent in the case of maxMIMO-layers =4 per CC.

Observation: when maxMIMO-layers =4 per CC, both Rel-18 type 1(collocated) and Rel-19 type 4(non-collocated) can be re-configured by NW.

Proposal: When maxMIMO-layers=4, if Rel-19 new BS signaling IE is present, it could be either collocated(Type-1) or non-collocated(Type-4) pending on value is 0 or 1; If Rel-19 new BS signaling IE is absent, UE follows Rel-18 specification.

# 3 Conclusion

In this contribution, we give our viewpoint about requirements for type 4a/4b for NonCol_intraB_ENDC_NR_CA.

The conclusions are:

Observation: when maxMIMO-layers =4 per CC, both Rel-18 type 1(collocated) and Rel-19 type 4(non-collocated) can be re-configured by NW.

Proposal: When maxMIMO-layers=4, if Rel-19 new BS signaling IE is present, it could be either collocated(Type-1) or non-collocated(Type-4) pending on value is 0 or 1; If Rel-19 new BS signaling IE is absent, UE follows Rel-18 specification.

# 4	Reference

- R4-2420399,WF on non-collocated intra-band EN-DC and NR-CA, KDDI

- R4-2502924, WF on UE requirements for non-collocated scenario, KDDI

- Annex

nonCollocatedTypeMRDC-r18 (NW signaling) This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements when indicating support of interBandMRDC-WithOverlapDL-Bands-r16.

nonCollocatedTypeNR-CA-r18 (NW signaling) This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NRCA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15] when indicating support of intraBandNR-CA-non-collocated-r18.

interBandMRDC-WithOverlapDL-Bands-r16(UE capability) -- R4 2-19 FDD-FDD or TDD-TDD inter-band MR-DC with overlapping or partially overlapping DL spectrum Indicates whether the UE supports FDD-FDD or TDD-TDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [5] and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 [5] and inter-band RF requirements. If the capability is not reported, the UE supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially overlapping DL bands with (NG)EN-DC/NE-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [5] and intra-band RF requirements.

requirementTypeIndication-r18(UE capability) -- R4 33-2: Support network control of requirement network applicability for UE supporting interBandMRDC-WithOverlapDL-Bands-r16 Indicates whether the UE supports network controlled indication of the MTTD/MRTD and RF requirements by nonCollocatedTypeMRDC-r18 for TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands as specified in TS 38.331 [9]. This field is only applicable to the UE indicating interBandMRDC-WithOverlapDL-Bands-r16.

intraBandNR-CA-non-collocated-r18 (UE capability) -- R4 33-1: Support of intra-band non-collocated NR CA operation Indicates whether the UE supports TDD-TDD intra-band non-collocated NR-CA operation with MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [5] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [2], and TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in TS 38.133 [5] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [2]. If the capability is not reported, the UE only supports TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in TS 38.133 [5] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [2]. A UE supporting this feature shall also support network controlled indication of the MTTD/MRTD and RF requirements by nonCollocatedTypeNR-CA-r18 for intra-band non-collocated NR-CA, as defined in TS 38.331 [9].