3GPP TSG-RAN WG4 Meeting #114bis	R4-2504198

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	Ericsson

Title:	UE Capability/UE behavior and network signaling for Type 4 EN-DC/NR-CA

Agenda item:	7.4.1.2

Document for:	Approval

# 1	Introduction

In RAN4#114 the following WF was agreed [1]:

… ### <Issue 2-2-7: BS signaling framework> Way forward: - Continue further discussions on following options in the next RAN4 meeting. - Option 1 (Samsung/Apple/KDDI) 1）maxMIMO-layers ＞4, it is collocated. 2）maxMIMO-layers=4, if Rel-19 new BS signaling IE is present, it could be either collocated(Type-1) or non-collocated(Type-4) pending on value is 0 or 1; If Rel-19 new BS signaling IE is absent, UE follows Rel-18 specification. 3）maxMIMO-layers=2, follow Rel-18 specification →This was already agreed in last meeting - Option 2 (Nokia) - UE is Type 1 (2/CC), Type 1 (4/CC) or Type 4 depending on present Rel-19 BS signaling element value. - When Rel-19 BS signaling is absent, UE follows Rel-18 BS signaling, i.e. is Type 4. - Option 3 (Qualcomm) 1）When maxMIMO-layers ＞4 - UE is Type 1 2）When maxMIMO-layers = 4 - Evaluate following options - Type 1 when rel-19 BS signaling is absent, and respective Rel-19 BS signaling - Type 4 when rel-19 BS signaling is absent, and respective Rel-19 BS signaling 3）When maxMIMO-layers = 2, UE follows Rel-18 - Option 4 (ZTE) 1）If maxMIMO-layer > 4, Rel-19 Type 1 is assumed. 2）If maxMIMO-layer = 4 and Rel-19 NW signaling is provided, the NW signaling needs to specify Type 4 or Type 1. If maxMIMO-layer = 4 and NW signaling is not provided, Type 1 is assumed. - Option 5 (Huawei) 1）When maxMIMO-layers ＞4 - UE is Type 1 2) When maxMIMO-layers = 4 - When BS signaling is absent, Type 4 is considered. (in accordance to the way Type 2 was specified in Rel-18) UE is Type 1. - When BS signalling is present, Type 1 is considered. (in accordance to the way Type 1 was specified in Rel-18). 3) When maxMIMO-layers = 2, UE follows Rel-18 …

In this contribution we discuss proposals related to the issues above.

# 2	Discussion

## 2.1 	Issue 2-2-7: BS signaling framework

Option 1, originally proposed by Samsung/Apple/KDDI are fine for us:

- Option 1 (Samsung/Apple/KDDI)

1）maxMIMO-layers ＞4, it is collocated.

2）maxMIMO-layers=4, if Rel-19 new BS signaling IE is present, it could be either collocated(Type-1) or non-collocated(Type-4) pending on value is 0 or 1; If Rel-19 new BS signaling IE is absent, UE follows Rel-18 specification.

3）maxMIMO-layers=2, follow Rel-18 specification →This was already agreed in last meeting

Observation 1: Option 1, originally proposed by Samsung/Apple/KDDI are fine for us:

- Option 1 (Samsung/Apple/KDDI)

1）maxMIMO-layers ＞4, it is collocated.

2）maxMIMO-layers=4, if Rel-19 new BS signaling IE is present, it could be either collocated(Type-1) or non-collocated(Type-4) pending on value is 0 or 1; If Rel-19 new BS signaling IE is absent, UE follows Rel-18 specification.

3）maxMIMO-layers=2, follow Rel-18 specification →This was already agreed in last meeting

The options 1 to 5 are similar, but it has been hard to reach consensus. The logical conclusion is then to let RAN2 decide based on RAN4 LS with appropriate pre-conditions, if any.

Observation 2: The options 1 to 5 are similar, but it has been hard to reach consensus.

Proposal 1: let RAN2 decide based on RAN4 LS with appropriate pre-conditions, if any.

# 3 	Summary

Observation 1: Option 1, originally proposed by Samsung/Apple/KDDI are fine for us:

- Option 1 (Samsung/Apple/KDDI)

1）maxMIMO-layers ＞4, it is collocated.

2）maxMIMO-layers=4, if Rel-19 new BS signaling IE is present, it could be either collocated(Type-1) or non-collocated(Type-4) pending on value is 0 or 1; If Rel-19 new BS signaling IE is absent, UE follows Rel-18 specification.

3）maxMIMO-layers=2, follow Rel-18 specification →This was already agreed in last meeting

Observation 2: The options 1 to 5 are similar, but it has been hard to reach consensus.

Proposal 1: let RAN2 decide based on RAN4 LS with appropriate pre-conditions, if any.

# References

[1] 		R4-2502924, WF on UE requirements for non-collocated scenario

[2]		R4-2418145, Topic summary for [113][123] NonCol_intraB_ENDC_NR_CA, Moderator (KDDI)