3GPP TSG-RAN WG4 Meeting #114bis	R4-2504348

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.1.1.3.2

Source:	MediaTek Inc.

Title:	Discussion on UE capability for 6 MIMO layer support

Document for:	Discussion

# Introduction

In this contribution, we provide our view about UE capability for supporting 6 layers.

- Discussion

At RAN4#114, RAN4 agreed to introduce 6-layer performance requirements for only FWA device [1].

Issue 2-1-1: Way forward for 6-layers support for handheld UE and FWA devices Way Forward: - RAN4 Chair to take note of the following in the meeting report: - “RAN4 unable to conclude on a common set of simulation assumptions to determine feasibility of 6 layer for handheld UE In Rel-19” - 6-layer performance requirements will be only defined for FWA. - 4-layer performance requirements will be defined and will apply to FWA and handheld UE devices. - The fact that 6-layer is only supported for FWA shall be explicitly captured at least in the RAN4 specifications. Issue 2-1-2: 6-layer Support as optional feature for FWA Way Forward: RAN4 to further discuss if 6-Layer support should be considered an optional feature per the following. - Introduce 6 MIMO layers support as an optional feature only for FWA.

Currently, there are only three options to report the maximum MIMO layers supported by UE, which are 2, 4, and 8 layers [2]. As RAN4 agreed to support 6-layer, the 6-layer capability needs to be introduced. Besides, it has been decided to limit the introduction of 6 layers to Fixed Wireless Access (FWA) only. Therefore, it is necessary to inform RAN2 to reflect the fact that 6-layer is only supported for FWA when introducing 6-layer capability.

Observation 1: Currently, only three options to report the maximum MIMO layers supported by UE, which are 2, 4, and 8 layers.

Observation 2: RAN4 agreed 6-layer performance requirements will be only defined for FWA. Also, the fact that 6-layer is only supported for FWA shall be explicitly captured at least in the RAN4 specifications.

Proposal 1: It is necessary to inform RAN2 to reflect the fact that 6-layer is only supported for FWA when introducing 6-layer capability.

- Conclusion

In this contribution, we provide our views about 6-layer capability support for 6Rx UE. The observations and proposals are summarized as:

Observation 1: Currently, only three options to report the maximum MIMO layers supported by UE, which are 2, 4, and 8 layers.

Observation 2: RAN4 agreed 6-layer performance requirements will be only defined for FWA. Also, the fact that 6-layer is only supported for FWA shall be explicitly captured at least in the RAN4 specifications.

Proposal 1: It is necessary to inform RAN2 to reflect the fact that 6-layer is only supported for FWA when introducing 6-layer capability.

# Reference

- R4-2503014, WF on 6Rx requirements, AT&T

- 3GPP TS38.331