3GPP TSG RAN WG4 Meeting #114-bis	R4-2504351

Wuhan, China, 7th – 11th April, 2025

Agenda item:			8.2

Source:	vivo

Title:	Reply LS on granularity definition of pdcch-RACH-AffectedBandsList-r18

Document for:	Discussion

# Introduction

In the RAN1#120 meeting, an LS [1] on granularity definition of pdcch-RACH-AffectedBandsList-r18 was approved. RAN4 is asked to discuss the understandings and provide clarification on which one is correct.

The UE capability pdcch-RACH-AffectedBandsList-r18 (FG39-4) is defined in RAN4 with the granularity as “Per band pair per band combination (between the target band for RACH transmission and band under UE’s current band combo)” For each reported capability, there are two possible values: interruption or noInterruption. Regarding the band pair (one source band +one target band) of the BC, RAN2 understands that the target band refers to the target band towards which the UE performs PDCCH-ordered early RACH. However, the definition of the source band of the band pair remains unclear in RAN2. - Understanding 1: The source band of the band pair represents the source band (or PCell) on which the PDCCH order is sent to the UE to perform PDCCH-ordered early RACH to the target cell/band. Regarding the interruption band(s), there are two further understandings, denoted by 1a and 1b: - Understanding 1a: The value “interruption/noInterruption” means that interruption is needed or not for all serving bands of the current band combination. - Understanding 1b: The value “interruption/noInterruption” means that interruption is needed or not for the source band (the same band in which the PDCCH order is sent) of the current band combination. For other serving bands, noInterruption can be always assumed. - Understanding 2: The source band of the band pair represents the specific source band that have interruption or not (depends on the value “interruption” or “noInterruption”) in the BC when UE performs PDCCH-ordered early RACH toward a target cell on target band. Please note that in Understanding 1, the UE cannot indicate which specific source band of the BC has an interruption or not (either all bands in current BC in 1a or the same source band that receives the PDCCH order in 1b). In Understanding 2, the information about which source band receives the PDCCH order is not provided.

In this contribution, we provide our views on granularity definition of pdcch-RACH-AffectedBandsList-r18.

# Discussion

The UE capability FG 39-4 is defined as follows.

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 39. NR_Mob_enh2 | 39-4 | Interruption on DL slot(s) due to PDCCH- ordered RACH transmission | Capability on whether UE may cause interruption on DL slot(s) on serving cells due to PDCCH-ordered RACH transmission |

This UE capability is relevant to UE architecture of band combination implementation. For some UE implementations, performing PDCCH-ordered RACH transmission on the target band may cause DL interruptions on some of the serving cells on bands under current band combination. Some other UE implementations may or may not cause DL interruptions on these band under same band combination. It mainly depends on hardware share status between source bands and target band whether DL interruptions will be caused or not. It is not relevant to whether PDCCH order is transmitted on the serving cell of the band.

Therefore, understanding 2 in RAN2 LS is aligned with RAN4 understanding. For the signaling design, there is no need to consider whether PDCCH order is transmitted on the band or not.

Proposal 1: Understanding 2 in the LS is the correct understanding of the UE capability FG 39-4.

# Summary

In this contribution, we provided views on granularity definition of pdcch-RACH-AffectedBandsList-r18. Following proposals are present.

Proposal 1: Understanding 2 in the LS is the correct understanding of the UE capability FG 39-4.

- References

- R2-2501392, LS to RAN4 on granularity definition of pdcch-RACH-AffectedBandsList-r18, RAN2

- Appendix: Draft reply LS on granularity definition of pdcch-RACH-AffectedBandsList-r18

3GPP TSG RAN WG4 Meeting #114-bis	R4-25x

Wuhan, China, 7th – 11th April, 2025

Title:	Reply LS on granularity definition of pdcch-RACH-AffectedBandsList-r18

Response to:	R2-2501392

Release:	Rel-18

Work Item:	NR_Mob_enh2-Core

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name:

E-mail Address:

1. Overall Description:

RAN4 Thanks RAN2 for the LS on granularity definition of pdcch-RACH-AffectedBandsList-r18. RAN4 discussed the questions in the LS and would like to provide following clarification.

- Understanding 2 is the correct understanding for the UE capability FG 39-4.

2. Actions:

To: RAN WG2

ACTION: 	RAN4 kindly asks RAN2 to take the above agreements into consideration in the future work.

3. Reference:

- R2-2501392, LS to RAN4 on granularity definition of pdcch-RACH-AffectedBandsList-r18, RAN2

4. Date of Next TSG RAN WG4 Meetings:

TSG RAN WG4 Meeting #115	 	   May 19 – May 23, 2025			Malta