3GPP TSG-RAN WG4 Meeting # 114-bis														R4-2404388

Wuhan, China, 7th – 11th April, 2025

Agenda item:			7.7.2

Source:	Samsung

Title:	UE RF requirements for LB CA via switching

Document for:	Discussion

1	Introduction

During the latest RAN4 meeting, RAN4 started to discuss a new RAN4-led Work Item (WI) on low band (LB) carrier aggregation (CA) via switching, which was approved in RAN#106 to address a market demand for LB enhancements [1]. During the first WI meeting at RAN4#114, RAN4 approved a WF [2] and an initial agreement on switching period values was sent to RAN1 [3] to help their discussion on the WI as shown below, considering the short period.

Define switching periods between FDD low band and SDL low band with the following values as a UE capability to accommodate different UE implementations - 35us, 140us

Building on the previous discussion and WF [2], in this contribution, we would like to continue providing our views focusing on the UE RF aspects taking into account the agreed assumption and restriction in the WID [1].

2	Discussion

In order to have a common understanding before going further, it would be good to confirm the target scenario, bands and assumptions or restrictions for this work item based on [1]. In this regard, RAN4 discussed and reached some agreements on the Rel-19 scope as captured in the WF to focus on the scenario with the switching between two low bands in Rel-19 WI as below [2]:

Issue 1: Scope clarification - Specify the switching period(s) based on the scenario with the switching between two low bands in Rel-19 WI - It is not precluded that such two low bands are further combined with the additional band - The combination of those two low bands with other band can be further discussed in the future release

With respect to the two bands mentioned in the WF above, they should be treated as CA_n5A-n29A, which serves as a single example band combination for this WI, as noted in [1]. Then, the switching scenario between LB-LB CA cases are limited to Case 1 and Case 2 with band n5 and n29, respectively, which are also related to following deployment constraints captured in [1]. Figure 1 shows the switching scenario of Rel-19 with its assumptions and constraints, which should be taken into account for the WI discussion in RAN4.

.

Figure 1: Switching scenario for Rel-19 (Case 1 and Case 2)

- The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers

Observation 1:	Switching scenario between LB-LB CA cases are limited to Case 1 and Case 2 with band n5 and n29, respectively, e.g., CA_n5A-n29A, which serves as a single example band combination for this WI.

Proposal 1:		RAN4 should take into account the switching scenario of Rel-19 with its assumptions and constraints for the WI discussion.

For switching period values, as sent an LS to RAN1 [3], two candidate values were agreed based on potential UE architectures for the LB-LB switching and previous uplink Tx switching discussions as follows:

Issue 2: Switching period values - Define switching periods between FDD low band and SDL low band with the following values as a UE capability to accommodate different UE implementations - 35us, 140us - RAN4 to further discuss 13us - LS will capture the currently confirmed values - It is not precluded to send another LS with more values in future meetings

For the value of 13us, which is still under further study, it could be excluded from Rel-19 unless it is specifically required by UE architecture for the switching between two bands, which is unlike the minimum absolute TA for a single TDD band. Moreover, it should be noted that the switching period also need to consider the minimum OFDM symbol length, which could be a basic parameter to determine the switching pattern and/or location to be defined in RAN1. From those aspects, 35us could be the minimum switching period value, which also can guide other numbers less than 35us to 35us. In addition, in terms of the symbol length-based switching period, 70us can be added as another candidate since it can be the smallest length for 15kHz SCS, which is in fact the sole assumption in the WID for both FDD low band and SDL low band switching. In our view, 70us should be counted as an option together with 35us and 140us.

Observation 2:	Switching period values should consider UE architecture for the switching between two bands.

Observation 3:	From OFDM symbol length aspects, 35us could be the minimum switching period value, which also can guide other numbers less than 35us to 35us.

Proposal 2:		The switching period value of 13us could be excluded from Rel-19 unless it is specifically required by UE architecture for the switching between two bands, which is unlike the minimum absolute TA for a single TDD band.

Observation 4:	70us can be added as another candidate since it can be the smallest length for 15kHz SCS, which is in fact the sole assumption in the WID for both FDD low band and SDL low band switching.

Proposal 3:		70us should be counted as an option together with 35us and 140us.

With that, as similar to the uplink Tx switching before, and also described in [1], the UE supports LB-LB CA through the switching would necessarily support a new UE capability, e.g., [lowBandSwitchingBandPair] per band combination, to indicate the switching capability between LBs for the network. One of the parameters under the UE capability can also indicate the length of carrier switching period (X) of the LB-LB switching, where the X could have 35us, 70us, 140us as discussed, at any case of switching between LB carriers, e.g., FDD to SDL and vice versa.

Proposal 4:		UE supports LB CA through the switching might be necessary to support a new UE capability, e.g., [lowBandSwitchingBandPair] per band combination, to indicate the switching capability between LBs.

Proposal 5:		Similar to the Tx switching, one of the parameters under the UE capability could also indicate the length of carrier switching period (X) of the LB-LB switching, where X should have 70us, e.g., [35, 70, 140], at any case of switching between LB carriers, e.g., FDD to SDL and vice versa.

RAN4 has considered several options regarding the time mask for switching as follows [2]:

Issue 4: Time mask for switching - Discuss the time mask for switching based on the following options: - Option 1: Switching period can be either within carrier 1 (FDD) or within carrier 2 (SDL) - Option 2: Switching period is always within the switch-from carrier - Option 3: Switching period is contained within the switching gap configured by the network - Other options are not precluded

In our understanding, multiple options can be considered for the switching/blanking location from two different motivation. First one would be to protect PDCCH, which can be located at the first few symbols of any downlink slot. In this case, the switching should be always within the switch-from carrier as Option 2. Figure 2 serves as a good starting point to discuss the switching mask from DL aspects of LB-LB CA for Option 2.

Figure 2: Proposed time mask for Option 2

Observation 5:	In order to protect PDCCH, which can be located at the first few symbols of any downlink slot, the switching should be always within the switch-from carrier as Option 2.

The other consideration can be of RAN1 specification impact on PCell operations. As shown in Figure 3 [4], a UL transmission on PCell (FDD UL) can overlap with a DL reception on SCell (SDL) due to the time advance (TA) around the switching point, e.g., the switching boundary from SDL and FDD UL. Then the simplest solution to address the collision issue could be allowing the switching/blacking period on SDL carrier only with the potential overlapping period as indicated by TTA period in Figure 4, where, TTA is the time advance as specified in TS 38.211 clause 4.3.1. In this regard, another option can be considered as “Option 4: Switching period is always within carrier 2 (SDL)”. For Option 4, the issue from overlapping with the first few symbols for PDCCH reception at the beginning of a slot on SDL can be addressed by either symbol level switching or configuring the PDCCH monitoring occasions on symbols not overlapping with the carrier switching period, which could be discussed in RAN1.

Figure 3: An example of switching boundary from SDL and FDD UL [4]

Figure 4: Proposed time mask for Option 4

Observation 6:	To mitigate the impact on RAN1 specifications regarding PCell operations, it could be considered to allow the switching/blanking period on SDL carrier only by adding “Option 4: Switching period is always within carrier 2 (SDL)”

Consequently, RAN4 needs to consider both aspects for the time being given that the issue for the switching period location is primarily concerned with the physical-layer channels or switching period pattern at least. In order to strive for the least RAN1 impact, it would be better for RAN4 to have multiple options until RAN1 concludes the issue by the next meeting.

Proposal 6:		RAN4 needs to consider both aspects for the time being given that the issue for the switching period location is primarily concerned with the physical-layer channels or switching period pattern in RAN1.

- Option 2: Switching period is always within the switch-from carrier

- (New) Option 4: Switching period is always within carrier 2 (SDL)

3	Conclusion

In this contribution, we provide our views focusing on the UE RF aspects considering the agreed assumption and restriction in the Rel-19 WID [1]. Following observations and proposals were derived.

Observation 1:	Switching scenario between LB-LB CA cases are limited to Case 1 and Case 2 with band n5 and n29, respectively, e.g., CA_n5A-n29A, which serves as a single example band combination for this WI.

Proposal 1:		RAN4 should take into account the switching scenario of Rel-19 with its assumptions and constraints for the WI discussion.

Observation 2:	Switching period values should consider UE architecture for the switching between two bands.

Observation 3:	From OFDM symbol length aspects, 35us could be the minimum switching period value, which also can guide other numbers less than 35us to 35us.

Proposal 2:		The switching period value of 13us could be excluded from Rel-19 unless it is specifically required by UE architecture for the switching between two bands, which is unlike the minimum absolute TA for a single TDD band.

Observation 4:	70us can be added as another candidate since it can be the smallest length for 15kHz SCS, which is in fact the sole assumption in the WID for both FDD low band and SDL low band switching.

Proposal 3:		70us should be counted as an option together with 35us and 140us.

Proposal 4:		UE supports LB CA through the switching might be necessary to support a new UE capability, e.g., [lowBandSwitchingBandPair] per band combination, to indicate the switching capability between LBs.

Proposal 5:		Similar to the Tx switching, one of the parameters under the UE capability could also indicate the length of carrier switching period (X) of the LB-LB switching, where X should have 70us, e.g., [35, 70, 140], at any case of switching between LB carriers, e.g., FDD to SDL and vice versa.

Observation 5:	In order to protect PDCCH, which can be located at the first few symbols of any downlink slot,

the switching should be always within the switch-from carrier as Option 2.

Observation 6:	To mitigate the impact on RAN1 specifications regarding PCell operations, it could be considered to allow the switching/blanking period on SDL carrier only by adding “Option 4: Switching period is always within carrier 2 (SDL)”

Proposal 6:		RAN4 needs to consider both aspects for the time being given that the issue for the switching period location is primarily concerned with the physical-layer channels or switching period pattern in RAN1.

- Option 2: Switching period is always within the switch-from carrier

- (New) Option 4: Switching period is always within carrier 2 (SDL)

Reference

[1] 	RP-243317, New WID on low band carrier aggregation via switching, RAN#106

[2]	R4-2503016, WF on low-low band switching, RAN4#114

[3]	R4-2502877, LS on switching periods for low-low band switching, RAN4#114

[4]	R4-2500313, Initial Views on UE RF requirements for LB CA via switching, RAN4#114