3GPP TSG-RAN WG4 Meeting #114-bis	R4-2504474

Wuhan, China, 7th – 11th April 2025

Source:	Nokia

Title:	Discussion on NR NTN SAN requirement for Ku band

Agenda item:	7.11.5

Document for:	Approval

Introduction

For WI on the introduction of NTN Ku Band [1], RAN4#114 has agreed the following Way-forward [2] regarding the SAN requirement for FR1-NTN and FR2-NTN Ku bands.

Sub-topic 2-1: General issues for Ku band SAN RF requirements Issue 2-1-1: Discussion on the supported SAN type for NR NTN Ku band - Regarding the SAN requirement for the NR NTN Ku band: - Define SAN type 1-O requirements for the FR1-NTN Ku bands - Define SAN type 2-O requirements for the FR2-NTN Ku bands - The SAN type 1-H will not be defined for the FR1-NTN Ku bands Issue 2-1-2: Alignment of SAN requirement between FR1-NTN and FR2-NTN - Aim to align the following SAN requirements between the FR1-NTN Ku bands and the FR2-NTN Ku bands at least - The tolerance for the radiated transmit power and SAN output power for FR1-NTN and FR2-NTN - The spurious frequency range for FR1-NTN and FR2-NTN - Companies are encouraged to further check other requirements between FR1-NTN and FR2-NTN, including the following aspects - Companies are encouraged to further check whether the RE power control dynamic range, OTA sensitivity and Rx dynamic range requirements shall be specified for NTN Ku-bands - FFS on other requirements

In this contribution, we discuss how we harmonize the requirement between FR1-NTN and FR2-NTN Ku bands.

Discussion

RF requirement for SAN is specified in TS 38.108, where three requirement types 1-H, 1-O and 2-O are currently specified. The most straightforward way from specification work perspective is to reuse the existing 1-O requirement for FR1-NTN Ku bands (i.e., n248 and n247 in [3]) and 2-O requirement for FR2-NTN Ku bands (i.e., n509 and 508 in [3]).

The most straightforward way (from specification work perspective) is to reuse the existing 1-O requirement for FR1-NTN Ku bands (i.e., n248 and n247) and 2-O requirement for FR2-NTN Ku bands (i.e., n509 and 508).

However, there have been a request from some satellite operators to align the requirement between FR1-NTN and FR2-NTN as they use the same frequency range with different numerologies. As mentioned above, RAN4#114 agreed at least to consider this for Tx power tolerance and spurious frequency range, as they have minimum specification impact.

The existing 1-O requirement originally comes from the requirement of TN FR1 bands. It was first derived for bands around 2 GHz (e.g., some fundamental assumptions are originated from IMT-2000) and they have been extended up to 7 GHz band by now. On the other hand, the existing 2-O requirement comes from the requirement for TN FR2 bands, which was first derived for 28 GHz band. The transmissions and reception requirements are generally more stringent for FR1 than FR2. It is most likely that the existing 1-O requirement (applicable up to 7GHz today in TN) is too stringent for NTN Ku bands, thus some relaxation may need to be considered.

However, some of the existing 1-O requirement is likely too stringent for Ku bands.

FR2-NTN requirement is considered as baseline for power tolerance and spurious frequency range also for Ku bands with FR1 numerologies (i.e., n248 and n247).

Also for other requirements, FR2-NTN requirement is better be applied to FR1-NTN when the existing FR-NTN requirement is too stringent for Ku bands.

Also for other requirements, FR2-NTN requirement is better be applied to Ku bands with FR1 numerologies (i.e., n248 and n247) if the existing FR1-NTN requirement is too stringent for Ku bands.

As agreed in the WF [2], companies are encouraged to further check whether the RE power control dynamic range, OTA sensitivity and Rx dynamic range requirements shall be specified for NTN Ku-bands. It is FFS on other requirements.

RE power control required for TN is currently specified for FR1-NTN. This feature may be used to adjust wide power dynamic range due to different path loss among UEs; however, the need of such fine power control is not clear for NTN because all UEs are anyway far from SAN and path loss difference among UEs is not as large as TN deployment. (For example, UE could be located very close to a base station site in TN. This scenario is unlikely in NTN.) Therefore, RE power control dynamic range is not essential for NTN in Ku bands.

RE power control dynamic range for NTN is not as essential as TN.

OTA sensitivity and Rx dynamic range requirements are specified in 1-O. This can be considered not applicable to Ku bands with FR1 numerologies.

OTA sensitivity and Rx dynamic range requirements can be omitted for Ku bands with FR1 numerologies.

Conclusion

The following Observations and Proposals were made:

Observation 1: The most straightforward way (from specification work perspective) is to reuse the existing 1-O requirement for FR1-NTN Ku bands (i.e., n248 and n247) and 2-O requirement for FR2-NTN Ku bands (i.e., n509 and 508).

Observation 2: However, some of the existing 1-O requirement is likely too stringent for Ku bands.

Proposal 1: FR2-NTN requirement is considered as baseline for power tolerance and spurious frequency range also for Ku bands with FR1 numerologies (i.e., n248 and n247).

Proposal 2: Also for other requirements, FR2-NTN requirement is better be applied to Ku bands with FR1 numerologies (i.e., n248 and n247) if the existing FR1-NTN requirement is too stringent for Ku bands.

Observation 3: RE power control dynamic range for NTN is not as essential as TN.

Observation 4: OTA sensitivity and Rx dynamic range requirements can be omitted for Ku bands with FR1 numerologies.

- References

- RP-250799 Revised WID: Introduction of Ku bands for NR NTN, RAN#107

- R4-2502288 WF for [114][314] NR_NTN_Ku_Band_UE_SAN_RF, RAN4#114

- R4-2502290 Way Forward for [114][313] NR_NTN_Ku_Band_General, RAN4#114