3GPP TSG-RAN WG4 Meeting #114-bis				                    					  R4-2504649 Wuhan, China 7th - 11th April 2025

Agenda item:			7.10.5

Source:	Moderator (Ericsson)

Title:	Topic summary for [114bis][324] NR_IoT_NTN_less_than_5MHz_demod

Document for:	Information

# Introduction

Briefly introduce background, the scope of this summary (e.g. list of treated agenda items).

This document is the topic summary for demodulation requirements for R19 IoT (Internet of Things) NTN (non-terrestrial network) less than 5MHz, including the following topics covered

- Topic#1: Demodulation performance requirements (AI 7.10.3.6)

Recommended issues for online discussion:

- Issue 1-1, 1-2, 1-3, 2-1, 2-2, 2-3

# Topic #1: Demodulation performance requirements (AI 7.10.3.6)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2503245 | QUALCOMM Europe Inc. - Spain | Observation 1: RAN4 has introduced PBCH and PDCCH demodulation requirements for NR with bandwidth less than 5MHz in FR1 FDD. Observation 2: According to the UE capability for this feature, PBCH will include 12 PRBs, while CORESET0 will include 15 PRBs. Proposal 1: Consider only 3MHz channel bandwidth with 15 PRB BWP for this WI. Proposal 2: Discuss the feasibility of introducing PBCH requirements for NR-NTN in FR1 FDD with 12 PRBs for PBCH. Proposal 3: Discuss the feasibility of introducing PDCCH requirements for NR-NTN in FR1 FDD with 15 PRBs for PDCCH CORESET0. Proposal 4: Don’t define PDSCH and CSI reporting requirements for NR-NTN with bandwidth less than 5MHz. |
| R4-2503910 | Ericsson | Observation 1: Consider introducing new PUSCH and PUCCH demodulation requirements for 3 MHz CBW with 15 KHz SCS, due to different channel models and FRC parameters for NTN. Observation 2: Introduce limited test cases of PUSCH and PUCCH demodulation requirements for 3 MHz CBW with 15 KHz SCS. Observation 3: Regarding test parameter configuration, existing SAN FR1 could be followed as the starting point. Based on the above discussion, following proposals are made: Proposal 1: Introduce new PUSCH demodulation requirements for NTN with 3 MHz CBW and 15 KHz SCS, if 3 MHz only support for SAN is needed or performance difference is observed from simulations between 3 and 5 MHz CBW. Proposal 2: Introduce new PUCCH demodulation requirements for NTN with 3 MHz CBW and 15 KHz SCS, if 3 MHz only support for SAN is needed or performance difference is observed from simulations between 3 and 5 MHz CBW. Proposal 3: Do not consider PRACH to define SAN demodulation requirements for NTN with 3 MHz CBW. Proposal 4: Consider defining new PUSCH FR1 3 MHz demodulation requirements for SAN: - Requirements for PUSCH with transform precoding disabled - Requirements for PUSCH with transform precoding enabled - Requirements for PUSCH repetition Type A Proposal 5: Consider defining new PUCCH FR1 3 MHz SAN demodulation requirements for format 0/1/2/3/4. Observation 4: RAN4 RF added 3MHz CBW with SCS=15kHz in FR1-NTN bands n254, n255, n256. Observation 5: RAN4 did not define PDSCH demodulation requirements for 3MHz CBW in Rel-18 because RAN4 assumed UE supporting 3MHz CBW should also support wider channel bandwidth such as 5MHz. Proposal 6: No new PDSCH demodulation requirements with 3MHz CBW in TS 38.101-5, as same as Rel-18. UE supporting 3MHz in NTN bands should pass the existing PDSCH demodulation requirements. Proposal 7: No new PDCCH/PBCH demodulation requirements with puncturing in TS 38.101-5. UE supporting 3MHz in NTN bands should pass the PDCCH/PBCH demodulation requirements specified for 3MHz CBW in Rel-18 TS38.101-4. Proposal 8: No new CSI reporting for 3MHz CBW in TS 38.101-5. |
| R4-2504538 | Samsung | UE demodulation requirement Observation 1: RAN4 has already introduced the punctured PDCCH and punctured PBCH requirement in Rel-18 for NR supporting less than 5MHz. No new PDSCH and CSI reporting requirements introduced Observation 2: RAN4 has only introduced the limited PDSCH performance requirement for NTN scenario. Proposal 1 No PBCH requirement introduced for NTN with less than 5MHz Proposal 2 No PDCCH requirement introduced for NTN with less than 5MHz Proposal 3 No CSI reporting requirement introduced for NTN with less than 5MHz Proposal 4 RAN4 should introduce PDSCH requirement for FR1 NTN with less than 5MHz SAN demodulation requirement Observation 3: No PUSCH requirements with 3MHz bandwidth for both non-HST and HST condition, as well as UL timing adjustment requirement were introduced in Rel-18 NR with less than 5MHz Observation 4: Only introduce new demodulation requirement for PUCCH format 2 with 3MHz and introduce the test applicability rule for new introduced UCI performance with 3MHz and legacy 5MHz only for format 2 Observation 5: No new PRACH requirement for NR with less than 5MHz and introduce the applicability rule for BS supporting less than 5MHz carrier bandwidth. Proposal 5: No New SAN PUSCH requirement with 3MHz bandwidth for PUSCH with transform precoding disabled, with transform precoding enabled, UL timing adjustment, PUSCH repetition Type A and PUSCH with DM-RS bundling if no SAN that supporting only less than 5MHz Proposal 6: FFS to introduce SAN PUCCH requirement with 3MHz bandwidth under NR NTN channel condition Proposal 7: No new SAN PRACH requirement for NTN with Less than 5MHz, and introducing the test applicability rule for SAN supporting less than 5MHz carrier bandwidth Proposal 8: Reuse the existing FR1 NTN PDSCH test cases as a starting point to specify the related PDSCH requirement with supporting less than 5MHz Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 1-1; TBD; 3/ 15; QPSK, 0.30; NTN-TDLA100-200; 1x2, ULA Low; 70; TBD. 1-2; TBD; 3/ 15; 16QAM, 0.48; NTN-TDLC5-200; 1x2, ULA Low; 70; TBD. 1-3; TBD; 3/ 15; QPSK, 0.30; NTN-TDLC5-200; 1x2, ULA Low; 70; TBD. 1-4; TBD; 3/ 15; QPSK, 0.30; NTN-TDLA100-200; 1x2, ULA Low; 70; TBD. Note1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled.. Proposal 9: The following simulation assumption can be considered for evaluation SAN PUCCH performance with less than 5MHz PUCCH format 0 Parameter; Test. Number of UCI information bits; 1. Number of PRBs; 1. First PRB prior to frequency hopping; 0. Intra-slot frequency hopping; Enabled. First PRB after frequency hopping; Option 1: The largest PRB index – (nrofPRBs – 1) Option 2: 11. Group and sequence hopping; neither. Hopping ID; 0. Initial cyclic shift; 0. First symbol; 12 for 2 symbols. PUCCH format 1 Parameter; Test. Number of information bits; 2. Number of PRBs; 1. Number of symbols; 14. First PRB prior to frequency hopping; 0. Intra-slot frequency hopping; enabled. First PRB after frequency hopping; Option 1: The largest PRB index – (nrofPRBs – 1) Option 2: 11. Group and sequence hopping; neither. Hopping ID; 0. Initial cyclic shift; 0. First symbol; 0. Index of orthogonal cover code (timeDomainOCC); 0. PUCCH format 2 Parameter; Value. Modulation order; QSPK. First PRB prior to frequency hopping; 0. Intra-slot frequency hopping; enabled. Frist PRB after frequency hopping; Option 1: The largest PRB index – (Number of PRBs – 1) Option 2: 3. Number of PRBs; 9. Number of symbols; 2. The number of UCI information bits; 22. First symbol; 12. DM-RS sequence generation; NID0=0. PUCCH format 3 Parameter; Test. Modulation order; QPSK. First PRB prior to frequency hopping; 0. Intra-slot frequency hopping; enabled. First PRB after frequency hopping; Option 1: The largest PRB index – (Number of PRBs – 1) Option 2:11. Group and sequence hopping; neither. Hopping ID; 0. Number of PRBs; 1. Number of symbols; 14. The number of UCI information bits; 16. First symbol; 0. PUCCH format 4 Parameter; Value. Modulation order; QPSK. First PRB prior to frequency hopping; 0. Number of PRBs; 1. Intra-slot frequency hopping; enabled. First PRB after frequency hopping; Option 1: The largest PRB index – (Number of PRBs – 1) Option 2: 11. Group and sequence hopping; neither. Hopping ID; 0. Number of symbols; 14. The number of UCI information bits; 22. First symbol; 0. Length of the orthogonal cover code; n2. Index of the orthogonal cover code; n0. Multi-slot PUCCH format 1 Parameter; Test. Number of information bits; 2. Number of PRBs; 1. Number of symbols; 14. First PRB prior to frequency hopping; 0. Intra-slot frequency hopping; disabled. Inter-slot frequency hopping; enabled. First PRB after frequency hopping; Option 1: The largest PRB index – (nrofPRBs – 1) Option 2: 11. Group and sequence hopping; neither. Hopping ID; 0. Initial cyclic shift; 0. First symbol; 0. Index of orthogonal cover code (timeDomainOCC); 0. Number of slots for PUCCH repetition; 2. |
| R4-2504315 | Huawei, HiSilicon | Proposal 1: No specification changes in TS 38.101-5. Proposal 2: Only define PUCCH format 2 cases for SAN. Proposal 3: Add same applicability rule in TS 38.181. |

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-Topic 1: SAN demodulation requirements

#### Issue 1-1: PUSCH – Scope of the SAN demodulation requirements

Proposals:

- Proposal 1 (Ericsson): Introduce new PUSCH demodulation requirements for NTN with 3 MHz CBW and 15 KHz SCS, if 3 MHz only support for SAN is needed or performance difference is observed from simulations between 3 and 5 MHz CBW.

  - Requirements for PUSCH with transform precoding disabled

  - Requirements for PUSCH with transform precoding enabled

  - Requirements for PUSCH repetition Type A

  - (Ericsson’s observation: Consider introducing new PUSCH demodulation requirements with limited test cases for 3 MHz CBW with 15 KHz SCS, due to different channel models and FRC parameters for NTN.)

- Proposal 2 (Samsung): No New SAN PUSCH requirement with 3MHz bandwidth for PUSCH with transform precoding disabled, with transform precoding enabled, UL timing adjustment, PUSCH repetition Type A and PUSCH with DM-RS bundling if no SAN that supporting only less than 5MHz

  - (Samsung’s observation: No PUSCH requirements with 3MHz bandwidth for both non-HST and HST condition, as well as UL timing adjustment requirement were introduced in Rel-18 NR with less than 5MHz)

- Proposal 3 (Huawei): No new PUSCH requirements

Recommended WF:

- Discuss first if there is a SAN that only support 3MHz

#### Issue 1-2: PUCCH – Scope of the SAN demodulation requirements

Proposals:

- Proposal 1 (Ericsson): Introduce new PUCCH demodulation requirements for NTN with 3 MHz CBW and 15 KHz SCS, if 3 MHz only support for SAN is needed or performance difference is observed from simulations between 3 and 5 MHz CBW.

  - Format 0/1/2/3/4.

  - (Ericsson’s observation: Consider introducing new PUCCH demodulation requirements prioritizing frequency hopping enabled test cases for 3 MHz CBW with 15 KHz SCS for NTN.)

- Proposal 2 (Samsung): FFS to introduce SAN PUCCH requirement with 3MHz bandwidth under NR NTN channel condition

  - (Samsung’s observation: Only introduce new demodulation requirement for PUCCH format 2 with 3MHz and introduce the test applicability rule for new introduced UCI performance with 3MHz and legacy 5MHz only for format 2)

- Proposal 3 (Huawei): Define PUCCH format 2 cases for SAN

Recommended WF:

- Discuss online

#### Issue 1-3: PRACH – Scope of the SAN demodulation requirements

Proposals:

- Proposal 1 (Ericsson): Do not consider PRACH to define SAN demodulation requirements for NTN with 3 MHz CBW

- Proposal 2 (Samsung, Huawei): No new SAN PRACH requirement for NTN with Less than 5MHz, and introducing the test applicability rule for SAN supporting less than 5MHz carrier bandwidth

Recommended WF:

- No new PRACH requirements

- Discuss further whether to introduce the test applicability rule

#### Issue 1-4: Simulation assumption for SAN requirements

Proposals:

- Proposal 1 (Samsung): The following simulation assumption can be considered for evaluation SAN PUCCH performance with less than 5MHz

PUCCH format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | Option 1: The largest PRB index – (nrofPRBs – 1) Option 2: 11 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 12 for 2 symbols |

PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | Option 1: The largest PRB index – (nrofPRBs – 1) Option 2: 11 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |

PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | Option 1: The largest PRB index – (Number of PRBs – 1) Option 2: 3 |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |

PUCCH format 3

| Parameter | Test |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | Option 1: The largest PRB index – (Number of PRBs – 1) Option 2:11 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| The number of UCI information bits | 16 |
| First symbol | 0 |

PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | Option 1: The largest PRB index – (Number of PRBs – 1) Option 2: 11 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |

Multi-slot PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | Option 1: The largest PRB index – (nrofPRBs – 1) Option 2: 11 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |

Recommended WF:

- Pending on the result of issue 1-2

### Sub-Topic 2: UE demodulation requirements

#### Issue 2-1: PDSCH – Scope of the UE demodulation requirements

Proposals:

- Proposal 1 (QC, Ericsson, Huawei ): No new PDSCH requirements

  - (Ericsson’s observation: RAN4 did not define PDSCH demodulation requirements for 3MHz CBW in Rel-18 because RAN4 assumed UE supporting 3MHz CBW should also support wider channel bandwidth such as 5MHz.)

- Proposal 2 (Samsung): Introduce PDSCH requirement for FR1 NTN with less than 5MHz

  - (Samsung’s observation: RAN4 has already introduced the punctured PDCCH and punctured PBCH requirement in Rel-18 for NR supporting less than 5MHz. No new PDSCH and CSI reporting requirements introduced)

Recommended WF:

- Discuss online

#### Issue 2-2: PDCCH – Scope of the UE demodulation requirements

Proposals:

- Proposal 1 (Ericsson, Samsung, Huawei): No new PDCCH requirements

  - (Ericsson’s observation: UE supporting 3MHz in NTN bands should pass the PDCCH/PBCH demodulation requirements specified for 3MHz CBW in Rel-18, according to the applicability rule in  TS 38.101-5 Table 8.2.1.1.2-1.)

- Proposal 2 (QC): Discuss the feasibility of introducing PDCCH requirements

  - 15PRB for PDCCH CORESET0

Recommended WF:

- Discuss online

#### Issue 2-3: PBCH – Scope of the UE demodulation requirements

Proposals:

- Proposal 1 (Ericsson, Samsung, Huawei): No new PBCH requirements

  - (Ericsson’s observation: UE supporting 3MHz in NTN bands should pass the PDCCH/PBCH demodulation requirements specified for 3MHz CBW in Rel-18, according to the applicability rule in  TS 38.101-5 Table 8.2.1.1.2-1.)

- Proposal 2 (QC): Discuss the feasibility of introducing PBCH requirements

  - 12PRBs

Recommended WF:

- Discuss online

#### Issue 2-4: CSI reporting – Scope of the UE demodulation requirements

Proposals:

- Proposal 1 (QC, Ericsson, Samsung, Huawei): No new CSI reporting requirements

Recommended WF:

- No new CSI reporting requirements

#### Issue 2-5: Simulation assumption for UE requirement (if agreed to define PDSCH demodulation requirements)

Proposals:

- Proposal 1 (Samsung): Reuse the existing FR1 NTN PDSCH test cases as a starting point to specify the related PDSCH requirement with supporting less than 5MHz

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | TBD | 3/ 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | TBD |
| 1-2 | TBD | 3/ 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | TBD |
| 1-3 | TBD | 3/ 15 | QPSK, 0.30 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | TBD |
| 1-4 | TBD | 3/ 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | TBD |
| Note1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. |  |  |  |  |  |  |  |

Recommended WF:

- Pending on the result of issue 2-1