3GPP TSG-RAN4 Meeting #114bis	R4-2504683

# Wuhan, China, 7th – 11th April 2025

Agenda item:			7.12.7

Source:	Moderator (CMCC)

Title:	Topic summary for [114bis][125] NR_ATG_enh

Document for:	Information

Introduction

RAN#106 meeting approved RP-242385 Revised WID on Enhancements for Air-to-ground network for NR in Rel-19.

This thread focuses on UE RF requirement parts for Rel-19 ATG enhancement and corresponds to agenda, 7.12.1.

Previous approved WF are listed as below:

- R4-2406594, WF on Rel-19 ATG UE requirements, CMCC, RAN4#110 bis

- R4-2410687, WF on Rel-19 ATG UE RF requirements, CMCC, RAN4#111

- R4-2414322, WF on Rel-19 ATG UE RF requirements, CMCC. RAN4#112

- R4-2417121, WF on UE RF for ATG enhancement, CMCC, RAN4#112bis

- R4-2420338, WF on NR ATG UE RF, CMCC, RAN4#113

- R4-2502952, WF on UE RF requirements for ATG, RAN4#114

Topic #1: R19 UE RF for inter-band CA

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2503396 | CMCC | Proposal 1: Consider improve the performance parameters of the n3 Rx filter. Proposal 2: MSD values of ATG UE should be specified based on separated antenna architecture. Table 3 CA_n3-n39 with band n39 UL MSD test configuration UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n39; n3; 1900; 30; 15; 160 (RBstart=0); 1817.5; 25; MSD Table X; >ACLR2. Table X CA_n3-n39 with band n39 UL MSD requirements for different output powers Output power; MSD (Based on Antenna Architecture with common antenna); MSD (Based on Antenna Architecture with separated antenna). 23dBm; 3.6; 0.6. 24dBm; 4.2; 1.2. 25dBm; 4.8; 1.9. 26dBm; 5.5; 2.6. 27dBm; 6.3; 3.3. 28dBm; 7.1; 4.1. 29dBm; 7.9; 4.9. 30dBm; 8.8; 5.8. 31dBm; 9.7; 6.7. 32dBm; 10.5; 7.6. 33dBm; 11.5; 8.5. 34dBm; 12.4; 9.4. 35dBm; 13.3; 10.4. 36dBm; 14.3; 11.4. 37dBm; 15.2; 12.3. 38dBm; 16.2; 13.3. 39dBm; 17.2; 14.3. 40 dBm; 18.2; 15.3. Table 6 CA_n3-n39 with band n39 UL MSD test configuration UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n3; n39; [1722.5]; [25]; [15]; [160 (RBstart=0)]; [1900]; [30]; MSD Table Y; [>ACLR2]. Table Y CA_n3-n39 with band n3 UL MSD requirements for different output powers Output power; MSD (Based on Antenna Architecture with common antenna); MSD (Based on Antenna Architecture with separated antenna). 23dBm; 2.4; 0.16. 24dBm; 2.5; 0.16. 25dBm; 2.7; 0.16. 26dBm; 2.8; 0.16. 27dBm; 2.9; 0.16. 28dBm; 3; 0.16. 29dBm; 3; 0.17. 30dBm; 3.1; 0.18. 31dBm; 3.1; 0.19. 32dBm; 3.2; 0.2. 33dBm; 3.2; 0.2. 34dBm; 3.3; 0.3. 35dBm; 3.3; 0.3. 36dBm; 3.4; 0.4. 37dBm; 3.6; 0.6. 38dBm; 3.8; 0.8. 39dBm; 4.2; 1.2. 40 dBm; 4.7; 1.7. Proposal 3: It is hard to determine the frequency separation between two bands, and this issue should be analyzed case by case. |
| R4-2503694 | Huawei, HiSilicon | Observation 1: 5MHz frequency gap was reserved for the band 3 and 39 spectrum allocation in China. Observation 2: From ATG operator’s perspective, the frequency separation between band 3 DL and band 39 UL is about 55MHz. Proposal 1: For ATG UE, RAN4 can specify the following MSD requirements for DL band n3 with band n39 UL interreference under different output powers considering operator’s spectrum allocation and 5MHz frequency gap in 1880~1885MHz. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n39; n3; 1900; 30; 15; 160 (RBstart=0); 1817.5; 25; MSD Table 4; ACLR2 and >ACLR2. Proposal 2: For ATG UE, RAN4 can specify the following MSD requirements for DL band n39 with band n3 UL interreference UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n3; n39; 1722.5; 25; 15; 133 (RBstart=0); 1900; 30; MSD Table 5; >ACLR2. |
| R4-2503723 | LG Electronics France | Proposal 1: Cross band isolation MSD value for n39 UL with 23 dBm and DL n3 is 5.5 dB Proposal 2: Cross band isolation MSD value for n39 UL with 23 dBm and DL n3 is 0 (-4.1) dB Proposal 3: Source of cross-band isolation interference for UL n39, DL n3 should be >ACLR2 Proposal 4: Source of cross-band isolation interference for UL n3, DL n39 should be >ACLR2 Proposal 5: Make the new MSD table for ATG UE as shown below Table 7 Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a aggressor NR ATG single UL band for DL NR ATG CA FR1 UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n39; n3; 1900; 30; 15; 160 (RBstart=0); 1817.5; 25; [Default MSD(5.5 dB) + △MSDUE for xxdBm]; >ACLR2. n3; n39; [1722.5]; [25]; [15]; [160 (RBstart=0)]; [1900]; [30]; [Default MSD(-4.1 dB) + △MSDUE for xxdBm]; >ACLR2. If the MSD value is less than 0, the MSD value is considered as 0.. Table 8 △MSD for ATG UE Output power; MSD (Based on Antenna Architecture with common antenna). △MSDUE for 23dBm; 0. △MSDUE for 24dBm; 0.8. △MSDUE for 25dBm; 1.6. △MSDUE for 26dBm; 2.4. △MSDUE for 27dBm; 3.2. △MSDUE for 28dBm; 4. △MSDUE for 29dBm; 4.8. △MSDUE for 30dBm; 5.6. △MSDUE for 31dBm; 6.4. △MSDUE for 32dBm; 7.2. △MSDUE for 33dBm; 8. △MSDUE for 34dBm; 8.8. △MSDUE for 35dBm; 9.6. △MSDUE for 36dBm; 10.4. △MSDUE for 37dBm; 11.2. △MSDUE for 38dBm; 12. △MSDUE for 39dBm; 12.8. △MSDUE for 40dBm; 13.6. Proposal 6: Support option 2 with X = 1 GHz - ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam is possible for specific band combination. - Antenna array type for inter-band CA is highly depending on frequency range of operating bands and UE implementation. - Option 2: Case 4 UE implementation is possible for CA_n3-n39 that DL frequency separation between two bands are less than [X=1 GHz]. |
| R4-2504038 | Ericsson | Observation 1 The feasibility of the ATG UE with an antenna array on the bands of PCC and SCC and one common Rx beam is highly dependent on the frequency separation. |
| R4-2504057 | Qualcomm Incorporated | Proposal 1: RAN4 to take the MSD values in Table 1 to define the MSD requirements for CA_n3-n39 with band n39 UL MSD requirements for different output powers. Proposal 2: RAN4 should only define one set of MSD requirements with common assumptions. Proposal 3: From RF perspective, we suggest RRM session to define the MRTD requirements based on the worst-case assumption for UE antenna type. Proposal 4: There is no need to further discuss ATG UE antenna type in RAN4 RF session which is highly depending on UE implementation. |
| R4-2504356 | ZTE Corporation, Sanechips | In this contribution, we shared some initial views on the RF requirements for ATG UE supporting inter-band DL CA in Rel-19 and proposals are made as following: Observation 1:MSD for 23dBm to 40dBm transmission power would be ranging from 4.75dB to 17.38dB which will bring the significant performance degradation of band n3 DL reception. Proposal 1: for CA_n3-n39 with uplink transmission as n39, MSD requirement could defined as following: CA_n3-n39 with UL n39; MSD. 23; 4.76dB. 24; 5.15dB. 25; 5.59dB. 26; 6.09dB. 27; 6.64dB. 28; 7.24dB. 29; 7.90dB. 30; 8.60dB. 31; 9.36dB. 32; 10.15dB. 33; 10.97dB. 34; 11.83dB. 35; 12.71dB. 36; 13.62dB. 37; 14.54dB. 38; 15.48dB. 39; 16.43dB. 40; 17.38dB. Proposal 2: for Rx antenna gain for CA_n3-n39, propose to go with option 1 to confirm the this specific band combination. - Option 1: Case 4 UE implementation is possible for CA_n3-n39 that frequency ranges between two bands are close to each other. |

Open issues summary

Sub-topic 1-1 MSD for inter-band CA_n3A-n39A with UL n39

Agreement during last meeting

Issue 2-1: Assumed parameters when supporting n39 UL for CA_n3A-n39A with common antenna. Agreement: - Companies are encouraged to provide MSD values based on the following parameters. Parameters; Values. Antenna Isolation; 20dB. The rejection for band n3 Rx filter at 1885~1915MHz; [35dB]. n39 Tx filter rejection at 1805~1830MHz; [50dB]. - Above parameters are only applicable for ATG UE with dedicated filter design Issue 2-2: MSD for CA_n3A-n39A with n39 UL Agreement: - Companies are encouraged to provide MSD values based on the following Tables. - MSD analysis based on separated antenna are not precluded. - All of these MSD values are only applicable to ATG UE. - Further study on how to capture these MSD values to in the specification. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n39; n3; 1900; 30; 15; 160 (RBstart=0); 1817.5; 25; MSD Table X; >ACLR2. Table X CA_n3-n39 with band n39 UL MSD requirements for different output powers Output power; MSD (Based on Antenna Architecture with common antenna). 23dBm; TBD. 24dBm; TBD. …; …. 39dBm; TBD. 40 dBm; TBD.

Issue 1-1-1 Assumed parameters when supporting n39 UL for CA_n3A-n39A

- Proposals

  - Option 1: Improve the performance parameters of the n3 Rx filter. (CMCC)

  - Option 2: Others.

- Recommended WF

  - Check with two Option 1.

Issue 1-1-2 Architecture for CA_n3A-n39A

- Proposals

  - Option 1: Define MSD requirements with separated antenna architecture. (CMCC)

  - Option 2: Define one set of MSD requirements with common assumptions. (Qualcomm)

- Recommended WF

  - Check with these two Options.

Issue 1-1-3: MSD for CA_n3A-n39A with n39 UL

- Proposals

  - Option 1: MSD requirements with common antenna architecture.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1817.5 | 25 | MSD Table X | >ACLR2 |

Table X CA_n3-n39 with band n39 UL MSD requirements for different output powers

| Output power | CMCC | Huawei | LGE | Qualcomm | ZTE | Linear Average |
| --- | --- | --- | --- | --- | --- | --- |
| 23dBm | 3.6 | 3.4 | 5.5 | 5.8 | 4.76 | 4.7 dB |
| 24dBm | 4.2 | 3.57 | 6.3 | 6.3 | 5.15 | 5.2 dB |
| 25dBm | 4.8 | 3.74 | 7.1 | 6.8 | 5.59 | 5.8 dB |
| 26dBm | 5.5 | 3.94 | 7.9 | 7.3 | 6.09 | 6.4 dB |
| 27dBm | 6.3 | 4.16 | 8.7 | 8.0 | 6.64 | 7.0 dB |
| 28dBm | 7.1 | 4.42 | 9.5 | 8.6 | 7.24 | 7.7 dB |
| 29dBm | 7.9 | 4.72 | 10.3 | 5.8 | 7.90 | 7.7 dB |
| 30dBm | 8.8 | 5.07 | 11.1 | 10.1 | 8.60 | 9.1 dB |
| 31dBm | 9.7 | 5.49 | 11.9 | 11.0 | 9.36 | 10.0 dB |
| 32dBm | 10.5 | 5.96 | 12.7 | 11.8 | 10.15 | 10.7 dB |
| 33dBm | 11.5 | 6.51 | 13.5 | 12.8 | 10.97 | 11.6 dB |
| 34dBm | 12.4 | 7.13 | 14.3 | 13.7 | 11.83 | 12.5 dB |
| 35dBm | 13.3 | 7.84 | 15.1 | 14.7 | 12.71 | 13.3 dB |
| 36dBm | 14.3 | 8.63 | 15.9 | 15.7 | 13.62 | 14.3 dB |
| 37dBm | 15.2 | 9.50 | 16.7 | 16.8 | 14.54 | 15.2 dB |
| 38dBm | 16.2 | 10.46 | 17.5 | 17.9 | 15.48 | 16.1 dB |
| 39dBm | 17.2 | 11.50 | 18.3 | 19.0 | 16.43 | 17.1 dB |
| 40 dBm | 18.2 | 12.63 | 19.1 | 20.2 | 17.38 | 18.1 dB |

  - Option 2: MSD requirements with separated antenna architecture. (CMCC)

- Table X CA_n3-n39 with band n39 UL MSD requirements for different output powers

| Output power | MSD (dB) |
| --- | --- |
| 23dBm | 0.6 |
| 24dBm | 1.2 |
| 25dBm | 1.9 |
| 26dBm | 2.6 |
| 27dBm | 3.3 |
| 28dBm | 4.1 |
| 29dBm | 4.9 |
| 30dBm | 5.8 |
| 31dBm | 6.7 |
| 32dBm | 7.6 |
| 33dBm | 8.5 |
| 34dBm | 9.4 |
| 35dBm | 10.4 |
| 36dBm | 11.4 |
| 37dBm | 12.3 |
| 38dBm | 13.3 |
| 39dBm | 14.3 |
| 40 dBm | 15.3 |

- Recommended WF

  - Further check the MSD in the table.

Issue 1-1-4: How to incorporate MSD requirements for different power levels into the spec.

- Proposals

  - Option 1:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1817.5 | 25 | [Default MSD(TBD dB) +/- △MSDUE for xxdBm] | >ACLR2 |

Table X △MSD for ATG UE

| Output power | MSD (Based on Antenna Architecture with common antenna) |
| --- | --- |
| △MSDUE for 23dBm | [TBD] |
| △MSDUE for 24dBm | [TBD] |
| … | … |
| △MSDUE for 39dBm | [TBD] |
| △MSDUE for 40dBm | [TBD] |

  - Option 2:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1817.5 | 25 | MSD Table X | >ACLR2 |

Table X MSD requirements for different output powers

| Output power | MSD (dB) |
| --- | --- |
| 23dBm | [TBD] |
| 24dBm | [TBD] |
| … | … |
| 39dBm | [TBD] |
| 40 dBm | [TBD] |

  - Option 3:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1817.5 | 25 | MSD Table X | >ACLR2 |

Table X MSD requirements for different output powers

| Output power | MSD (dB) |
| --- | --- |
| [23 dBm] | [TBD] |
| [24~30 dBm] | [TBD] |
| [31~35 dBm] | [TBD] |
| [36~40 dBm] | [TBD] |

  - Option 4: others.

- Recommended WF

  - Further check these above options.

Sub-topic 1-2 MSD for inter-band CA_n3A-n39A with UL n3

Agreement during last meeting

Issue 2-3: Assumed parameters for CA_n3A-n39A with n3 UL WF: - Use the following parameters in the approved Tdoc R4-2410641 with 20dB antenna isolation as baseline to analysis the MSD value for n39 DL. - Other parameters are not precluded. Parameters; Value. Antenna isolation; [20dB]. Tx attenuation at 1880~1920MHz for band n3 duplexer; [30dB]. Rx attenuation at 1710~1785MHz for band n3+n39 Rx filter; [40dB]. Issue 2-4: MSD for CA_n3A-n39A with n3 UL WF: - Companies are encouraged to provide MSD values based on the following Tables. - Other MSD test configurations are not precluded. - All of these MSD values are only applicable to ATG UE. - Further study on how to capture these MSD values to in the specification. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n3; n39; [1722.5]; [25]; [15]; [160 (RBstart=0)]; [1900]; [30]; MSD Table Y; [>ACLR2]. Table Y CA_n3-n39 with band n3 UL MSD requirements for different output powers Output power; MSD (Based on Antenna Architecture with common antenna). 23dBm; TBD. 24dBm; TBD. …; …. 39dBm; TBD. 40 dBm; TBD.

Issue 1-2-1: MSD for CA_n3A-n39A with n3 UL

- Proposals

  - Option 1: MSD requirements with common antenna architecture.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n39 | [1722.5] | [25] | [15] | [160 (RBstart=0)] | [1900] | [30] | MSD Table Y | [>ACLR2] |

Table X CA_n3-n39 with band n39 UL MSD requirements for different output powers

| Output power | CMCC | Huawei | LGE |
| --- | --- | --- | --- |
| 23dBm | 2.4 | 0.16 | 0 |
| 24dBm | 2.5 | 0.19 | 0 |
| 25dBm | 2.7 | 0.24 | 0 |
| 26dBm | 2.8 | 0.29 | 0 |
| 27dBm | 2.9 | 0.36 | 0 |
| 28dBm | 3 | 0.44 | 0 |
| 29dBm | 3 | 0.55 | 0.7 |
| 30dBm | 3.1 | 0.68 | 1.5 |
| 31dBm | 3.1 | 0.84 | 2.3 |
| 32dBm | 3.2 | 1.04 | 3.1 |
| 33dBm | 3.2 | 1.28 | 3.9 |
| 34dBm | 3.3 | 1.58 | 4.7 |
| 35dBm | 3.3 | 1.94 | 5.5 |
| 36dBm | 3.4 | 2.36 | 6.3 |
| 37dBm | 3.6 | 2.87 | 7.2 |
| 38dBm | 3.8 | 3.46 | 7.9 |
| 39dBm | 4.2 | 4.16 | 8.7 |
| 40 dBm | 4.7 | 4.95 | 9.5 |

  - Option 2: MSD requirements with separated antenna architecture. (CMCC)

- Table X CA_n3-n39 with band n3 UL MSD requirements for different output powers

| Output power | MSD (dB) |
| --- | --- |
| 23dBm | 0.16 |
| 24dBm | 0.16 |
| 25dBm | 0.16 |
| 26dBm | 0.16 |
| 27dBm | 0.16 |
| 28dBm | 0.16 |
| 29dBm | 0.17 |
| 30dBm | 0.18 |
| 31dBm | 0.19 |
| 32dBm | 0.2 |
| 33dBm | 0.2 |
| 34dBm | 0.3 |
| 35dBm | 0.3 |
| 36dBm | 0.4 |
| 37dBm | 0.6 |
| 38dBm | 0.8 |
| 39dBm | 1.2 |
| 40 dBm | 1.7 |

- Recommended WF

  - Further check the MSD in the table.

Sub-topic 1-3 UE antenna type for inter-band CA

Last meeting agreements are listed as below:

Issue 3-4-1: whether preclude Case 4. - ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam is possible for specific band combination. - Antenna array type for inter-band CA is highly depending on frequency range of operating bands and UE implementation. - Option 1: Case 4 UE implementation is possible for CA_n3-n39 that frequency ranges between two bands are close to each other. - Option 2: Case 4 UE implementation is possible for CA_n3-n39 that DL frequency separation between two bands are less than [X=FFS, e.g, 500 MHz].

Issue 1-3-1: Case 4 related issues.

- Proposals

  - Option 1: Case 4 UE implementation is possible for CA_n3-n39 that frequency ranges between two bands are close to each other. (Qualcomm, CMCC, ZTE).

    - No need to further discuss in RAN4 RF, and suggest RRM session to define the MRTD requirements based on the worst-case assumption for UE antenna type. (Qualcomm)

    - This issue should be analyzed case by case. (CMCC)

  - Option 2: Case 4 UE implementation is possible for CA_n3-n39 that DL frequency separation between two bands are less than 1GHz. (LGE)

- Recommended WF

  - Option 1. Since it is hard to determine the frequency separation between two bands.

    - No more suggestion to RRM session. Since the architecture of common or separated antenna has no impact on MRTD requirements, and RRM session had already define the MRTD requirements.

Topic #2: R19 UE RF for UL-MIMO

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2503395 | CMCC | Proposal 1: Supporting ULFPTx mode for ATG UL-MIMO will be beneficial for the implementation of different ATG UE. Proposal 2: ATG UE maximum output power should be clarified in the section “UE Maximum Output Power for UL MIMO". Proposal 3: Apply the single antenna port-related requirements for ATG UEs, and the legacy requirements for TN UEs could be reused. Proposal 4: Tx Diversity should be supported for ATG. |
| R4-2503695 | Huawei, HiSilicon | Observation 1: the two PAs implementation in the proposed option 1 is not typical for ATG UE. Proposal 1: As a compromise, it’s necessary to mention whether the ULFPTx mode is supported or not in the way forward. However, it’s up to ATG UE implementation whether to leverage the existing ULFPTx mode or not since current Handheld ULFPTx mode is not always applicable to all of the ATG UE implementation. Proposal 2: RAN4 can discuss whether the single antenna port related requirements for ATG UE without indicating TxD have been included in the spec. And further discuss whether to explicitly specify the single antenna port related requirements for ATG UE with indicating TxD. |
| R4-2504039 | Ericsson | Observation 1 The ATG UEs are likely to use single-layer operation on LoS-dominated, lower multipath environments. Proposal 1 ULFPTx mode should be supported for ATG UL-MIMO Proposal 2 Includes single antenna port-related requirements for ATG UE. |
| R4-2504357 | ZTE Corporation, Sanechips | Proposal 1: ULFPTx Mode is not relevant to this feature. Proposal 2: not to consider the single antenna port transmission TxD for ATG UE operation. |

Open issues summary

Sub-topic 2-1 UL MIMO General requirements

Issue 2-1-1: ULFPTx mode for UL-MIMO

- Proposals

  - Option 1: ULFPTx Mode is not relevant to this feature. (ZTE)

  - Option 2: ULFPTx mode still applies for ATG UE to support different implementation (CMCC, Ericsson)

    - Support ULFPTx mode without specification impact except to clarify ATG UE maximum output power in the section “UE maximum output power for UL MIMO”. (CMCC)

  - Option 3: Depend on ATG UE implementation whether to leverage the existing ULFPTx mode. (Huawei)

- Recommended WF

  - TBD.

Issue 2-1-2: whether preclude the single antenna port related requirement

- Proposals

  - Option 1: no (CMCC, Ericsson, Huawei)

  - Option 2: not to consider the single antenna port transmission TxD for ATG UE operation. (ZTE)

- Recommended WF

  - Option 1.

Issue 2-1-3: Tx Diversity for UL-MIMO

- Proposals

  - Option 1: Tx Diversity should be supported for ATG.

  - Option 2: Others

- Recommended WF

  - TBD