3GPP TSG RAN WG5 Meeting #35	R5-071344

- Kobe, Japan, 7 - 11 May 2007

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.171 | CR | CRNum |  rev | - |  Current version: | 6.5.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Change to Pass/Fail criteria for Moving Scenario test case 5.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Spirent Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | R5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | TEI6_Test |  |  |  |  |  |  | Date:  |  | 8/05/2007 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-6 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | It is stated in the minimum requirements for the moving scenario and periodic update performance A-GPS test case that the UE shall meet the accuracy requirements with the periodical reporting interval of 2 seconds. However, in the current test procedure, the capability of reporting according to the periodic interval is not properly tested. At the moment, there is a penalty if the number of measurement reports sent is too few, but no real penalty if the number of measurement reports sent is too many. Hence, a UE can send measurement reports with a periodic reporting interval of 1 second (instead of the requested periodic reporting interval of 2 seconds) and still pass the performance test. In order to be consistent, there should be a similar penalty for sending too many measurement reports. After much discussion it has been proposed that, in order to test that the measurements really are periodic with the correct reporting interval, the following procedure shall be used: A measurement report that is received outside a window of +/- 20% of the correct 2 second interval (plus measurement uncertainty) shall be considered as a bad result. Hence, a measurement report needs to be received in the interval between 1.5 and 2.5 seconds after the previous measurement report to be evaluated regarding position accuracy. (That is 1.6 and 2.4 seconds plus 100ms for measurement uncertainty). This procedure is no more stringent than test cases in the RAN5 signaling specifications where timers, which do not have a tolerance specified in the core specifications, are tested to be within +/- 10%. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Test procedure is changed to test periodical reporting interval of 2 seconds +/- 20% plus measurement system uncertainty of 100ms. Differential response time uncertainty of 100ms added to test system uncertainties |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Moving propagation test will be unnecissarily strict on the periodic reporting timer requirement for which there is currently no core specification. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 5.6.4, 5.6.5. F.1, F.2, F.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  | This CR applies from Rel-6 and later releases |  |  |  |  |  |  |  |  |

## 5.6	Moving Scenario and Periodic Update Performance

### 5.6.1	Definition and applicability

Moving scenario and periodic update performance measures the accuracy of the UE's A-GPS position estimates and the periodic update capability of the UE in a moving scenario.

The requirements and this test apply to all types of UTRA for the FDD UE that supports A-GPS.

### 5.6.2	Minimum requirements

The position estimates, after the first reported position estimate, shall meet the accuracy requirement in table 5.6.2 with the periodical reporting interval of 2 seconds for the parameters specified in table 5.6.1.

NOTE:	In the actual testing the UE may report error messages until it has been able to acquire GPS measured results or a position estimate. The SS shall only consider the first measurement report different from an error message as the first position estimate in the requirement in table 5.6.2.

Table 5.6.1: Test parameters for Moving Scenario and Periodic Update Performance

| Parameters | Unit | Value |
| --- | --- | --- |
| Number of generated satellites | - | 5 |
| HDOP Range | - | 1.8 to 2.5 |
| Propagation condition | - | AWGN |
| GPS signal for all satellites | dBm | -130 |

Table 5.6.2: Minimum requirements for Moving Scenario and Periodic Update Performance

| Success Rate | 2-D position error |
| --- | --- |
| 95 % | 100 m |

The reference for this requirement is 3GPP TS 25.171 [6], clause 5.5.1.

### 5.6.3	Test purpose

To verify the UE's position estimates, after the first reported position estimate, meet the minimum requirements under GPS satellite signal conditions that simulate a moving scenario. A good tracking performance, with regular position estimate reporting is essential for certain location services.

### 5.6.4	Method of test

#### 5.6.4.1	Initial conditions

Test environment:  normal; see clause E.2.

The UE is requested to use periodical reporting with a reporting interval of 2 seconds.

The GPS signals simulate the UE moving on a rectangular trajectory of 940 m by 1 440 m with rounded corners defined in figure 5.6.1 and table 5.6.3. The initial reference is first defined followed by acceleration to final speed of 100 km/h in 250 m. The UE then maintains the speed for 400 m. This is followed by deceleration to final speed of 25 km/h in 250 m. The UE then turn 90 degrees with turning radius of 20 m at 25 km/h. This is followed by acceleration to final speed of 100 km/h in 250 m. The sequence is repeated to complete the rectangle.

Table 5.6.3: Trajectory Parameters for Moving Scenario and Periodic Update Performance test case

| Parameter | Distance (m) | Speed (km/h) |
| --- | --- | --- |
| l11, l15, l21, l25 | 20 | 25 |
| l12, l14, l22, l24 | 250 | 25 to 100 and 100 to 25 |
| l13 | 400 | 100 |
| l23 | 900 | 100 |

Figure 5.6.1: Rectangular Trajectory for Moving Scenario and Periodic Update Performance test case

1.	Connect SS and GSS to the UE antenna connector or antenna connectors as shown in figures A.1 or A.2.

2.	Set the GPS test parameters as specified in table 5.6.4 for GPS scenario #3.

3.	Switch on the UE.

4.	Set up a connection using the procedure in clause D.2.

#### 5.6.4.2	Procedure

1.	Start GPS scenario #3 as specified in 3GPP TS 34.108 [3], clause 10.1.2

2.	Send a RESET UE POSITIONING STORED INFORMATION message followed by RRC MEASUREMENT CONTROL messages containing appropriate assistance data; as specified in 3GPP TS 34.108 [3], clauses 10.2 and 10.6 for UE based testing; or clauses 10.4 and 10.6 for UE assisted testing; as required to obtain fixes using the procedure specified in 3GPP TS 34.108 [3], clauses 7.5.2 or 7.5.5.

3.	Ignore any error messages that the UE may report in MEASUREMENT REPORT messages until it has been able to acquire the GPS signals and reports the first GPS measured result or position estimate.

4.	Discard the first GPS measured result or position estimate.

5.	Record the time of reception of the next MEASUREMENT REPORT message after reception of the first GPS measured result or position estimate.

6.	After the reception of the first GPS measured result or position estimate reported in a MEASUREMENT REPORT message, every time the UE returns a GPS measured result or position estimate in the MEASUREMENT REPORT message record the time of reception and the result. If the difference between the time of reception and the time of reception of the previous result is less than 1.5 seconds or greater than 2.5 seconds, or if the UE reports a UE positioning error in any MEASUREMENT REPORT messages, then record one Bad Result. Otherwise process the result as specified in step 7.

7.	For UE based testing compare the reported position estimate in the MEASUREMENT REPORT message against the simulated position of the UE at the time of applicability reported in the position estimate and calculate the 2D position error as specified in clause 5.1.1.3. Compare the 2D position error against the value in table 5.6.5 and record one Good Result or Bad Result as appropriate; or

For UE assisted testing convert the GPS measured results reported in the MEASUREMENT REPORT message to a 2D position using the method described in Annex B and then compare the result against the simulated position of the UE at the time of applicability reported in the GPS measured results and calculate the 2D position error as specified in clause 5.1.1.3. Compare the 2D position error against the value in table 5.6.5 and record one Good Result or Bad Result as appropriate.

8.	If the UE sends the first MEASUREMENT REPORT that contains a measured result or position estimate later than 240s after the start of the GPS scenario, fail the UE and stop the test early. Otherwise collect MEASUREMENT REPORTs during 900s, starting from the time recorded in step 5. If at any time the difference between the times of reception of two consecutive results is greater than 240s, fail the UE and stop the test early.  Use the collected Good Results and Bad Results to determine the PASS/FAIL according to clause 5.6.5.

9.	Release the connection using the procedure in clause D.3.

### 5.6.5	Test Requirements

For the parameters specified in table 5.6.4, after the first reported position estimate, the UE shall meet the accuracy requirement and the success rate specified in table 5.6.5 with a periodical reporting interval of 2 seconds +/- 20%  plus measurement system uncertainty of 100ms.

Note:	Due to the statistical nature of the results it is not possible to design a test with predefined confidence level for the success rate in Table 5.6.5, therefore a simple PASS/FAIL of the results gathered against this success rate is used.

Table 5.6.4: Test parameters for Moving Scenario and Periodic Update Performance

| Parameters | Unit | Value |
| --- | --- | --- |
| Number of generated satellites | - | 5 |
| HDOP Range | - | 1.8 to 2.5 |
| Propagation condition | - | AWGN |
| GPS signal for all satellites | dBm | -130 |

Table 5.6.5: Test requirements for Moving Scenario and Periodic Update Performance

| Success Rate | 2-D position error |
| --- | --- |
| 95 % | 101.3 m |

NOTE 1:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause F.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in clause F.4.

NOTE 2:	In the actual testing the UE may report error messages until it has been able to acquire GPS measured results or a position estimate. The test equipment shall only consider the first measurement report different from an error message as the first position estimate in the requirement in table 5.6.5.

NEXT CHANGED SECTION

Annex F (normative): 
General test conditions and declarations

The requirements of this clause apply to all applicable tests in the present document.

In all the relevant clauses in this clause all 2 D position error measurements shall be carried out according to the general rules for statistical testing in clause F.6.

# F.1	Acceptable uncertainty of Test System

The maximum acceptable uncertainty of the Test System is specified below for each test, where appropriate. The Test System shall enable the stimulus signals in the test case to be adjusted to within the specified range, and the equipment under test to be measured with an uncertainty not exceeding the specified values. All ranges and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated.

A confidence level of 95 % is the measurement uncertainty tolerance interval for a specific measurement that contains 95 % of the performance of a population of test equipment.

It should be noted that the uncertainties in clause F.1 apply to the Test System operating into a nominal 50 ohm load and do not include system effects due to mismatch between the DUT and the Test System.

## F.1.1	Measurement of test environments

The measurement accuracy of the UE environmental test conditions, defined in annex E, shall be:

| Pressure | 5 kPa |
| --- | --- |
| Temperature | 2 degrees |
| Relative Humidity | 5 % |
| DC Voltage | 1.0 % |
| AC Voltage | 1.5 % |
| Vibration | 10 % |
| Vibration frequency | 0.1 Hz |

The above values shall apply unless the test environment is otherwise controlled and the specification for the control of the test environment specifies the uncertainty for the parameter.

## F.1.2	A-GPS Performance requirements

Table F.1.2: Maximum Test System Uncertainty for A-GPS Performance tests

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 5.2.1 Sensitivity Coarse Time Assistance | Coarse Time Assistance 200 ms |  |
|  | Absolute GPS signal level 1 dB |  |
|  | Position error 0.05 m | Position error consists of 0.05 m system error. The effect of position reporting resolution of approximately 1.2 m (see note) is not included in the allowable test system uncertainty but is included in the test tolerances since this resolution limitation limits the reporting capability of the UE. . For simplicity the combined test tolerance is given as 1.3 m |
|  | Response time  300 ms |  |
| 5.2.2 Sensitivity Fine Time Assistance | Coarse Time Assistance 200 ms |  |
|  | Fine Time Assistance 1 us |  |
|  | Absolute GPS signal level 1 dB |  |
|  | Position error 0.05 m | Position error as above |
|  | Response time  300 ms |  |
| 5.3 Nominal Accuracy | Coarse Time Assistance 200 ms |  |
|  | Absolute GPS signal level 1 dB |  |
|  | Position error 0.05 m | Position error as above |
|  | Response time  300 ms |  |
| 5.4 Dynamic Range | Coarse Time Assistance 200 ms |  |
|  | Absolute GPS signal level 1 dB |  |
|  | Relative GPS signal level 0.2 dB |  |
|  | Position error 0.05 m | Position error as above |
|  | Response time  300 ms |  |
| 5.5 Multi-path Performance | Coarse Time Assistance 200 ms |  |
|  | Absolute GPS signal level 1 dB |  |
|  | Relative GPS signal level 0.2 dB |  |
|  | Position error 0.05 m | Position error as above |
|  | Response time  300 ms |  |
| 5.6 Moving Scenario and Periodic Update Performance | Absolute GPS signal level 1 dB |  |
|  | Position error 0.05 m | Position error as above |
|  | Differential response time  100 ms |  |

Note:	For UE based mode the effect of position reporting resolution is given by:

90×2×π×R2E23×3602+360×2×π×R×cosφ2E24×3602where R is the radius of the earth and φ is the latitude of the location. For the two GPS scenarios defined in TS 34.108 [3] this equates to approximately 2.32 m and 2.24 m. For simplicity this is given as 1.2 m.

For UE assisted mode it is assumed that the output from the WLS position solution calculation in clause B.3 is coded using the same position coding method as for UE based mode before being used to calculate position error.  Therefore the effect of reporting resolution will be the same as for UE based mode.

# F.2	Test Tolerances (This clause is informative)

The Test Tolerances defined in this clause have been used to relax the Minimum Requirements in the present document to derive the Test Requirements.

The Test Tolerances are derived from Test System uncertainties, regulatory requirements and criticality to system performance. As a result, the Test Tolerances may sometimes be set to zero.

The test tolerances should not be modified for any reason e.g. to take account of commonly known test system errors (such as mismatch, cable loss, etc.).

## F.2.1	A-GPS Performance requirements

Table F.2.1: Test Tolerances for A-GPS Performance tests

| Clause | Test Tolerance |  |
| --- | --- | --- |
| 5.2.1 Sensitivity Coarse Time Assistance | Coarse Time Assistance | 200 ms |
|  | Absolute GPS signal level | 1 dB |
|  | Position error | 1.3 m |
|  | Response time | 300 ms |
| 5.2.2 Sensitivity Fine Time Assistance | Coarse Time Assistance | 200 ms |
|  | Fine Time Assistance | 1 us |
|  | Absolute GPS signal level | 1 dB |
|  | Position error | 1.3 m |
|  | Response time | 300 ms |
| 5.3 Nominal Accuracy | Coarse Time Assistance | 200 ms |
|  | Absolute GPS signal level | 0 dB |
|  | Position error | 1.3 m |
|  | Response time | 300 ms |
| 5.4 Dynamic Range | Coarse Time Assistance | 200 ms |
|  | Absolute GPS signal level | 0 dB |
|  | Relative GPS signal level | 0.2 dB |
|  | Position error | 1.3 m |
|  | Response time | 300 ms |
| 5.5 Multi-path Performance | Coarse Time Assistance | 200 ms |
|  | Absolute GPS signal level | 0 dB |
|  | Relative GPS signal level | 0.2 dB |
|  | Position error | 1.3 m |
|  | Response time | 300 ms |
| 5.6 Moving Scenario and Periodic Update Performance | Absolute GPS signal level | 0 dB |
|  | Position error | 1.3 m |
|  | Differential Response Time | 100 ms |

# F.3	Interpretation of measurement results

The measurement results returned by the Test System are compared - without any modification - against the Test Requirements as defined by the shared risk principle.

The Shared Risk principle is defined in TR 102 273-1-2 [14], clause 6.5.

The actual measurement uncertainty of the Test System for the measurement of each parameter shall be included in the test report.

The recorded value for the Test System uncertainty shall be, for each measurement, equal to or lower than the appropriate figure in clause F.1.

If the Test System for a test is known to have a measurement uncertainty greater than that specified in clause F.1, it is still permitted to use this apparatus provided that an adjustment is made value as follows.

Any additional uncertainty in the Test System over and above that specified in clause F.1 shall be used to tighten the Test Requirement - making the test harder to pass. (This may require modification of stimulus signals). This procedure will ensure that a Test System not compliant with clause F.1does not increase the chance of passing a device under test where that device would otherwise have failed the test if a Test System compliant with clause F.1 had been used.

# F.4	Derivation of Test Requirements (This clause is informative)

The Test Requirements in the present document have been calculated by relaxing the Minimum Requirements of the core specification using the Test Tolerances defined in clause F.2. When the Test Tolerance is zero, the Test Requirement will be the same as the Minimum Requirement. When the Test Tolerance is non-zero, the Test Requirements will differ from the Minimum Requirements, and the formula used for this relaxation is given in table F.4.1.

Table F.4.1: Derivation of Test Requirements (A-GPS tests)

| Test | Minimum Requirement in 3GPP TS 25.171 [6] | Test Tolerance (TT) | Test Requirement in the present document |
| --- | --- | --- | --- |
| 5.2.1 Sensitivity Coarse Time Assistance | Coarse Time Assistance 2 s | 200 ms | Formulas: UL-TT, LL+TT: 1.8 s |
|  | Absolute GPS signal level -142, -147 dBm | 1 dB | Level + TT: -141, -146 dBm |
|  | Position error 100 m | 1.3 m | Error +TT: 101.3 m |
|  | Response time 20 s | 300 ms | Time + TT: 20.3 s |
| 5.2.2 Sensitivity Fine Time Assistance | Coarse Time Assistance 2 s | 200 ms | Formulas: UL-TT, LL+TT: 1.8 s |
|  | Fine Time Assistance 10 us | 1 us | UL-TT, LL+TT: 9 us |
|  | Absolute GPS signal level -147 dBm | 1 dB | Level + TT: -146 dBm |
|  | Position error 100 m | 1.3 m | Error +TT: 101.3 m |
|  | Response time 20 s | 300 ms | Time + TT: 20.3 s |
| 5.3 Nominal Accuracy | Coarse Time Assistance 2 s | 200 ms | Formulas: UL-TT, LL+TT: 1.8 s |
|  | Absolute GPS signal level -130 dBm | 0 dB | Level + TT: -130 dBm |
|  | Position error 30 m | 1.3 m | Error +TT: 31.3 m |
|  | Response time 20 s | 300 ms | Time + TT: 20.3 s |
| 5.4 Dynamic Range | Coarse Time Assistance 2 s | 200 ms | Formulas: UL-TT, LL+TT: 1.8 s |
|  | Absolute GPS signal level -129 to -147 dBm | 1 dB | Level + TT: each level +1 dBm |
|  | Relative GPS signal level 18 dB | 0.2 dB | Level - TT: highest level -0.2 dB: -128.2dBm |
|  | Position error 100 m | 1.3 m | Error +TT: 101.3 m |
|  | Response time 20 s | 300 ms | Time + TT: 20.3 s |
| 5.5 Multi-path Performance | Coarse Time Assistance 2 s | 200 ms | Formulas: UL-TT, LL+TT: 1.8 s |
|  | Absolute GPS signal level -130 dBm | 0 dB | Level + TT: -130 dBm |
|  | Relative GPS signal level 6 dB | 0.2 dB | Level + TT: lower level - 0.2dB: -136.2 dBm |
|  | Position error 100 m | 1.3 m | Error +TT: 101.3 m |
|  | Response time 20 s | 300 ms | Time + TT: 20.3 s |
| 5.6 Moving Scenario and Periodic Update Performance | Absolute GPS signal level -130 dBm | 0 dB | Formulas: Level + TT: -130 dBm |
|  | Position error 100 m | 1.3 m | Error +TT: 101.3 m |
|  | Differential response time 2s  20 % | 100 ms | Time +TT: 1.5 s and 2.5 s |