3GPP TSG-RAN4 Meeting #43 	Tdoc R4-070806

Kobe, Japan, 7 - 11 May 2007

| CR-Form-v8 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.104 | CR | 0292 |  rev | - |  Current version: | 7.6.0 |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | BS perforamnce requirements for high speed train |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source:  | NTT DoCoMo, Panasonic, NEC, Ericsson, Fujitsu |  |  |  |  |  |  |  |  |  |
| Work item code:  | TEI7 |  |  |  |  |  |  | Date:  |  | 07/05/2007 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Ph2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | BS performance requiement for high speed train is added to release 7. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Requirement for demodulation of DCH and RACH, and propagation condition for high speed train are added. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | There will be no requirements for high speed train. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 8.1, 8.5A, 8.7, B.4A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | 25.141 |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at http://www.3gpp.org/specs/CR.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://ftp.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2001-03 contains the specifications resulting from the March 2001 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

## 8.1	General

Performance requirements for the BS are specified for the measurement channels defined in Annex A and the propagation conditions in Annex B. The requirements only apply to those measurement channels that are supported by the base station. The performance requirements for the high speed train conditions which scenarios defined in Annex B.4A are optional.

For BS with dual receiver antenna diversity, only the BS performance requirements with Rx diversity apply, the required Eb/N0 shall be applied separately at each antenna port.

For BS without receiver antenna diversity, only the BS performance requirements without Rx diversity apply, the required Eb/N0 shall be applied at the BS Rx antenna port.

The Eb/No used in this section is defined as:

EbNo=EcNo⋅LchipLinf

Where:

Ec is the received total energy of DPDCH, DPCCH, HS-DPCCH, E-DPDCH and E-DPCCH per PN chip per antenna from all paths.

No is the total one-sided noise power spectral density due to all noise sources

Lchip is the number of chips per frame

Linf is the number of information bits in DTCH excluding CRC bits per frame

Table 8.1: Summary of Base Station performance targets

| Physical channel | Measurement channel | Static | Multi-path Case 1 | Multi-path Case 2 | Multi-path Case 3 | Moving | Birth / Death | High speed train |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Performance metric |  |  |  |  |  |  |
| DCH | 12.2 kbps | BLER<10-2 | BLER<10-2 | BLER<10-2 | BLER<10-2 | BLER< | BLER< | BLER<10-2 |
|  | 64 kbps | BLER< 10-1,10-2 | BLER< 10-1, 10-2 | BLER< 10-1,10-2 | BLER< 10-1, 10-2,10-3 | BLER< | BLER< | - |
|  | 144 kbps | BLER< 10-1,10-2 | BLER< 10-1,10-2 | BLER< 10-1,10-2 | BLER< 10-1, 10-2,10-3 | - | - | - |
|  | 384 kbps | BLER< 10-1,10-2 | BLER< 10-1,10-2 | BLER< 10-1,10-2 | BLER< 10-1, 10-2,10-3 | - | - | - |

### {Unchanged sections}

## 8.5A	Demodulation of DCH in high speed train conditions

### 8.5A.1	General

The performance requirement of DCH in high speed train conditions is determined by the maximum BLER allowed when the receiver input signal is at a specified Eb/N0 limit. The BLER is calculated for the measurement channel supported by the base station.

### 8.5A.2	Minimum requirement

The BLER shall not exceed the limit for the Eb/N0 specified in Table 8.7A.

Table 8.7A: Performance requirements in high speed train conditions

| Scenario | Measurement channel | Received Eb/N0 For BS with Rx Diversity | Received Eb/N0 For BS without Rx Diversity | Required BLER |
| --- | --- | --- | --- | --- |
| 1 | 12.2 kbps | 6.5 dB | 8.8 dB | < 10-2 |
| 2 | 12.2 kbps | n.a. | 8.8 dB | < 10-2 |
| 3 | 12.2 kbps | n.a. | 10.1 dB | < 10-2 |

### {Unchanged sections}

## 8.7	Performance requirement for RACH

Performance requirement for RACH consists of two parts: preamble detection and message demodulation. Requirements for these are in sections 8.7.1 and 8.7.2, respectively. Requirements are defined for three propagation conditions: static, fading case 3, and high speed train conditions. The propagation conditions are defined in annexes B.1, B.2, and B.4A.

### 8.7.1	Performance requirement for RACH preamble detection

Probability of false alarm, Pfa (=false detection of the preamble) when the preamble was not sent, shall be 10-3 or less. The performance measure Required Ec/N0 at probability of detection, Pd of 0.99 and 0.999. Only 1 signature is used and it is known by the receiver. The requirement for preamble detection, when the preamble was sent is in table 8.9, 8.10, and 8.10A for static, case 3 fading, and high speed train conditions.

Table 8.9: Requirements for Ec/N0 of Pd in static propagation condition

|  | Ec/N0 for required Pd  0.99 | Ec/N0 for required Pd  0.999 |
| --- | --- | --- |
| BS with Rx Diversity | -20.5 dB | -20.1 dB |
| BS without Rx Diversity | -17.6 dB | -16.8 dB |

Table 8.10: Requirements of Ec/N0 of Pd in case 3 fading

|  | Ec/N0 for required Pd  0.99 | Ec/N0 for required Pd  0.999 |
| --- | --- | --- |
| BS with Rx Diversity | -15.5 dB | -13.4 dB |
| BS without Rx Diversity | -9.4 dB | -6.4 dB |

Table 8.10A: Requirements of Ec/N0 of Pd in high speed train conditions

| Scenario |  | Ec/N0 for required Pd  0.99 | Ec/N0 for required Pd  0.999 |
| --- | --- | --- | --- |
| 1 | BS with Rx Diversity | -18.1 dB | -17.9 dB |
|  | BS without Rx Diversity | -15.1 dB | -14.8 dB |
| 2 | BS with Rx Diversity | n.a. | n.a. |
|  | BS without Rx Diversity | -15.6 dB | -14.8 dB |
| 3 | BS with Rx Diversity | n.a. | n.a. |
|  | BS without Rx Diversity | -15.3 dB | -15.1 dB |

### 8.7.2	Demodulation of RACH message

The performance measure is required Eb/N0 for block error rate (BLER) of 10-1 and 10-2. Both measurement channels have TTI=20 ms. Payloads are 168 and 360 bits. Channel coding is rate ½ convolutional coding.

#### 8.7.2.1	Minimum requirements for Static Propagation Condition

Table 8.11: Required Eb/N0 for static propagation

| Transport Block size TB and TTI in frames | 168 bits, TTI = 20 ms |  | 360 bits, TTI = 20 ms |  |
| --- | --- | --- | --- | --- |
|  | Eb/N0 for required BLER < 10-1 | Eb/N0 for required BLER < 10-2 | Eb/N0 for required BLER < 10-1 | Eb/N0 for required BLER < 10-2 |
| BS with Rx Diversity | 4.1 dB | 5.0 dB | 3.9 dB | 4.8 dB |
| BS without Rx Diversity | 7.2 dB | 8.1 dB | 6.9 dB | 7.8 dB |

#### 8.7.2.2	Minimum requirements for Multipath Fading Case 3

Table 8.12: Required Eb/N0 for case 3 fading

| Transport Block size TB and TTI in frames | 168 bits, TTI = 20 ms |  | 360 bits, TTI = 20 ms |  |
| --- | --- | --- | --- | --- |
|  | Eb/N0 for required BLER < 10-1 | Eb/N0 for required BLER < 10-2 | Eb/N0 for required BLER < 10-1 | Eb/N0 for required BLER < 10-2 |
| BS with Rx Diversity | 7.4 dB | 8.5 dB | 7.3 dB | 8.3 dB |
| BS without Rx Diversity | 11.1 dB | 12.4 dB | 11.0 dB | 12.1 dB |

#### 8.7.2.3	Minimum requirements for high speed train conditions

Table 8.12A: Required Eb/N0 for high speed train conditions

| Transport Block size TB and TTI in frames |  | 168 bits, TTI = 20 ms |  | 360 bits, TTI = 20 ms |  |
| --- | --- | --- | --- | --- | --- |
| Scenario |  | Eb/N0 for required BLER < 10-1 | Eb/N0 for required BLER < 10-2 | Eb/N0 for required BLER < 10-1 | Eb/N0 for required BLER < 10-2 |
| 1 | BS with Rx Diversity | 5.1 dB | 6.4 dB | 5.3 dB | 6.2 dB |
|  | BS without Rx Diversity | 7.9 dB | 8.8 dB | 7.5 dB | 8.4 dB |
| 2 | BS with Rx Diversity | n.a. | n.a. | n.a. | n.a. |
|  | BS without Rx Diversity | 7.7 dB | 8.6 dB | 7.4 dB | 8.3 dB |
| 3 | BS with Rx Diversity | n.a. | n.a. | n.a. | n.a. |
|  | BS without Rx Diversity | 8.2 dB | 9.6 dB | 8.4 dB | 9.3 dB |

### {Unchanged sections}

## B.4A	High speed train conditions

The high speed train conditions for the test of the baseband performance are 2 non fading propagation channels (scenario 1 and 3) and 1 fading propagation channel  (scenario 2) with one tap. For BS with Rx diversity defined in scenario 1, the Doppler shift variation is the same between anttenas.

For scenario 1 and 3, Doppler shift is given by

fst=fdcosθt	(B.2)

where fst is the Doppler shift and fd is the maximum Doppler frequency. The cosine of angle θtis given by

cosθt=Ds/2−vtDmin2+Ds/2−vt2, 0≤t≤Ds/v	(B.3)

where Ds/2 is the initial distance of the train from BS, and Dmin is BS-Railway track distance, both in meters; v is the velocity of the train in m/s, t is time in seconds.

For scenario 2, Rician fading is considered where Rician factor, K is defined as the ratio between the dominant signal power and the variant of the other weaker signals.

The parameters in the equation are shown in table B.2A. Accordingly, Doppler shift for scenario 1 and 3 change to the following figure B.3 and B.4, respectively.

Table B.2A: Parameters for high speed train conditions

| Parameter | Value |  |  |
| --- | --- | --- | --- |
|  | Scenario 1 | Scenario 2 | Scenario 3 |
| Ds | 1000 m | Infinity | 300 m |
| Dmin | 50 m | - | 2 m |
| K | - | 10 dB | - |
| v | 350 km/h | 300 km/h | 300 km/h |
| fd | 1340 Hz | 1150 Hz | 1150 Hz |

- Figure B.3: Doppler shift trajectory for scenario 1

- Figure B.4: Doppler shift trajectory for scenario 3