3GPP TSG-RAN4 Meeting #66-BIS	 R4-130991

# Chicago, USA, 15-19 April, 2013

Source: 	SONY

Title: 	TRP/TRS Performance Measurement Data from 5 Laptop Types

Agenda Item: 	5.7

Document for:	Discussion

# 1		Introduction R4-125719

Several proposal for TRP/TRS performance requirements for LEE devices has been presented but no consensus has been reached (e.g. [1], [4])

In this contribution TRP/TRS performance measurement data from 5 different commercially available laptop models are presented.

# 2	Measurement Results

TRP data for WCDMA Band 1 and Band 8 are listed in Table 2.1. TRS data for WCDMA Band 1 and Band 8 are listed in Table 2.2.

Table 2.1:  TRP data

| Band | Freq[MHz] | TCH | FS [dBm] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Model A | Model B | Model C | Model D | Model E |
| WCDMA FDD B1 | 1922.40 | 9612 | 21.7 | 22.4 | 22.6 | 21.8 | 21.3 |
| WCDMA FDD B1 | 1950.00 | 9750 | 21.7 | 22.4 | 22.6 | 22.9 | 20.9 |
| WCDMA FDD B1 | 1977.60 | 9888 | 22.1 | 21.8 | 21.8 | 22.6 | 20.0 |
| WCDMA FDD B8 | 882.40 | 2712 | 20.8 | 20.5 | 20.1 | 20.5 | 20.7 |
| WCDMA FDD B8 | 897.60 | 2788 | 20.5 | 20.6 | 19.9 | 20.7 | 20.2 |
| WCDMA FDD B8 | 912.60 | 2863 | 20.3 | 20.9 | 19.8 | 21.5 | 20.0 |

Table 2.2: TRS data

| Band | Freq[MHz] | TCH | FS [dBm] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Model A | Model B | Model C | Model D | Model E |
| WCDMA FDD B1 | 2112.40 | 10562 | -106.6 | -106.7 | -108.3 | -107.0 | -106.9 |
| WCDMA FDD B1 | 2140.00 | 10700 | -107.4 | -106.9 | -108.2 | -106.5 | -108.1 |
| WCDMA FDD B1 | 2167.60 | 10838 | -108.4 | -107.4 | -107.7 | -105.8 | -106.9 |
| WCDMA FDD B8 | 927.40 | 2937 | -105.1 | -105.0 | -103.3 | -105.6 | -107.4 |
| WCDMA FDD B8 | 942.60 | 3013 | -101.1 | -103.6 | -101.8 | -107.0 | -108.3 |
| WCDMA FDD B8 | 957.60 | 3088 | -105.6 | -101.9 | -100.5 | -106.3 | -106.8 |

Averaged data for TRP are presented in Table 2.3 together with summarized minimum values.  Table 2.4 lists averaged TRS data together with minimum values.

Table 2.3: Averaged TRP data

|  | B1 | B1 | B8 | B8 |
| --- | --- | --- | --- | --- |
|  | Average [dBm] | Min [dBm] | Average [dBm] | Min [dBm] |
| Model A | 21.9 | 21.7 | 20.5 | 20.3 |
| Model B | 22.2 | 21.8 | 20.9 | 20.5 |
| Model C | 22.3 | 21.8 | 20 | 19.8 |
| Model D | 22.5 | 21.8 | 20.9 | 20.5 |
| Model E | 20.7 | 20 | 20.3 | 20 |
| Average | 22 | 21.4 | 20.5 | 20.2 |

Table 2.4: Averaged TRS data

|  | B1 | B1 | B8 | B8 |
| --- | --- | --- | --- | --- |
|  | Average [dBm] | Min [dBm] | Average [dBm] | Min [dBm] |
| Model A | -107.5 | -106.6 | -104.3 | -101.1 |
| Model B | -107 | -106.7 | -103.6 | -101.8 |
| Model C | -108 | -107.7 | -102 | -100.5 |
| Model D | -106.5 | -105.8 | -106.3 | -105.6 |
| Model E | -107.3 | -106.9 | -107.5 | -107.4 |
| Average | -107.3 | -106.8 | -105.3 | -104 |

# 3	Conclusions

We have presented TRP/TRS performance measurement data from 5 different commercially available laptop models. Typically TRP average for operating Band 1 is between 21-22 dBm and for operating Band 8 between 20-21 dBm. TRS average for operating Band 1 is between -107 dBm and -108 dBm and for operating Band 8 is between -102 dBm and -108

This result is in line with earlier measurements [2], [3].

# 4	Reference

[1] R4-115568, “TRP/TRS requirement values for LEE (Laptop Embedded Equipment) devices”, Ericsson, RAN4#61, November 2011

[2] R4-123268, “TRP and TRS measurements for LEE devices”, Orange, RAN4#63, May 2012

[3] R4-123066, “LEE TRP TRS measurement results”, Vodafone Group, RAN4#63, May 2012

[4] R4-126206, “Proposal TRS performance requirements for LEE (Laptop Embedded Equipment) devices”, Vodafone, Orange, Telecom Italia, RAN4#65, Nov 2012