TSG-RAN Working Group 4 (Radio) meeting #71	R4-143485

Seoul, Korea, 19 - 22 May, 2014

Source:	NSN

Title:	TP for TR 36.851: The clarification of LTE TDD-FDD joint operation in BS RF requirement

Agenda item:	7.7.2 [LTE_CA_TDD_FDD-Core]

Document for:	Approval

# Introduction

BS specification impacts and which multi-band configurations to include for TDD-FDD joint operation [1] have been discussed in [2-10]. Although some multi-band configurations are identified feasible by some companies [8-9], the following wayforward [11] is agreed in order to complete the WI in Release 12 timeframe.

- The discussion on feasibility of multi-band BS configuration supporting both FDD and TDD bands shall be decoupled from the completion of this WI.

  - Multi-band BS configuration supporting both FDD and TDD bands is not supported in Release 12 BS specification

In this contribution, it is proposed to capture the agreement and summarize the general impacts of TDD-FDD joint operations to RAN4 BS specifications in TR 36.851.

# Discussion

Because of the agreed wayfoward, the assumption in MB-MSR work [12] is still valid for this WI, “The MB-MSR application scenarios also cover both FDD and TDD, but are limited to MB-MSR for FDD only scenarios and TDD only scenarios respectively.”  Therefore, there is no need to include new transmitter or receiver requirement for multi-band BS configurations supporting both FDD and TDD bands in Release 12.

For TDD-FDD joint operation, multi-band BS configuration supporting both TDD and FDD is excluded in Release 12. In principle, separate active RF components for TDD and FDD are assumed to support TDD-FDD CA. The RF requirements are applied separately for TDD and FDD. Therefore, the requirements in clause 6 and clause 7 of BS core specs, TS36.104 and 37.104, do not need to be modified to introduce TDD-FDD joint operation. This shall be captured in the BS core specs in the following.

- The RF requirements in the present specification are not applicable for a BS capable of multi-band operation supporting bands for both FDD and TDD.

To include TDD-FDD CA in BS specifications, new CA band combinations shall be defined in the same way as other inter-band CA combinations. In other word, TDD-FDD CA shall be included in the same table as FDD-FDD or TDD-TDD CA as a new band combination, CA_x-y, where x and y are FDD or TDD bands. (As proposed in [13].)

# Conclusion

It is proposed that the attached text proposal below is approved and included into TR 36.851.

# Reference

- RP-140465	Revised WID: LTE TDD-FDD joint operation including Carrier Aggregation, Nokia Corporation, NSN,  CMCC, Qualcomm, CATT, CATR, NEC, Mediatek, Huawei, Hisilicon, Orange, TeliaSonera, KDDI, Samsung, CeWIT, Deutsche Telekom, T-Mobile USA, Softbank Mobile, KT, Verizon Wireless, Sharp, DAC-UPC, IAESI, Panasonic, Intel, BlackBerry UK limited, ZTE, III, Texas Instruments, Ericsson

- R4-136017	Discussion of BS issues for TDD/FDD joint operation including Carrier Aggregation, Huawei

- R4-140272	Consideration on BS Specification for TDD-FDD joint operation including Carrier Aggregation, NTT DOCOMO

- R4-140824	BS RF core requirements update for CA TDD-FDD, Ericsson

- R4-141336	Further Consideration on BS Specification for TDD-FDD joint operation including Carrier Aggregation, NTT DOCOMO

- R4-141414	Discussion on transmitter off power for FDD/TDD CA, CATT

- R4-142238	Discussion on implementation for FDD and TDD joint operation including carrier aggregation, ZTE, Tejet

- R4-141652 	On LTE TDD-FDD CA multi-band configuration, NSN

- R4-142138	BS multi-band configurations, Ericsson

- R4-142267	Multiband BS and TDD-FDD CA concerns, Huawei

- R4-142486	Way forward on Multi-band BS for TDD-FDD joint operation including Carrier Aggregation, NTT DOCOMO, NSN, Ericsson

- TR37.812 ver 11.3.0	Radio Frequency (RF) requirements for Multi-band and Multi-standard radio (MB-MSR) Base Station (BS)

- R4-141337	CR for TDD-FDD CA for CA_1A-42A(TS36.104/141), NTT DOCOMO

TEXT PROPOSAL:

<start of text proposal >

7            TDD-FDD Inter-band Carrier aggregation

## 7.1         General part

### 7.1.1      BS specific

For TDD-FDD joint operation, multi-band BS configuration supporting both TDD and FDD is excluded in Release 12. In principle, separate active RF components for TDD and FDD are assumed to support TDD-FDD CA. The RF requirements are applied separately for TDD and FDD. Therefore, the requirements in clause 6 and clause 7 of BS core specs, TS36.104 and 37.104, do not need to be modified to introduce TDD-FDD joint operation. This shall be captured in the BS core specs in the following.

- The RF requirements in the present specification are not applicable for a BS capable of multi-band operation supporting bands for both FDD and TDD.

To include TDD-FDD CA in BS specifications, new CA band combinations shall be defined in the same way as other inter-band CA combinations. In other word, TDD-FDD CA shall be included in the same table as FDD-FDD or TDD-TDD CA as a new band combination, CA_x-y, where x and y are FDD or TDD bands.

<end of text proposal >