3GPP TSG-RAN Working Group 4 Meeting #72bis 	R4-145726

Singapore, 6th October – 10th October 2014

Source:	DISH Network

Title:	Variable Duplexing for Extended AWS Band

Agenda item:	9.1.1

Document for:	Approval

## 1	Introduction

A new Study Item (SI) on “AWS-Extension Band for LTE” was approved at RAN #64 [1].  The objective of the SI is to evaluate various candidate band plans for an extended AWS band plan and recommend a single band plan to proceed to a Work Item.  This contribution discusses variable duplexing related concerns raised in RAN4 #72 and proposes solutions to address them.

## 2	Discussion

The SI proposes three main band plan options for a new extended AWS band [1]:

- 70+70 (1710 - 1780 MHz / 2110 - 2180 MHz, fixed duplex)

- 70+90 (1710 - 1780 MHz / 2110 - 2200 MHz, variable duplex)

- 85+90 (1695 - 1780 MHz / 2110 - 2200 MHz, variable duplex)

The two asymmetric band plan options (70+90 MHz and 85+90 MHz) are specified with variable duplexing.  sIn RAN4 #72, many contributions [2-6] discussed variable duplexing.  It is pointed [2] that the signalling to support variable duplexing is already in place in the current specifications and RAN4 has implemented a variable duplex scenario for the Band 23 work item [2].   The remaining concerns around variable duplexing are summarized below:

- Specification, testing and verification complexities involving a large number of duplexing spacing options

- Deployment uncertainties involving variable duplexing

Regarding the specification and testing/verification complexities, it was suggested in [2,5] that these requirements could be reduced to specific deployment scenarios so that the cases are bounded.

Regarding the deployment uncertainties, it is proposed that RAN4 address specific deployment scenarios involving variable duplexing during the subsequent work item phase when the band plan and operator requirements are known.

Per the SI objective [1], the decision for variable duplexing support could be made based on “technical feasibility, impediments, interoperability and efficient spectrum utilization”; actual scope and specific implementation of variable duplexing can be addressed in the work item phase.

## 3	Proposal

While the signalling necessary for supporting variable duplexing exists in the current specifications, some concerns regarding testing/verification complexities and deployment uncertainties have been raised previously. Therefore, it is proposed for asymmetric candidate band plans that:

Proposal #1:  Variable duplexing be supported to achieve maximum spectrum utilization and flexibility

Proposal #2:  Variable duplexing related specifications, testing and verification requirements be limited to specific deployment cases and not be left unbounded

Proposal #3:  RAN4 can address specific deployment scenarios requiring variable duplexing, as well as corresponding specifications requirements, in the work item phase

## References

RP-141037, “New Study Item Proposal: AWS-Extension Band for LTE”, Verizon, US Cellular, AT&T, Qualcomm, Alcatel-Lucent, LGE, Motorola Mobility, Samsung, SouthernLINC Wireless, Intel, T-Mobile USA, NSN, Nokia, DISH Network, Broadcom, 3GPP RAN#64, Sophia Antipolis, France, June 2014

R4-144236, “Variable duplexing for extended AWS band”, DISH Network, 3GPP RAN4 #72, Dresden, Germany, August 2014

R4-144241, “Analysis of AWS extension options”, Nokia, 3GPP RAN4 #72, Dresden, Germany, August 2014

R4-144992, “Possible band plans for the AWS extension”, Ericsson, 3GPP RAN4 #72, Dresden, Germany, August 2014

R4-145134, “Variable duplexing options for extended AWS band”, Qualcomm Incorporated, 3GPP RAN4 #72, Dresden, Germany, August 2014

R4-145258, “AWS3 Usage”, Intel Corporation, 3GPP RAN4 #72, Dresden, Germany, August 2014

****  This communication is submitted pursuant to the FCC’s AWS-3 Order, which states, among other things, that industry standard-setting meetings “would not fall within the prohibition in section 1.2105(c) unless they divulge bids or bidding strategies or discuss or negotiate settlement agreements, arrangements or understandings of any kind relating to the licenses being auctioned, including agreements relating to the post-auction market structure.”  See Amendment of the Commission’s Rules with Regard to Commercial Operations in the 1695-1710 MHz, 1755-1780 MHz, and 2155-2180 MHz Bands, Report and Order, 29 FCC Rcd 4610, 4676 (2014) (“AWS-3 Order”).  In light of this, discussions regarding the AWS Extension Study should be able to continue during the period in which the anti-collusion rule is in effect, as 3GPP discussions involve analyses of engineering and technical data regarding the AWS-3 band, including achieving the FCC’s mandates.   As part of the auction process for AWS-3, DISH filed an application to participate as a potential bidder for those spectrum assets. Because of the FCC’s anti-collusion rules, we are not able to discuss what, if any, spectrum resources we may intend to bid on, divulge bids or bidding strategies or discuss or negotiate settlement agreements, arrangements or understandings of any kind relating to the licenses being auctioned, including agreements relating to the post-auction market structure.