3GPP TSG-RAN WG4 #74bis	R4-151339

Rio de Janeiro, Brazil 20th April– 24th April 2015

Source: 	Ericsson

Title:	Interference modelling for TM10 with CRS-IM

Agenda Item:	7.5.1

Document for:	Discussion

# Introduction

In RAN4#74 meeting, the group agreed one WF [1] to encourage companies provide input on the interference modelling for TM10. In this paper, we provide our analysis on the interference modelling for TM10 based on system level simulation.

# System level simulation parameters and methodology for interference modelling

## System level simulation parameters

According to the agreements [1], both scenario 1 and scenario 3 are considered for TM10 interference modelling extraction. Some key system parameters are shown in Table 1.

Table 1: System parameters for TM10

| Parameter | Values used for evaluation |
| --- | --- |
| Deployment scenarios | Scenario 1 and Scenario 3 |
| Number of low power node per macro-cell | N = 4 |
| Marco TX power | 46 dBm |
| Low power node TX power (Ptotal) | 30 dBm |
| System bandwidth | 10 MHz |
| Transmission schemes in DL | DPS |
| Network synchronization | Synchronized |
| Number of antennas at transmission point | 2 Tx |
| Number of antennas at UE | 2 Rx |
| Antenna configuration | Cross polarized |
| Antenna pattern | Follow Annex A 2.1.1.1 Table A.2.1.1-2 in TR36.814 |
| Antenna gain + connector loss | For macro eNB and high-power RRH: 17 dBi in ITU, 14 dBi in 3GPP Case 1 For low power node: 5 dBi |
| Placing of UEs | Uniform distribution for homogeneous networks For heterogeneous networks, placement according to the configuration. |
| Traffic model | non-full buffer traffic model 1 |

## Methodology for interference modelling extraction

The methodology defined in 36.863 [2] are reused. Network interference statistics are computed using the following defined measures. Noc is defined as

Noc=M+2NBSIorj+σ2,

where Iorj is the received power spectral density of the j-th strongest base station measured at serving cell CRS REs (average power obtained within the RE and normalized to the subcarrier spacing), M is the number of dominant interfering cell(s) which interference can be mitigated by the UE. Here, M=2 in this simulation. Specifically, Ior1 denotes the received power spectral density of the instantaneous transmission point (TP), Iorj(j=2,⋯M+1) denotes the received power spectral density of the (j-1)th dominant interfering cells other than the instantaneous TP. The instantaneous TP may be serving cell (e.g, with the strongest RSRP), it may be other cells (e.g, with weaker RSRP). is the power spectral density of thermal noise (average power per RE normalised), and NBS is the total number of base stations considered including the serving cell, The quantity Noc models the interference from all cells, excluding the contribution of the M dominant interfering cell(s) which interference is mitigated.

The interference profiles for the dominant TP are defined by IorjNoc (j=2, ⋯,M+1), and instantaneous TP signal profiles are characterized by Ior1Noc.

Geometry G is defined as

G=Ior1Ioc=Ior1j=2j=M+1Iorj+Noc

Here, all the interfering TPs are assumed with full load.

The development of a method to derive IorjNoc (j=1, ⋯,M+1) based on the following steps:

Step 1: Decide envisioned G value (G) based on the 5% of G distribution assuming full load for all the dominant interfering TPs.

Step 2: Select UEs. From a sample of randomly dropped UEs, select those UEs with G close to G with a tolerance of +/- 0.5 dB. IorjNoc (j=1, ⋯,M+1) values are logged for those UEs, and multiple realizations are performed in order to obtain a significant number of samples.

Step 3: Decide IorjNoc (j=1, ⋯,M+1): After saving the condition IorjNoc (j=1, ⋯,M+1) values from all samples, the Ior2/Noc values are sorted in ascending order. The data set is binned in 5-percentile bands; and a mean of IorjNoc (j=1, ⋯,M+1) inside each 5-percentile band is taken, yielding one characteristic set of IorjNoc (j=1, ⋯,M+1) values per each 5-percentile band. At the end of the process, 20 characteristic set of IorjNoc (j=1, ⋯,M+1) values are obtained.

Note 1:	The “mean” of IorjNoc (j=1, ⋯,M+1) inside each 5-percentile band is performed in the linear domain.

To account for the partial loading on the non-dominant interferers, Noc is obtained as

Noc=M+2NBSαIorj+σ2,

where α∈0,1 is the resource utilization factor in the interfering cells.

Remark: Compared with the non-TM10 case, for TM10, the main difference is the TP selection. In non-TM10 case, the RSRP-based selection scheme is used for cell selection. Hence, the TP is always with the strongest signal. However, in DPS, the UE may select one transmission point with weaker received signal due to scheduler.

In the next sections, the detail system level simulation results for Scenario 1 and Scenario 3 are provided.

# System level simulation results

## Scenario 1

For scenario 1, the distribution of G is shown in Figure 1 when resource utilization is 30%. The selection probability for different strongest cells is shown in Figure 2. From Figure 2, we can see that about 70% UE will select the strongest cell, and about 20% UE will select the second strongest cell and another about 10% UE even select the third strongest cells. Hence, it will lead G  to be much lower than non-TM10 case. As indicated in Figure 1, the G (G value @5%) is about -11.3 dB. It means the interference would be much stronger then the desired signal. The main reason is  G will be lower if the second strongest or the third strongest cell is selected as instantaneous TP. For the UE with  G≈-11.3 dB, the distribution of  I2/Noc and I3/Noc are shown in Table 2.

In Figure 3, (I1/Noc, I2/Noc) for all UEs (denoted by blue circle) and (I1/Noc, I2/Noc) for selected UEs are plotted (Note: For the selected UE, (I1/Noc, I2/Noc) is the average value for each bin with 5-percentile band, they are tabulated in Table 2). They are denoted by blue circle and red circle, respectively. Figure 3 is aligned with previous analysis.

Figure 1: The G distribution for Scenario 1 when RU=30%

Figure 2: The selection probability for different strongest cells

Table 2: Evaluation results on 5%-tile Geometry when RU=30%

| Set | I1/Noc (a.k.a) (Es/Noc) | I2/Noc | I3/Noc |
| --- | --- | --- | --- |
| 1 | -4.8722 | 5.4454 | -2.2775 |
| 2 | -2.0428 | 7.9165 | -0.1307 |
| 3 | 1.4830 | 10.8028 | 8.2887 |
| 4 | 1.9596 | 12.2536 | 6.1547 |
| 5 | 2.6794 | 13.3188 | 6.6509 |
| 6 | 4.1467 | 14.5658 | 9.1613 |
| 7 | 4.6491 | 15.7762 | 7.9699 |
| 8 | 6.0105 | 16.6951 | 7.5045 |
| 9 | 6.9123 | 17.5367 | 9.6198 |
| 10 | 8.4546 | 19.2403 | 12.6606 |
| 11 | 11.6335 | 22.3877 | 16.5353 |
| 12 | 15.4665 | 26.4932 | 15.4923 |
| 13 | 19.6592 | 31.1469 | 18.1418 |
| 14 | 21.7081 | 32.8575 | 24.3303 |
| 15 | 24.3298 | 34.8786 | 27.5571 |
| 16 | 31.4508 | 42.8873 | 31.5810 |
| 17 | 34.5146 | 46.1518 | 32.8900 |
| 18 | 36.8322 | 48.6274 | 31.0735 |
| 19 | 38.6831 | 49.7714 | 37.3632 |
| 20 | 43.6624 | 55.4326 | 43.2946 |

Figure 3: (I1/Noc, I2/Noc) for all UEs (denoted by blue circle) and (I1/Noc, I2/Noc) for selected UEs

## Scenario3

In this section, system level simulations are given for scenario 3. Similar like Scenario 1, the G (G value @5%) is about -13.4 dB as shown in Figure 4.  More than 20% UE select the second strongest cell and another about 10% UE even select the third strongest cells. The distribution of I2/Noc and I3/Noc are shown in Table 3. Figure 6 plots (I1/Noc, I2/Noc) for all UEs and selected UEs. For selected UE, (I1/Noc, I2/Noc) is the average value for each bin with 5-percentile band, and they are tabulated in Table 3.

Figure 4: The G distribution for Scenario 3 when RU=30%

Figure 5: The selection probability for different strongest cells

Table 3: Evaluation results on 5%-tile Geometry when RU=30%

| Set | Es/Noc | D1/Noc | D2/Noc |
| --- | --- | --- | --- |
| 1 | -1.5861 | 9.9957 | 5.6224 |
| 2 | -0.3691 | 11.7585 | 6.2033 |
| 3 | 0.7532 | 13.289 | 6.7534 |
| 4 | 2.6012 | 14.481 | 9.4442 |
| 5 | 3.1425 | 15.5844 | 9.6425 |
| 6 | 4.4689 | 16.8618 | 8.109 |
| 7 | 5.2076 | 17.9787 | 10.9073 |
| 8 | 5.8607 | 19.1158 | 5.6538 |
| 9 | 8.0243 | 20.5376 | 11.4747 |
| 10 | 8.1667 | 21.5157 | 9.1687 |
| 11 | 10.0317 | 23.4173 | 11.2776 |
| 12 | 12.2789 | 24.8902 | 16.039 |
| 13 | 14.028 | 26.882 | 17.2402 |
| 14 | 18.3936 | 31.2032 | 18.5516 |
| 15 | 21.7826 | 34.528 | 26.9571 |
| 16 | 23.0226 | 36.1293 | 21.3879 |
| 17 | 24.4377 | 37.7105 | 23.0591 |
| 18 | 27.4333 | 40.4217 | 29.6012 |
| 19 | 31.6901 | 44.3919 | 34.277 |
| 20 | 45.4645 | 58.1786 | 46.8596 |

Figure 6: (I1/Noc, I2/Noc) for all UEs (denoted by blue circle) and (I1/Noc, I2/Noc) for selected UEs

# Conclusion

In this paper, interference modelling for TM10 is discussed. The interference levels for the UEs with severe interference are provided.

# Reference

- R4-151106, WF on CRS-IM demodulation (TM10), Ericsson, Samsung, LGE, Feb, 2015.

- 3GPP 36.863, Study on Cell-specific Reference Signals (CRS) interference mitigation for homogenous deployments of LTE, 3GPP, Dec. 2103