3GPP TSG-RAN WG4 Meeting #76bis 	R4-155924

Sophia Antipolis, France 12 – 16 October 2015

Source:	DISH Network

Title:	TP for Section 5 of TR36.870

Work item	FS_LTE_AWS_3_4

Agenda item:	9.4.1

Document for:	Approval

Abstract

This document provides a TP for Section 5 (AWS-3/4 Band Plan). (pCR)

## 1.0	Background

In the United States, two recently completed auctions made additional 75 MHz of mid-band spectrum available for wireless broadband.  In early 2014, the FCC successfully auctioned off the PCS H block consisting of 1915-1920 MHz UL and 1995-2020 MHz DL.  In 2015, the successful completion of Auction #97 freed up the following:

- 1695-1710 MHz UL (unpaired AWS-3); 15 MHz of unpaired spectrum

- 1755-1780 MHz UL & 2155-2180 MHz DL (paired AWS-3); 50 MHz of paired spectrum

It is noted that the paired AWS-3 spectrum is currently being standardized in the on-going Band 66 WI [1].  Additionally, the FCC has granted the AWS-4 licensee the flexibility to convert the 2000-2020 MHz block to downlink [2].   The flexibility provides a better harmonization with the neighboring spectrum blocks (Band 2/25 and H block) and thereby allows for more efficient use of the 2000-2020 MHz band by eliminating the co-existence issues relating to its legacy uplink designation.

It is proposed to standardize a new E-UTRA operating band incorporating the newly available mid-band spectrum mentioned above. The proposed band achieves the following key objectives:

- Pairing standalone uplink (1695-1710 MHz) and downlink (2000-2020 MHz) spectrum blocks made available through recent FCC auction and rule changes

- Maximizing the downlink utility by incorporating the adjacent H block downlink (1995-2000 MHz)

Figure 1-1 below depicts the arrangement of the proposed band plan.  It is noted that the AWS-3 unpaired uplink spectrum was auctioned off in two separate blocks; A1 (1695-1700 MHz) and B1 (1700-1710 MHz).

Figure 1-1:  Proposed 15+25 MHz band plan

The proposed band plan is an asymmetric band plan which consists of both paired and unpaired blocks:

- 1695-1710 MHz UL & 1995-2010 MHz DL paired blocks

- 2010-2020 MHz unpaired DL block

Figure 1-2 below depicts the arrangement of the proposed band plan AWS-3/4 [BXX] relative to the adjacent spectrum.

Figure 1-2:  AWS-3/4 [BXX]

## 1.1	Protection of Federal Satellite Receiver sites

In the US, AWS-3 unpaired uplink licensees are required protect legacy government satellite receiver locations which operate in both co-channel and adjacent channel to the 1695-1710 MHz band.  These locations are grouped into 27 protection zones and commercial licensees are required to coordinate with the government in order to operate commercial services within these zones [3].  There are no coordination requirements for commercial licensees to operate outside the protection zones.  These protection zones impact over 10% of the overall US population; however, the population impact will be much bigger if the licenses choose to operate the uplink at higher power than 20 dBm EIRP [4].

Protection of these federal satellite locations will be coordinated based on a certain receiver protection criteria for the satellite receivers within the protection zones.  14 zones out of the 27 zones have satellite receiver frequencies tuned to 1695 MHz and below [4].

Figure 1.1-1 Federal government operations in and around 1695-1710 MHz band

Figure 1.1-2:  1695-1710 MHz protection zones in the continental US [5]

Whether operating outside protection zones or, after successful coordination, within protection zones, commercial licensees will be under an obligation not to cause harmful interference to the co-channel and adjacent channel federal sites.

## 2		Summary

The following text proposal is recommended to be approved for Section 5 of the TR, “Deployment Scenarios”.

## 3	Reference

RP-150428, “New WI: AWS-Extension Band for LTE”, Ericsson, Nokia Networks, Verizon Wireless, AT&T, DISH Network, Rogers, Telus, Samsung, Huawei, HiSilicon, Alcatel-Lucent, LGE, Nokia Corporation, Intel, T-Mobile, US Cellular, 3GPP RAN#67, Shanghai, China, March 2015

FCC, DA 13-2409, “AWS-4 waiver Memorandum Opinion and Order”, December 20th, 2013

FCC, GN Docket No. 13-185, “AWS-3 Report and Order”, March 31st, 2014

FCC, DA 14-1023, “Coordination procedures in the 1695-1710 MHz and 1755-1780 MHz bands”, July 18th, 2014

Commerce Spectrum Management Advisory Committee WG-1 Final Report, July 23rd, 2013

### ---------------- Start of TP -----------------------

# 5	Band plan

## 5.1	Background

In the United States, two recently completed auctions made additional 75 MHz of mid-band spectrum available for wireless broadband.  In early 2014, the FCC successfully auctioned off the PCS H block consisting of 1915-1920 MHz UL and 1995-2020 MHz DL.  In 2015, the successful completion of Auction #97 freed up the following:

- 1695-1710 MHz UL (unpaired AWS-3); 15 MHz of unpaired spectrum

- 1755-1780 MHz UL & 2155-2180 MHz DL (paired AWS-3); 50 MHz of paired spectrum

It is noted that the paired AWS-3 spectrum is currently being standardized in the on-going Band 66 WI [1].  Additionally, the FCC has granted the AWS-4 licensee the flexibility to convert the 2000-2020 MHz block to downlink [2].   The flexibility provides a better harmonization with the neighboring spectrum blocks (Band 2/25 and H block) and thereby allows for more efficient use of the 2000-2020 MHz band by eliminating the co-existence issues relating to its legacy uplink designation.

It is proposed to standardize a new E-UTRA operating band incorporating the newly available mid-band spectrum mentioned above. The proposed band achieves the following key objectives:

- Pairing standalone uplink (1695-1710 MHz) and downlink (2000-2020 MHz) spectrum blocks made available through recent FCC auction and rule changes

- Maximizing the downlink utility by incorporating the adjacent H block downlink (1995-2000 MHz)

Figure 5.1-1 below depicts the arrangement of the proposed band plan.  It is noted that the AWS-3 unpaired uplink spectrum was auctioned off in two separate blocks; A1 (1695-1700 MHz) and B1 (1700-1710 MHz).

Figure 5.1-1:  Proposed 15+25 MHz band plan

The proposed band plan is an asymmetric band plan which consists of both paired and unpaired blocks:

- 1695-1710 MHz UL & 1995-2010 MHz DL paired blocks

- 2010-2020 MHz unpaired DL block

Figure 5.1-2 below depicts the arrangement of the proposed band plan AWS-3/4 [BXX] relative to the adjacent spectrum.

Figure 5.1-2:  AWS-3/4 [BXX]

## 5.1.1	Protection of Federal Satellite Receiver sites

In the US, AWS-3 unpaired uplink licensees are required protect legacy government satellite receiver locations which operate in both co-channel and adjacent channel to the 1695-1710 MHz band.  These locations are grouped into 27 protection zones and commercial licensees are required to coordinate with the government in order to operate commercial services within these zones [3].  There are no coordination requirements for commercial licensees to operate outside the protection zones.  These protection zones impact over 10% of the overall US population; however, the population impact will be much bigger if the licenses choose to operate the uplink at higher power than 20 dBm EIRP [4].

Protection of these federal satellite locations will be coordinated based on a certain receiver protection criteria for the satellite receivers within the protection zones.  14 zones out of the 27 zones have satellite receiver frequencies tuned to 1695 MHz and below [4].

Figure 5.1.1-1 Federal government operations in and around 1695-1710 MHz band

Figure 5.1.1-2:  1695-1710 MHz protection zones in the continental US [5]

Whether operating outside protection zones or, after successful coordination, within protection zones, commercial licensees will be under an obligation not to cause harmful interference to the co-channel and adjacent channel federal sites.

# 6	Deployment scenarios