3GPP TSG-RAN WG4 Meeting #78 	R4-10094

St Julian’s, Malta, 15 – 19 February 2016

Source:	DISH Network, [ ]

Title:	TP for TR36.749v.0.0.0 Section 10 (Channel numbering for E-UTRA, MSR)

Work item:	LTE_AWS_3_4

Agenda item:	8.9

Document for:	Approval

## Introduction

This document provides a TP for TR36.749v.0.0.0 Section 10 (Channel numbering for E-UTRA, MSR. (pCR)

## Introduction

<Text to be added>

## 3.0	Channel numbering for E-UTRA, MSR

Table 3-1 and Table 3-2 include the Operating band number and the EARFCN numbers proposed for Band [XX], respectively.

Table 3-1: Operating band definition for AWS-extension

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| [XX] | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz | FDDx |
| NOTE X: Frequencies within the range of 2010-2020 MHz of the DL operating band may be restricted for E-UTRA carrier aggregation |  |  |  |  |  |  |  |

Table 3-2: EARFCN numbers for Band [XX]

| E-UTRA Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low (MHz) | NOffs-DL | Range of NDL | FUL_low (MHz) | NOffs-UL | Range of NUL |
| [XX] | 1995 | 67836 | 67836-67985 | 1695 | 132972 | 132972-133221 |

## 3.0	References

[3]	3GPP TR 36.870: "Study on New AWS 3/4 Band for LTE".

## Proposal for TR36.749v0.0.0

---- Start of TP -------

# 10	Channel numbering for E-UTRA, MSR

Table 10-1 and Table 10-2 include the Operating band number and the EARFCN numbers proposed for Band [XX], respectively.

Table 10-1: Operating band definition for AWS-extension

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| [XX] | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz | FDDx |
| NOTE X: Frequencies within the range of 2010-2020 MHz of the DL operating band may be restricted for E-UTRA carrier aggregation |  |  |  |  |  |  |  |

Table 10-2: EARFCN numbers for Band [XX]

| E-UTRA Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low (MHz) | NOffs-DL | Range of NDL | FUL_low (MHz) | NOffs-UL | Range of NUL |
| [XX] | 1995 | 67836 | 67836-67985 | 1695 | 132972 | 132972-133221 |