3GPP TSG-RAN WG4 Meeting #78 	R4-160376

# Malta, Malta, 15 – 20, Feb. 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 3423 | rev | - | Current version: | 13.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FRC for TM10 with CRS assistance information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CRSIM-Perf |  |  |  |  |  |  | Date: |  | 2016-2-14 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The FRC for TM10 cases are not captured when CRS assistance information is configured |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add FRC for TM10 cases when CRS assistance informaiton is configured |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no FRC for TM10 cases when CRS assistance information is configured |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS36.521 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

### A.3.1.1	Overview of DL reference measurement channels

In Table A.3.1.1-1 are listed the DL reference measurement channels specified in annexes A.3.2 to A.3.10  of this release of TS 36.101. This table is informative and serves only to a better overview. The reference for the concrete reference measurement channels and corresponding implementation’s parameters as to be used for requirements are annexes A.3.2 to A.3.10  as appropriate.

Table A.3.1.1-1: Overview of DL reference measurement channels

| Duplex | Table | Name | BW | Mod | TCR | RB | RB Offset | UE Categ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD, Receiver requirements |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.2-1 |  | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| FDD | Table A.3.2-1 |  | 3 | QPSK | 1/3 | 15 |  | ≥ 1 |  |
| FDD | Table A.3.2-1 |  | 5 | QPSK | 1/3 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.2-1 |  | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.2-1 |  | 15 | QPSK | 1/3 | 75 |  | ≥ 1 |  |
| FDD | Table A.3.2-1 |  | 20 | QPSK | 1/3 | 100 |  | ≥ 1 |  |
| FDD / HD-FDD | Table A.3.2-1a |  | 1.4 | QPSK | 1/3 | 6 |  | 0 |  |
| FDD / HD-FDD | Table A.3.2-1a |  | 3 | QPSK | 1/3 | 14 |  | 0 |  |
| FDD / HD-FDD | Table A.3.2-1a |  | 5 | QPSK | 1/3 | 14 |  | 0 |  |
| FDD / HD-FDD | Table A.3.2-1a |  | 10 | QPSK | 1/3 | 14 |  | 0 |  |
| FDD / HD-FDD | Table A.3.2-1a |  | 15 | QPSK | 1/3 | 14 |  | 0 |  |
| FDD / HD-FDD | Table A.3.2-1a |  | 20 | QPSK | 1/3 | 14 |  | 0 |  |
| TDD, Receiver requirements |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.2-2 |  | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.2-2 |  | 3 | QPSK | 1/3 | 15 |  | ≥ 1 |  |
| TDD | Table A.3.2-2 |  | 5 | QPSK | 1/3 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.2-2 |  | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.2-2 |  | 15 | QPSK | 1/3 | 75 |  | ≥ 1 |  |
| TDD | Table A.3.2-2 |  | 20 | QPSK | 1/3 | 100 |  | ≥ 1 |  |
| TDD | Table A.3.2-2a |  | 1.4 | QPSK | 1/3 | 6 |  | 0 |  |
| TDD | Table A.3.2-2a |  | 3 | QPSK | 1/3 | 14 |  | 0 |  |
| TDD | Table A.3.2-2a |  | 5 | QPSK | 1/3 | 14 |  | 0 |  |
| TDD | Table A.3.2-2a |  | 10 | QPSK | 1/3 | 14 |  | 0 |  |
| TDD | Table A.3.2-2a |  | 15 | QPSK | 1/3 | 14 |  | 0 |  |
| TDD | Table A.3.2-2a |  | 20 | QPSK | 1/3 | 14 |  | 0 |  |
| FDD, Receiver requirements, Maximum input level for UE Categories 3-5 |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.2-3 |  | 1.4 | 64QAM | 3/4 | 6 |  | - |  |
| FDD | Table A.3.2-3 |  | 3 | 64QAM | 3/4 | 15 |  | - |  |
| FDD | Table A.3.2-3 |  | 5 | 64QAM | 3/4 | 25 |  | - |  |
| FDD | Table A.3.2-3 |  | 10 | 64QAM | 3/4 | 50 |  | - |  |
| FDD | Table A.3.2-3 |  | 15 | 64QAM | 3/4 | 75 |  | - |  |
| FDD | Table A.3.2-3 |  | 20 | 64QAM | 3/4 | 100 |  | - |  |
| FDD, Receiver requirements, Maximum input level for UE Categories 1 |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.2-3a |  | 1.4 | 64QAM | 3/4 | 6 |  | - |  |
| FDD | Table A.3.2-3a |  | 3 | 64QAM | 3/4 | 15 |  | - |  |
| FDD | Table A.3.2-3a |  | 5 | 64QAM | 3/4 | 18 |  | - |  |
| FDD | Table A.3.2-3a |  | 10 | 64QAM | 3/4 | 17 |  | - |  |
| FDD | Table A.3.2-3a |  | 15 | 64QAM | 3/4 | 17 |  | - |  |
| FDD | Table A.3.2-3a |  | 20 | 64QAM | 3/4 | 17 |  | - |  |
| FDD, Receiver requirements, Maximum input level for UE Categories 2 |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.2-3b |  | 1.4 | 64QAM | 3/4 | 6 |  | - |  |
| FDD | Table A.3.2-3b |  | 3 | 64QAM | 3/4 | 15 |  | - |  |
| FDD | Table A.3.2-3b |  | 5 | 64QAM | 3/4 | 25 |  | - |  |
| FDD | Table A.3.2-3b |  | 10 | 64QAM | 3/4 | 50 |  | - |  |
| FDD | Table A.3.2-3b |  | 15 | 64QAM | 3/4 | 75 |  | - |  |
| FDD | Table A.3.2-3b |  | 20 | 64QAM | 3/4 | 83 |  | - |  |
| FDD, Receiver requirements, Maximum input level for UE DL Categories 0 |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.2-3c |  | 1.4 | 64QAM | 3/4 | 2 |  | - |  |
| FDD | Table A.3.2-3c |  | 3 | 64QAM | 3/4 | 2 |  | - |  |
| FDD | Table A.3.2-3c |  | 5 | 64QAM | 3/4 | 2 |  | - |  |
| FDD | Table A.3.2-3c |  | 10 | 64QAM | 3/4 | 2 |  | - |  |
| FDD | Table A.3.2-3c |  | 15 | 64QAM | 3/4 | 2 |  | - |  |
| FDD | Table A.3.2-3c |  | 20 | 64QAM | 3/4 | 2 |  | - |  |
| TDD, Receiver requirements, Maximum input level for UE Categories 3-5 |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.2-4 |  | 1.4 | 64QAM | 3/4 | 6 |  | - |  |
| TDD | Table A.3.2-4 |  | 3 | 64QAM | 3/4 | 15 |  | - |  |
| TDD | Table A.3.2-4 |  | 5 | 64QAM | 3/4 | 25 |  | - |  |
| TDD | Table A.3.2-4 |  | 10 | 64QAM | 3/4 | 50 |  | - |  |
| TDD | Table A.3.2-4 |  | 15 | 64QAM | 3/4 | 75 |  | - |  |
| TDD | Table A.3.2-4 |  | 20 | 64QAM | 3/4 | 100 |  | - |  |
| TDD, Receiver requirements, Maximum input level for UE Categories 1 |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.2-4a |  | 1.4 | 64QAM | 3/4 | 6 |  | - |  |
| TDD | Table A.3.2-4a |  | 3 | 64QAM | 3/4 | 15 |  | - |  |
| TDD | Table A.3.2-4a |  | 5 | 64QAM | 3/4 | 18 |  | - |  |
| TDD | Table A.3.2-4a |  | 10 | 64QAM | 3/4 | 17 |  | - |  |
| TDD | Table A.3.2-4a |  | 15 | 64QAM | 3/4 | 17 |  | - |  |
| TDD | Table A.3.2-4a |  | 20 | 64QAM | 3/4 | 17 |  | - |  |
| TDD, Receiver requirements, Maximum input level for UE Categories 2 |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.2-4b |  | 1.4 | 64QAM | 3/4 | 6 |  | - |  |
| TDD | Table A.3.2-4b |  | 3 | 64QAM | 3/4 | 15 |  | - |  |
| TDD | Table A.3.2-4b |  | 5 | 64QAM | 3/4 | 25 |  | - |  |
| TDD | Table A.3.2-4b |  | 10 | 64QAM | 3/4 | 50 |  | - |  |
| TDD | Table A.3.2-4b |  | 15 | 64QAM | 3/4 | 75 |  | - |  |
| TDD | Table A.3.2-4b |  | 20 | 64QAM | 3/4 | 83 |  | - |  |
| TDD, Receiver requirements, Maximum input level for UE DL Categories 0 |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.2-4c |  | 1.4 | 64QAM | 3/4 | 2 |  | - |  |
| TDD | Table A.3.2-4c |  | 3 | 64QAM | 3/4 | 2 |  | - |  |
| TDD | Table A.3.2-4c |  | 5 | 64QAM | 3/4 | 2 |  | - |  |
| TDD | Table A.3.2-4c |  | 10 | 64QAM | 3/4 | 2 |  | - |  |
| TDD | Table A.3.2-4c |  | 15 | 64QAM | 3/4 | 2 |  | - |  |
| TDD | Table A.3.2-4c |  | 20 | 64QAM | 3/4 | 2 |  | - |  |
| FDD, Receiver requirements, Maximum input level for UE Categories 11/12/13/14/15 and UE DL categories 13,14 |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.2-5 |  | 1.4 | 256QAM | 4/5 | 6 |  | - |  |
| FDD | Table A.3.2-5 |  | 3 | 256QAM | 4/5 | 15 |  | - |  |
| FDD | Table A.3.2-5 |  | 5 | 256QAM | 4/5 | 25 |  | - |  |
| FDD | Table A.3.2-5 |  | 10 | 256QAM | 4/5 | 50 |  | - |  |
| FDD | Table A.3.2-5 |  | 15 | 256QAM | 4/5 | 75 |  | - |  |
| FDD | Table A.3.2-5 |  | 20 | 256QAM | 4/5 | 100 |  | - |  |
| TDD, Receiver requirements, Maximum input level for UE Categories 11/12/13/14/15 and UE DL categories 13,14 |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.2-6 |  | 1.4 | 256QAM | 4/5 | 6 |  | - |  |
| TDD | Table A.3.2-6 |  | 3 | 256QAM | 4/5 | 15 |  | - |  |
| TDD | Table A.3.2-6 |  | 5 | 256QAM | 4/5 | 25 |  | - |  |
| TDD | Table A.3.2-6 |  | 10 | 256QAM | 4/5 | 50 |  | - |  |
| TDD | Table A.3.2-6 |  | 15 | 256QAM | 4/5 | 75 |  | - |  |
| TDD | Table A.3.2-6 |  | 20 | 256QAM | 4/5 | 100 |  | - |  |
| FDD, PDSCH Performance, Single-antenna transmission (CRS) |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.3.1-1 | R.4 FDD | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-1 | R.42 FDD | 20 | QPSK | 1/3 | 100 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-1 | R.42-1 FDD | 3 | QPSK | 1/3 | 15 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-1 | R.42-2 FDD | 5 | QPSK | 1/3 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-1 | R.42-3 FDD | 15 | QPSK | 1/3 | 75 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-1 | R.2 FDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-2 | R.3-1 FDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-2 | R.3 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.1-3 | R.5 FDD | 3 | 64QAM | 3/4 | 15 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-3 | R.6 FDD | 5 | 64QAM | 3/4 | 25 |  | ≥ 2 |  |
| FDD | Table A.3.3.1-3 | R.7 FDD | 10 | 64QAM | 3/4 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.1-3 | R.8 FDD | 15 | 64QAM | 3/4 | 75 |  | ≥ 2 |  |
| FDD | Table A.3.3.1-3 | R.9 FDD | 20 | 64QAM | 3/4 | 100 |  | ≥ 3 |  |
| FDD | Table A.3.3.1-3a | R.6-1 FDD | 5 | 64QAM | 3/4 | 18 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-3a | R.7-1 FDD | 10 | 64QAM | 3/4 | 17 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-3a | R.8-1 FDD | 15 | 64QAM | 3/4 | 17 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-3a | R.9-1 FDD | 20 | 64QAM | 3/4 | 17 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-3a | R.9-2 FDD | 20 | 64QAM | 3/4 | 83 |  | ≥ 2 |  |
| FDD | Table A.3.3.1-6 | R.41 FDD | 10 | QPSK | 1/10 | 50 |  | ≥ 1 |  |
| FDD, PDSCH Performance, Single-antenna transmission (CRS), Single PRB (Channel edge) |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.3.1-4 | R.0 FDD | 3 | 16QAM | 1/2 | 1 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-4 | R.1 FDD | 10 / 20 | 16QAM | 1/2 | 1 |  | ≥ 1 |  |
| FDD, PDSCH Performance, Single-antenna transmission (CRS), Single PRB (MBSFN Configuration) |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.3.1-5 | R.29 FDD | 10 | 16QAM | 1/2 | 1 |  | ≥ 1 |  |
| FDD, PDSCH Performance: Carrier aggregation with power imbalance |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.3.1-7 | R.49 FDD | 20 | 64QAM | 0.84-0.87 | 100 |  | ≥ 5 |  |
| FDD | Table A.3.3.1-7 | R.49-1 FDD | 10 | 64QAM | 0.84-0.87 | 50 |  | ≥2 |  |
| FDD | Table A.3.3.1-7 | R.49-2 FDD | 5 | 64QAM | 0.84-0.86 | 25 |  | ≥2 |  |
| FDD, PDSCH Performance, Multi-antenna transmission (CRS), Two antenna ports |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.3.2.1-1 | R.10 FDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-1 | R.10-2 FDD | 5 | QPSK | 1/3 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-1 | R.11 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-1 | R.11-2 FDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-1 | R.11-3 FDD | 10 | 16QAM | 1/2 | 40 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-1 | R.11-4 FDD | 10 | QPSK | 1/2 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-1 | R.30 FDD | 20 | 16QAM | 1/2 | 100 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-1 | R.30-1 FDD | 15 | 16QAM | 1/2 | 75 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-1 | R.35 FDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-1 | R.35-1 FDD | 20 | 64QAM | 0.39 | 100 |  | 4 |  |
| FDD | Table A.3.3.2.1-1 | R.35-2 FDD | 15 | 64QAM | 0.39 | 75 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-1 | R.35-3 FDD | 10 | 64QAM | 0.39 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-2 | R.35-4 FDD | 10 | 64QAM | 0.47 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-2 | R.46 FDD | 10 | QPSK |  | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.47 FDD | 10 | 16QAM |  | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.11-5 FDD | 1.4 | 16QAM | 1/2 | 6 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.11-6 FDD | 3 | 16QAM | 1/2 | 15 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.11-7 FDD | 15 | 16QAM | 1/2 | 75 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-2 | R.11-8 FDD | 10 | QPSK | 3/5 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-2 | R.11-9 FDD | 10 | QPSK | 0.58 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.11-10 FDD | 10 | QPSK | 0.67 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.65 FDD | 10 | 256QAM | 0.6 | 50 |  | 11-15 |  |
| FDD | Table A.3.3.2.1-3 | R. 62 FDD | 10 | 16QAM | 1/2 | 3 |  | 0 |  |
| FDD | Table A.3.3.2.1-3 | R.63 FDD | 10 | 64QAM | 1/2 | 1 |  | 0 |  |
| FDD, PDSCH Performance, Multi-antenna transmission (CRS), Four antenna ports |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.3.2.2-1 | R.12 FDD | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.13 FDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.14 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.2-1 | R.14-1 FDD | 10 | 16QAM | 1/2 | 6 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.14-2 FDD | 10 | 16QAM | 1/2 | 3 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.14-3 FDD | 20 | 16QAM | 1/2 | 100 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.2-1 | R.36 FDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.2-1 | R.14-4 FDD | 1.4 | 16QAM | 1/2 | 6 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.14-5 FDD | 3 | 16QAM | 1/2 | 15 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.14-6 FDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.14-7 FDD | 15 | 16QAM | 1/2 | 75 |  | ≥ 2 |  |
| FDD, PDSCH Performance (UE specific RS) without CSI-RS |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.3.3.0-1 | R.70 FDD | 10 | QPSK | 0.65 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.3.0-1 | R.71 FDD | 10 | 16QAM | 0.6 | 50 |  | ≥ 2 |  |
| FDD, PDSCH Performance (UE specific RS) Two antenna ports (CSI-RS) |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.3.3.1-1 | R.51 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD, PDSCH Performance (UE specific RS) Two antenna ports (CSI-RS, non Quasi Co-located) |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.3.3.1-2 | R.52 FDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.3.1-2 | R.52-1 FDD | 10 | 16QAM | 2/3 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.3.1-2 | R.53 FDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.3.1-2 | R.54 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD, PDSCH Performance (UE specific RS) Four antenna ports (CSI-RS) |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.3.3.2-1 | R.43 FDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.3.2-1 | R.50 FDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.3.2-2 | R.44 FDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.3.2-2 | R.45 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.3.2-2 | R.45-1 FDD | 10 | 16QAM | 1/2 | 39 |  | ≥ 1 |  |
| FDD | Table A.3.3.3.2-1 | R.48 FDD | 10 | QPSK |  | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.3.2-2 | R.60 FDD | 10 | QPSK | 1/2 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.3.2-3 | R.64 FDD | 10 | QPSK | 1/3 | 6 |  | 0 |  |
| FDD | Table A.3.3.3.2-1 | R.66 FDD | 10 | 64QAM |  | 50 |  | 11-15 |  |
| FDD | Table A.3.3.3.2-1 | R.69 FDD | 10 | QPSK | 0.74-0.8 | 50 |  | ≥ 1 |  |
| TDD, PDSCH Performance, Single-antenna transmission (CRS) |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.1-1 | R.4 TDD | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-1 | R.42 TDD | 20 | QPSK | 1/3 | 100 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-1 | R.2 TDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-1 | R.2A TDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-1 | R.42-1 TDD | 3 | QPSK | 1/3 | 15 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-1 | R.42-2 TDD | 5 | QPSK | 1/3 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-1 | R.42-3 TDD | 15 | QPSK | 1/3 | 75 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-2 | R.3-1 TDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-2 | R.3 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.1-3 | R.5 TDD | 3 | 64QAM | 3/4 | 15 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-3 | R.6 TDD | 5 | 64QAM | 3/4 | 25 |  | ≥ 2 |  |
| TDD | Table A.3.4.1-3 | R.7 TDD | 10 | 64QAM | 3/4 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.1-3 | R.8 TDD | 15 | 64QAM | 3/4 | 75 |  | ≥ 2 |  |
| TDD | Table A.3.4.1-3 | R.9 TDD | 20 | 64QAM | 3/4 | 100 |  | ≥ 3 |  |
| TDD | Table A.3.4.1-3a | R.6-1 TDD | 5 | 64QAM | 3/4 | 18 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-3a | R.7-1 TDD | 10 | 64QAM | 3/4 | 17 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-3a | R.8-1 TDD | 15 | 64QAM | 3/4 | 17 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-3a | R.9-1 TDD | 20 | 64QAM | 3/4 | 17 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-3a | R.9-2 TDD | 20 | 64QAM | 3/4 | 83 |  | ≥ 2 |  |
| TDD | Table A.3.4.1-6 | R.41 TDD | 10 | QPSK | 1/10 | 50 |  | ≥ 1 |  |
| TDD, PDSCH Performance, Single-antenna transmission (CRS), Single PRB (Channel edge) |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.1-4 | R.0 TDD | 3 | 16QAM | 1/2 | 1 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-4 | R.1 TDD | 10 / 20 | 16QAM | 1/2 | 1 |  | ≥ 1 |  |
| TDD, PDSCH Performance, Single-antenna transmission (CRS), Single PRB (MBSFN Configuration) |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.1-5 | R.29 TDD | 10 | 16QAM | 1/2 | 1 |  | ≥ 1 |  |
| TDD, PDSCH Performance: Carrier aggregation with power imbalance |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.1-7 | R.49 TDD | 20 | 64QAM | 0.81-087 | 100 |  | ≥ 5 |  |
| TDD | Table A.3.4.1-7 | R.49-1 TDD | 15 | 64QAM | 0.80-0.86 | 75 |  | ≥ 3 |  |
| TDD, PDSCH Performance, Multi-antenna transmission (CRS), Two antenna ports |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.2.1-1 | R.10 TDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-1 | R.11 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-1 | R.11-1 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-1 | R.11-2 TDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-1 | R.11-3 TDD | 10 | 16QAM | 1/2 | 40 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-1 | R.11-4 TDD | 10 | QPSK | 1/2 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-1 | R.30 TDD | 20 | 16QAM | 1/2 | 100 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-1 | R.30-1 TDD | 20 | 16QAM | 1/2 | 100 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-1 | R.30-2 TDD | 20 | 16QAM | 1/2 | 100 |  | 3 |  |
| TDD | Table A.3.4.2.1-1 | R.35 TDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-1 | R.35-1 TDD | 20 | 64QAM | 0.39 | 100 |  | 4 |  |
| TDD | Table A.3.4.2.1-2 | R.35-2 TDD | 10 | 64QAM | 0.47 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-2 | R.46 TDD | 10 | QPSK |  | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.47 TDD | 10 | 16QAM |  | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.11-5 TDD | 1.4 | 16QAM | 1/2 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.11-6 TDD | 3 | 16QAM | 1/2 | 15 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.11-7 TDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.11-8 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-2 | R.11-9 TDD | 15 | 16QAM | 1/2 | 75 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-2 | R.11-10 TDD | 10 | QPSK | 3/5 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-2 | R.11-11 TDD | 10 | QPSK | 0.48-0.58 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.11-12 TDD | 10 | QPSK | 0.54-0.66 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-3 | R.62 TDD | 10 | 16QAM | 1/2 | 3 |  | 0 |  |
| TDD | Table A.3.4.2.1-3 | R.63 TDD | 10 | 64QAM | 1/2 | 1 |  | 0 |  |
| TDD | Table A.3.4.2.1-4 | R.65 TDD | 20 | 256QAM | 0.6 | 100 |  | 11-15 |  |
| TDD | Table A.3.4.2.1-5 | R.67 TDD | 10 | 16QAM | 0.4 | 50 |  | ≥ 1 |  |
| TDD, PDSCH Performance, Multi-antenna transmission (CRS), Four antenna ports |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.2.2-1 | R.12 TDD | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.13 TDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.14 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.2-1 | R.14-1 TDD | 10 | 16QAM | 1/2 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.14-2 TDD | 10 | 16QAM | 1/2 | 3 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.43 TDD | 20 | 16QAM | 1/2 | 100 |  | ≥2 |  |
| TDD | Table A.3.4.2.2-1 | R.36 TDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.2-1 | R.43-1 TDD | 1.4 | 16QAM | 1/2 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.43-2 TDD | 3 | 16QAM | 1/2 | 15 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.43-3 TDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.43-4 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.2-1 | R.43-5 TDD | 15 | 16QAM | 1/2 | 75 |  | ≥ 2 |  |
| TDD, PDSCH Performance, Single antenna port (DRS) |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.3.1-1 | R.25 TDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.3.1-1 | R.26 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.3.1-1 | R.26-1 TDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.4.3.1-1 | R.27 TDD | 10 | 64QAM | 3/4 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.3.1-1 | R.27-1 TDD | 10 | 64QAM | 3/4 | 18 |  | ≥ 1 |  |
| TDD | Table A.3.4.3.1-1 | R.28 TDD | 10 | 16QAM | 1/2 | 1 |  | ≥ 1 |  |
| TDD, PDSCH Performance, Two antenna ports (DRS) |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.3.2-1 | R.31 TDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.3.2-1 | R.32 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.3.2-1 | R.32-1 TDD | 5 | 16QAM | 1/2 | [25] |  | ≥ 1 |  |
| TDD | Table A.3.4.3.2-1 | R.33 TDD | 10 | 64QAM | 3/4 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.3.2-1 | R.33-1 TDD | 10 | 64QAM | 3/4 | [18] |  | ≥ 1 |  |
| TDD | Table A.3.4.3.2-1 | R.34 TDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.3.2 | R.70 TDD | 10 | QPSK | 0.54-0.65 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.3.2 | R.71 TDD | 10 | 16QAM | 0.5-0.6 | 50 |  | ≥ 2 |  |
| TDD, PDSCH Performance (UE specific RS) Two antenna ports (CSI-RS) |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.3.3-1 | R.51 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD, PDSCH Performance (UE specific RS) Two antenna ports (CSI-RS, non Quasi Co-located) |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.3.3-2 | R.52 TDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.3.3-2 | R.52-1 TDD | 10 | 16QAM | 2/3 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.3.3-2 | R.53 TDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.3.3-2 | R.54 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD, PDSCH Performance (UE specific RS) Four antenna ports (CSI-RS) |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.3.4-1 | R.44 TDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.3.4-1 | R.48 TDD | 10 | QPSK |  | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.3.4-2 | R.60 TDD | 10 | QPSK | 1/2 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.3.4-2 | R.61 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.3.4-2 | R.61-1 TDD | 10 | 16QAM | 1/2 | 39 |  | ≥ 1 |  |
| TDD | Table A.3.4.3.4-3 | R.64 TDD | 10 | QPSK | 1/3 | 6 |  | 0 |  |
| TDD | Table A.3.4.3.4-1 | R.66 TDD | 20 | 256QAM |  | 100 |  | 11-15 |  |
| TDD | Table A.3.4.3.4-1 | R.69 TDD | 10 | QPSK | 0.61-0.8 | 50 |  | ≥ 1 |  |
| TDD, PDSCH Performance (UE specific RS) Eight antenna ports (CSI-RS) |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.3.5-1 | R.50 TDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.3.5-2 | R.45 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.3.5-2 | R.45-1 TDD | 10 | 16QAM | 1/2 | 39 |  | ≥ 1 |  |
| FDD, PDCCH / PCFICH Performance |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.5.1-1 | R.15 FDD | 10 | PDCCH |  |  |  |  |  |
| FDD | Table A.3.5.1-1 | R.15-1 FDD | 10 | PDCCH |  |  |  |  |  |
| FDD | Table A.3.5.1-1 | R.15-2 FDD | 10 | PDCCH |  |  |  |  |  |
| FDD | Table A.3.5.1-1 | R.16 FDD | 10 | PDCCH |  |  |  |  |  |
| FDD | Table A.3.5.1-1 | R.17 FDD | 5 | PDCCH |  |  |  |  |  |
| TDD, PDCCH / PCFICH Performance |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.5.2-1 | R.15 TDD | 10 | PDCCH |  |  |  |  |  |
| TDD | Table A.3.5.2-1 | R.15-1 TDD | 10 | PDCCH |  |  |  |  |  |
| TDD | Table A.3.5.2-1 | R.15-2 TDD | 10 | PDCCH |  |  |  |  |  |
| TDD | Table A.3.5.2-1 | R.16 TDD | 10 | PDCCH |  |  |  |  |  |
| TDD | Table A.3.5.2-1 | R.17 TDD | 5 | PDCCH |  |  |  |  |  |
| FDD / TDD, PHICH Performance |  |  |  |  |  |  |  |  |  |
| FDD / TDD | Table A.3.6-1 | R.18 | 10 | PHICH |  |  |  |  |  |
| FDD / TDD | Table A.3.6-1 | R.19 | 10 | PHICH |  |  |  |  |  |
| FDD | Table A.3.6.1 | R.19-1 | 5 | PHICH |  |  |  |  |  |
| FDD / TDD | Table A.3.6-1 | R.20 | 5 | PHICH |  |  |  |  |  |
| FDD / TDD | Table A.3.6-1 | R.24 | 10 | PHICH |  |  |  |  |  |
| FDD / TDD, PBCH Performance |  |  |  |  |  |  |  |  |  |
| FDD / TDD | Table A.3.7-1 | R.21 | 1.4 | QPSK | 40/ 1920 |  |  |  |  |
| FDD / TDD | Table A.3.7-1 | R.22 | 1.4 | QPSK | 40/ 1920 |  |  |  |  |
| FDD / TDD | Table A.3.7-1 | R.23 | 1.4 | QPSK | 40/ 1920 |  |  |  |  |
| FDD, PMCH Performance |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.8.1-1 | R.40 FDD | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-1 | R.37 FDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-2 | R.38 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-3 | R.39-1 FDD | 5 | 64QAM | 2/3 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-3 | R.39 FDD | 10 | 64QAM | 2/3 | 50 |  | ≥ 2 |  |
| TDD, PMCH Performance |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.8.2-1 | R.40 TDD | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.8.2-1 | R.37 TDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.8.2-2 | R.38 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.8.2-3 | R.39-1 TDD | 5 | 64QAM | 2/3 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.8.2-3 | R.39 TDD | 10 | 64QAM | 2/3 | 50 |  | ≥ 2 |  |
| FDD, Sustained data rate (CRS) |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.9.1-1 | R.31-1 FDD | 10 | 64QAM | 0.40 |  |  | ≥ 1 |  |
| FDD | Table A.3.9.1-1 | R.31-2 FDD | 10 | 64QAM | 0.59-0.64 |  |  | ≥ 2 |  |
| FDD | Table A.3.9.1-1 | R.31-3 FDD | 20 | 64QAM | 0.59-0.62 |  |  | ≥ 2 |  |
| FDD | Table A.3.9.1-1 | R.31-3A FDD | 10 | 64QAM | 0.85-0.90 |  |  | ≥ 2 |  |
| FDD | Table A.3.9.1-1 | R.31-3C FDD | 15 | 64QAM | 0.87-0.91 |  |  | ≥ 3 |  |
| FDD | Table A.3.9.1-1 | R.31-4 FDD | 20 | 64QAM | 0.87-0.90 |  |  | ≥ 3 |  |
| FDD | Table A.3.9.1-1 | R.31-4B FDD | 15 | 64QAM | 0.85-0.88 |  |  | ≥ 4 |  |
| FDD | Table A.3.9.1-1 | R.31-5 FDD | 15 | 64QAM | 0.85-0.91 |  |  | ≥ 3 |  |
| FDD | Table A.3.9.1-2 | R.31-6 FDD | 15 | 64QAM | 0.83-0.85 |  |  | ≥ 2 |  |
| FDD | Table A.3.9.1-3 | R.68 FDD | 20 | 256QAM | 0.74-0.85 |  |  | 11-12 |  |
| FDD | Table A.3.9.1-3 | R.68-1 FDD | 15 | 256QAM | 0.74-0.88 |  |  | 11-12 |  |
| FDD | Table A.3.9.1-3 | R.68-2 FDD | 10 | 256QAM | 0.74-0.85 |  |  | 11-12 |  |
| FDD | Table A.3.9.1-3 | R.68-3 FDD | 5 | 256QAM | 0.77-0.85 |  |  | 11-12 |  |
| TDD, Sustained data rate (CRS) |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.9.2-1 | R.31-1 TDD | 10 | 64QAM | 0.40 |  |  | ≥ 1 |  |
| TDD | Table A.3.9.2-1 | R.31-2 TDD | 10 | 64QAM | 0.59-0.64 |  |  | ≥ 2 |  |
| TDD | Table A.3.9.2-1 | R.31-3 TDD | 20 | 64QAM | 0.59-0.62 |  |  | ≥ 2 |  |
| TDD | Table A.3.9.2-1 | R.31-3A TDD | 15 | 64QAM | 0.87-0.90 |  |  | ≥ 2 |  |
| TDD | Table A.3.9.2-1 | R.31-4 TDD | 20 | 64QAM | 0.87-0.90 |  |  | ≥ 3 |  |
| TDD | Table A.3.9.2-1 | R.31-4A TDD | 20 | 64QAM | 0.87-0.90 |  |  | ≥ 3 |  |
| TDD | Table A.3.9.2-1 | R.31-5 TDD | 15 | 64QAM | 0.85-0.88 |  |  | ≥ 3 |  |
| TDD | Table A.3.9.2-1 | R.31-5A TDD | 15 | 64QAM | 0.85-0.88 |  |  | ≥ 3 |  |
| TDD | Table A.3.9.2-2 | R.68 TDD | 20 | 256QAM |  |  |  | 11-12 |  |
| TDD | Table A.3.9.2-2 | R.68-1 TDD | 15 | 256QAM |  |  |  | 11-12 |  |
| TDD | Table A.3.9.2-2 | R.68-2 TDD | 10 | 256QAM |  |  |  | 11-12 |  |
| TDD | Table A.3.9.2-2 | R.68-3 TDD | 20 | 256QAM |  |  |  | 11-12 |  |
| TDD | Table A.3.9.2-2 | R.68-4 TDD | 15 | 256QAM |  |  |  | 11-12 |  |
| FDD, Sustained data rate test with EPDCCH scheduling (CRS) |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.9.3-1 | R.31E-1 FDD | 10 | 64QAM | 0.40-0 |  |  | ≥ 1 |  |
| FDD | Table A.3.9.3-1 | R.31E-2 FDD | 10 | 64QAM | 0.59-0.66 |  |  | ≥ 2 |  |
| FDD | Table A.3.9.3-1 | R.31E-3 FDD | 20 | 64QAM | 0.59-0.63 |  |  | ≥ 2 |  |
| FDD | Table A.3.9.1-1 | R.31E-3C FDD | 15 | 64QAM | 0.87-0.92 |  |  | ≥ 3 |  |
| FDD | Table A.3.9.3-1 | R.31E-3A FDD | 10 | 64QAM | 0.85-0.92 |  |  | ≥ 2 |  |
| FDD | Table A.3.9.3-1 | R.31E-4 FDD | 20 | 64QAM | 0.87-0.91 |  |  | ≥ 3 |  |
| FDD | Table A.3.9.1-1 | R.31E-4B FDD | 15 | 64QAM | 0.87-0.90 |  |  | ≥ 4 |  |
| TDD, Sustained data rate test with EPDCCH scheduling (CRS) |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.9.4-1 | R.31E-1 TDD | 10 | 64QAM | 0.40-0.41 |  |  | ≥ 1 |  |
| TDD | Table A.3.9.4-1 | R.31E-2 TDD | 10 | 64QAM | 0.59-0.65 |  |  | ≥ 2 |  |
| TDD | Table A.3.9.4-1 | R.31E-3 TDD | 20 | 64QAM | 0.59-0.63 |  |  | ≥ 2 |  |
| TDD | Table A.3.9.4-1 | R.31E-3A TDD | 15 | 64QAM | 0.87-0.92 |  |  | ≥ 2 |  |
| TDD | Table A.3.9.4-1 | R.31E-4 TDD | 20 | 64QAM | 0.87-0.90 |  |  | ≥ 3 |  |
| FDD, ePDCCH performance |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.10.1-1 | R.55 FDD | 10 | EPDCCH |  |  |  |  |  |
| FDD | Table A.3.10.1-1 | R.56 FDD | 10 | EPDCCH |  |  |  |  |  |
| FDD | Table A.3.10.1-1 | R.57 FDD | 10 | EPDCCH |  |  |  |  |  |
| FDD | Table A.3.10.1-1 | R.58 FDD | 10 | EPDCCH |  |  |  |  |  |
| FDD | Table A.3.10.1-1 | R.59 FDD | 10 | EPDCCH |  |  |  |  |  |
| TDD, ePDCCH performance |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.10.2-1 | R.55 TDD | 10 | EPDCCH |  |  |  |  |  |
| TDD | Table A.3.10.2-1 | R.56 TDD | 10 | EPDCCH |  |  |  |  |  |
| TDD | Table A.3.10.2-1 | R.57 TDD | 10 | EPDCCH |  |  |  |  |  |
| TDD | Table A.3.10.2-1 | R.58 TDD | 10 | EPDCCH |  |  |  |  |  |
| TDD | Table A.3.10.2-1 | R.59 TDD | 10 | EPDCCH |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

#### A.3.3.3.1	Two antenna port (CSI-RS)

The reference measurement channels in Table A.3.3.3.1-1 apply for verifying demodulation performance for UE-specific reference symbols with two cell-specific antenna ports and two CSI-RS antenna ports.

Table A.3.3.3.1-1: Fixed Reference Channel for CDM-multiplexed DM RS with two CSI-RS antenna ports

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | R.51 FDD |
| Channel bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 (Note 3) |
| Allocated subframes per Radio Frame |  | 9 |
| Modulation |  | 16QAM |
| Target Coding Rate |  | 1/2 |
| Information Bit Payload |  |  |
| For Sub-Frames 1,4,6,9 | Bits | 11448 |
| For Sub-Frames 2,3,7,8 | Bits | 11448 |
| For Sub-Frame 5 | Bits | N/A |
| For Sub-Frame 0 | Bits | 9528 |
| Number of Code Blocks (Note 4) |  |  |
| For Sub-Frames 1,4,6,9 | Code blocks | 2 |
| For Sub-Frames 2,3,7,8 | Code blocks | 2 |
| For Sub-Frame 5 | Bits | N/A |
| For Sub-Frame 0 | Bits | 2 |
| Binary Channel Bits |  |  |
| For Sub-Frames 1,4,6,9 | Bits | 24000 |
| For Sub-Frames 2,7 |  | 23600 |
| For Sub-Frames 3,8 |  | 23200 |
| For Sub-Frame 5 | Bits | N/A |
| For Sub-Frame 0 | Bits | 19680 |
| Max. Throughput averaged over 1 frame | Mbps | 10.1112 |
| UE Category |  | ≥ 2 |
| Note 1: 2 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: 50 resource blocks are allocated in sub-frames 1, 2, 3, 4, 6, 7, 8, 9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |

The reference measurement channels in Table A3.3.3.1-2 apply for verifying demudlation performance for UE-specific reference symbols with two cell specific antenna ports and two CSI-RS antenna ports with ZP CSI-RS and NZP CSI-RS in same subframe.

Table A.3.3.3.1-2: Fixed Reference Channel for CDM-multiplexed DM RS with two CSI-RS antenna ports with ZP CSI-RS and NZP CSI-RS

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.52 FDD | R.52-1 FDD | R.53 FDD | R.54 FDD |  |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 |  |
| Allocated resource blocks |  | 50 (Note 3) | 50 (Note 3) | 50 (Note 3) | 50 (Note 3) |  |
| Allocated subframes per Radio Frame |  | 9 | 9 | 9 | 9 |  |
| Modulation |  | 64QAM | 16QAM | 64QAM | 16QAM |  |
| Target Coding Rate |  | 1/2 | 2/3 | 1/2 | 1/2 |  |
| Information Bit Payload |  |  |  |  |  |  |
| For Sub-Frames 1,3,4,6,8,9 | Bits | 18336 | 15264 | 18336 | 11448 |  |
| For Sub-Frames 2,7 | Bits | 16416 | 14112 | 16416 | 11448 |  |
| For Sub-Frame 5 | Bits | n/a | n/a | n/a | n/a |  |
| For Sub-Frame 0 | Bits | 14688 | 12576 | 14688 | 9528 |  |
| Number of Code Blocks (Note 4) |  |  |  |  |  |  |
| For Sub-Frames 1,3,4,6,8,9 | Code blocks | 3 | 3 | 3 | 2 |  |
| For Sub-Frames 2, 7 | Code blocks | 3 | 3 | 3 | 2 |  |
| For Sub-Frame 5 | Bits | n/a | n/a | n/a | n/a |  |
| For Sub-Frame 0 | Bits | 3 | 3 | 3 | 2 |  |
| Binary Channel Bits |  |  |  |  |  |  |
| For Sub-Frames 1,3,4,6,8,9 | Bits | 36000 | 24000 | 36000 | 24000 |  |
| For Sub-Frames 2,7 |  | 34200 | 22800 | 33600 | 22800 |  |
| For Sub-Frame 5 | Bits | n/a | n/a | n/a | n/a |  |
| For Sub-Frame 0 | Bits | 29520 | 19680 | 29520 | 19680 |  |
| Max. Throughput averaged over 1 frame | Mbps | 15.7536 | 13.2384 | 15.7536 | 10.1112 |  |
| Note 1: 2 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: 50 resource blocks are allocated in sub-frames 1, 2, 3, 4, 6, 7, 8, 9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

#### A.3.4.3.3	Two antenna ports (CSI-RS)

The reference measurement channels in Table A.3.4.3.3-1 apply for verifying demodulation performance for CDM-multiplexed UE specific reference symbols with two cell-specific antenna ports and two CSI-RS antenna ports.

Table A.3.4.3.3-1: Fixed Reference Channel for CDM-multiplexed DM RS with two CSI-RS antenna ports

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | R.51 TDD |
| Channel bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 (Note 5) |
| Uplink-Downlink Configuration (Note 3) |  | 1 |
| Allocated subframes per Radio Frame (D+S) |  | 3+2 |
| Modulation |  | 16QAM |
| Target Coding Rate |  | 1/2 |
| Information Bit Payload |  |  |
| For Sub-Frames 4,9 (non CSI-RS subframe) | Bits | 11448 |
| For Sub-Frame 4,9 | Bits | 11448 |
| For Sub-Frames 1,6 | Bits | 7736 |
| For Sub-Frame 5 | Bits | N/A |
| For Sub-Frame 0 | Bits | 9528 |
| Number of Code Blocks (Note 4) |  |  |
| For Sub-Frames 4, 9 (non CSI-RS subframe) | Code blocks | 2 |
| For Sub-Frames 4,9 | Code blocks | 2 |
| For Sub-Frames 1,6 | Code blocks | 2 |
| For Sub-Frame 5 |  | N/A |
| For Sub-Frame 0 | Code blocks | 2 |
| Binary Channel Bits |  |  |
| For Sub-Frames 4, 9 (non CSI-RS subframe) | Bits | 24000 |
| For Sub-Frames 4,9 |  | 22800 |
| For Sub-Frames 1,6 |  | 15744 |
| For Sub-Frame 5 | Bits | N/A |
| For Sub-Frame 0 | Bits | 19680 |
| Max. Throughput averaged over 1 frame | Mbps | 4.7896 |
| UE Category |  | ≥ 2 |
| Note 1: 2 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: as per Table 4.2-2 in TS 36.211 [4]. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: 50 resource blocks are allocated in sub-frames 4,9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0 and the DwPTS portion of sub-frames 1,6. |  |  |

The reference measurement channels in Table A3.4.3.3-2 apply for verifying demudlation performance for UE-specific reference symbols with two cell specific antenna ports and two CSI-RS antenna ports with ZP CSI-RS and NZP CSI-RS in same subframe.

Table A.3.4.3.3-2: Fixed Reference Channel for CDM-multiplexed DM RS with two CSI-RS antenna ports with ZP CSI-RS and NZP CSI-RS

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.52 TDD | R.52-1 TDD | R.53 TDD | R.54 TDD |  |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 |  |
| Allocated resource blocks |  | 50 (Note 5) | 50 (Note 5) | 50 (Note 5) | 50 (Note 5) |  |
| Uplink-Downlink Configuration (Note 3) |  | 1 | 1 | 1 | 1 |  |
| Allocated subframes per Radio Frame (D+S) |  | 3+2 | 3+2 | 3+2 | 3+2 |  |
| Modulation |  | 64QAM | 16QAM | 64QAM | 16QAM |  |
| Target Coding Rate |  | 1/2 | 2/3 | 1/2 | 1/2 |  |
| Information Bit Payload |  |  |  |  |  |  |
| For Sub-Frame 4,9 | Bits | 16416 | 15264 | 16416 | 11448 |  |
| For Sub-Frames 1,6 | Bits | 11832 | 10680 | 11832 | 7736 |  |
| For Sub-Frame 5 | Bits | n/a | n/a | n/a | n/a |  |
| For Sub-Frame 0 | Bits | 14688 | 13536 | 14688 | 9528 |  |
| Number of Code Blocks (Note 4) |  |  |  |  |  |  |
| For Sub-Frames 4,9 | Code blocks | 3 | 3 | 3 | 2 |  |
| For Sub-Frames 1,6 | Code blocks | 2 | 2 | 2 | 2 |  |
| For Sub-Frame 5 |  | n/a | n/a | n/a | n/a |  |
| For Sub-Frame 0 | Code blocks | 3 | 3 | 3 | 2 |  |
| Binary Channel Bits |  |  |  |  |  |  |
| For Sub-Frames 4,9 |  | 34200 | 22800 | 33600 | 22800 |  |
| For Sub-Frames 1,6 |  | 23616 | 15744 | 23616 | 15744 |  |
| For Sub-Frame 5 | Bits | n/a | n/a | n/a | n/a |  |
| For Sub-Frame 0 | Bits | 29520 | 19680 | 29520 | 19680 |  |
| Max. Throughput averaged over 1 frame | Mbps | 7.1184 | 6.5424 | 7.1184 | 4.7896 |  |
| UE Category |  | ≥ 2 | ≥2 | ≥ 2 | ≥ 2 |  |
| Note 1: 2 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: as per Table 4.2-2 in TS 36.211 [4]. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: 50 resource blocks are allocated in sub-frames 4, 9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0 and the DwPTS portion of sub-frames 1, 6. |  |  |  |  |  |

-----------------------------------------------------------------End of Change ------------------------------------------------------------