3GPP TSG-RAN4 Meeting #78 	R4-161500

# Malta, 15th-19th February 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 3430 | rev | 2 | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections and bracket removals to B46 specifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA |  |  |  |  |  |  | Date: |  | 2016-02-19 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | ACS case2 interferer power is incorrect in table 7.5.1A-5. In-band blocker level is incorrect in table 7.6.1.1A-4. Out-of-band blocking for CA_42A_46A is incorrect in table 7.6.2.1A-0a. Several B46 specification numbers are still in brackets.Harmonic exceptions in CA REFSENS are not included in current specification. Usage of shared antenna in CA_42A_46A is very challenging with the current zero RX relaxation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | ACS case2 blocker numerical component is changed from -43.4 to -44.5 in table 7.5.1A-6. In-band blocker 1 is changed from -56 to -50 in table 7.6.1.1A-4. The relaxed interferer power for Out-of-band blocking range3 is changed in NOTE 5 in table 7.6.2.1A-0a. NOTE5 was edited when LTE-U CR was approved in previous meeting and likely there was a typing error as the current NOTE5 would tighten the Out-of-band blocking range 3 for B42 in CA with unlicensed carrier. All brackets from B46 specifications are removed. A note is added to indicate REFSENS is not defined are frequencies where harmonic overlap occurs. dRib for B42 is changed to [0] in B42+B46 CA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Erroneous and unfinished specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1, 7.3.1A, 7.5.1A, 7.6.1.1A, 7.6.2.1A, 7.8.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

##### ---Start of changes---

## 7.3	Reference sensitivity power level

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories except category 0, or to the single antenna port for UE category 0, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

### 7.3.1	Minimum requirements (QPSK)

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.1-1 and Table 7.3.1-2

Table 7.3.1-1: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
| 2 | -102.7 | -99.7 | -98 | -95 | -93.2 | -92 | FDD |
| 3 | -101.7 | -98.7 | -97 | -94 | -92.2 | -91 | FDD |
| 4 | -104.7 | -101.7 | -100 | -97 | -95.2 | -94 | FDD |
| 5 | -103.2 | -100.2 | -98 | -95 |  |  | FDD |
| 6 |  |  | -100 | -97 |  |  | FDD |
| 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
| 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
| 9 |  |  | -99 | -96 | -94.2 | -93 | FDD |
| 10 |  |  | -100 | -97 | -95.2 | -94 | FDD |
| 11 |  |  | -100 | -97 |  |  | FDD |
| 12 | -101.7 | -98.7 | -97 | -94 |  |  | FDD |
| 13 |  |  | -97 | -94 |  |  | FDD |
| 14 |  |  | -97 | -94 |  |  | FDD |
| … |  |  |  |  |  |  |  |
| 17 |  |  | -97 | -94 |  |  | FDD |
| 18 |  |  | -1007 | -977 | -95.27 |  | FDD |
| 19 |  |  | -100 | -97 | -95.2 |  | FDD |
| 20 |  |  | -97 | -94 | -91.2 | -90 | FDD |
| 21 |  |  | -100 | -97 | -95.2 |  | FDD |
| 22 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| 23 | -104.7 | -101.7 | -100 | -97 | -95.2 | -94 | FDD |
| 24 |  |  | -100 | -97 |  |  | FDD |
| 25 | -101.2 | -98.2 | -96.5 | -93.5 | -91.7 | -90.5 | FDD |
| 26 | -102.7 | -99.7 | -97.56 | -94.56 | -92.76 |  | FDD |
| 27 | -103.2 | -100.2 | -98 | -95 |  |  | FDD |
| 28 |  | -100.2 | -98.5 | -95.5 | -93.7 | -91 | FDD |
| 30 |  |  | -99 | -96 |  |  | FDD |
| 31 | -99.0 | -95.7 | -93.5 |  |  |  | FDD |
| … |  |  |  |  |  |  |  |
| 33 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 34 |  |  | -100 | -97 | -95.2 |  | TDD |
| 35 | -106.2 | -102.2 | -100 | -97 | -95.2 | -94 | TDD |
| 36 | -106.2 | -102.2 | -100 | -97 | -95.2 | -94 | TDD |
| 37 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 38 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 41 |  |  | -98 | -95 | -93.2 | -92 | TDD |
| 42 |  |  | -99 | -96 | -94.2 | -93 | TDD |
| 43 |  |  | -99 | -96 | -94.2 | -93 | TDD |
| 44 |  | [-100.2] | [-98] | [-95] | [-93.2] | [-92] | TDD |
| 45 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| ... |  |  |  |  |  |  |  |
| 65 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: For the UE which supports both Band 3 and Band 9 the reference sensitivity level is FFS. NOTE 5: For the UE which supports both Band 11 and Band 21 the reference sensitivity level is FFS. NOTE 6: 6 indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 7: For a UE that support both Band 18 and Band 26, the reference sensitivity level for Band 26 applies for the applicable channel bandwidths. |  |  |  |  |  |  |  |

For UE(s) equipped with 4 antenna ports the following additional requirements apply. The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.1-1a and Table 7.3.1-2.

Table 7.3.1-1a: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 2 | [-105.4] | [-102.4] | [-100.7] | [-97.7] | [-95.9] | [-94.7] | FDD |
| 3 | [-104.4] | [-101.4] | [-99.7] | [-96.7] | [-94.9] | [-93.7] |  |
| … |  |  |  |  |  |  |  |
| 7 |  |  | [-100.7] | [-97.7] | [-95.9] | [-94.7] | FDD |
| … |  |  |  |  |  |  |  |
| 20 |  |  | [-99.7] | [-96.7] | [-93.9] | [-92.7] | FDD |
| … |  |  |  |  |  |  |  |
| 39 |  |  | [-102.7] | [-99.7] | [-97.9] | [-96.7] | TDD |
| … |  |  |  |  |  |  |  |
| 42 |  |  | [-101.2] | [-98.2] | [-96.4] | [-95.2] | TDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port. |  |  |  |  |  |  |  |

The reference receive sensitivity (REFSENS) requirement specified in Table 7.3.1-1 (two antenna ports) and Table 7.3.1-1a (four antenna ports) shall be met for an uplink transmission bandwidth less than or equal to that specified in Table 7.3.1-2.

NOTE:	Table 7.3.1-2 is intended for conformance tests and does not necessarily reflect the operational conditions of the network, where the number of uplink and downlink allocated resource blocks will be practically constrained by other factors. Typical receiver sensitivity performance with HARQ retransmission enabled and using a residual BLER metric relevant for e.g. Speech Services is given in the Annex G (informative).For the UE which supports inter-band carrier aggregation configuration with the uplink in one or two E-UTRA bands, the minimum requirement for reference sensitivity in Table 7.3.1-1 and Table 7.3.1-1a shall be increased by the amount given in ΔRIB,c in Table 7.3.1-1A, Table 7.3.1-1B and Table 7.3.1-1C for the applicable E-UTRA bands.

Table 7.3.1-1A: ΔRIB,c (two bands)

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A | 1 | 0 |
|  | 3 | 0 |
| CA_1A-3C | 1 | 0 |
|  | 3 | 0 |
| CA_1A-5A | 1 | 0 |
|  | 5 | 0 |
| CA_1A-7A | 1 | 0 |
|  | 7 | 0 |
| CA_1A-8A | 1 | 0 |
|  | 8 | 0 |
| CA_1A-11A | 1 | 0 |
|  | 11 | 0 |
| CA_1A-18A | 1 | 0 |
|  | 18 | 0 |
| CA_1A-19A | 1 | 0 |
|  | 19 | 0 |
| CA_1A-20A | 1 | 0 |
|  | 20 | 0 |
| CA_1A-21A | 1 | 0 |
|  | 21 | 0 |
| CA_1A-26A | 1 | 0 |
|  | 26 | 0 |
| CA_1A-28A | 1 | 0 |
|  | 28 | 0.2 |
| CA_1A-40A | 1 | 0 |
|  | 40 | 0 |
| CA_1A-41A8 | 1 | 0 |
|  | 41 | 0 |
| CA_1A-41C8 | 1 | 0 |
|  | 41 | 0 |
| CA_1A-42A | 1 | 0 |
|  | 42 | 0.5 |
| CA_1A-42C | 1 | 0 |
|  | 42 | 0.5 |
| CA_2A-4A | 2 | 0.3 |
|  | 4 | 0.3 |
| CA_2A-2A-4A | 2 | 0.3 |
|  | 4 | 0.3 |
| CA_2A-4A-4A | 2 | 0.3 |
|  | 4 | 0.3 |
| CA_2A-2A-4A-4A | 2 | 0.3 |
|  | 4 | 0.3 |
| CA_2A-5A | 2 | 0 |
|  | 5 | 0 |
| CA_2A-2A-5A | 2 | 0 |
|  | 5 | 0 |
| CA_2C-5A | 2 | 0 |
|  | 5 | 0 |
| CA_2A-12A | 2 | 0 |
|  | 12 | 0 |
| CA_2A-2A-12A | 2 | 0 |
|  | 12 | 0 |
| CA_2A-2A-12B | 2 | 0 |
|  | 12 | 0 |
| CA_2A-12B | 2 | 0 |
|  | 12 | 0 |
| CA_2C-12A | 2 | 0 |
|  | 12 | 0 |
| CA_2A-13A | 2 | 0 |
|  | 13 | 0 |
| CA_2A-2A-13A | 2 | 0 |
|  | 13 | 0 |
| CA_2A-17A | 2 | 0 |
|  | 17 | 0.5 |
| CA_2A_28A | 2 | 0 |
|  | 28 | 0 |
| CA_2A-30A | 2 | 0.4 |
|  | 30 | 0.5 |
| CA_2C-30A | 2 | 0.4 |
|  | 30 | 0.5 |
| CA_3A-5A | 3 | 0 |
|  | 5 | 0 |
| CA_3C-5A | 3 | 0 |
|  | 5 | 0 |
| CA_3A-7A | 3 | 0 |
|  | 7 | 0 |
| CA_3A-7B | 3 | 0 |
|  | 7 | 0 |
| CA_3A-7C | 3 | 0 |
|  | 7 | 0 |
| CA_3C-7A | 3 | 0 |
|  | 7 | 0 |
| CA_3A-8A | 3 | 0 |
|  | 8 | 0 |
| CA_3A-3A-8A | 3 | 0 |
|  | 8 | 0 |
| CA_3A-19A | 3 | 0 |
|  | 19 | 0 |
| CA_3A-20A | 3 | 0 |
|  | 20 | 0 |
| CA_3A-26A | 3 | 0 |
|  | 26 | 0 |
| CA_3A-27A | 3 | 0 |
|  | 27 | 0 |
| CA_3A-28A | 3 | 0 |
|  | 28 | 0 |
| CA_3A-31A | 3 | 0 |
|  | 31 | 0.2 |
| CA_3A-38A | 3 | 0 |
|  | 38 | 0 |
| CA_3A-40A | 3 | 0 |
|  | 40 | 0 |
| CA_3A-40C | 3 | 0 |
|  | 40 | 0 |
| CA_3A-41A | 3 | 0 |
|  | 41 | 010 |
|  |  | 0.511 |
| CA_3A-41C | 3 | 0 |
|  | 41 | 010 |
|  |  | 0.511 |
| CA_3A-42A | 3 | 0.2 |
|  | 42 | 0.5 |
| CA_3A-42C | 3 | 0.2 |
|  | 42 | 0.5 |
| CA_4A-5A | 4 | 0 |
|  | 5 | 0 |
| CA_4A-4A-5A | 4 | 0 |
|  | 5 | 0 |
| CA_4A-7A | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_4A-4A-7A | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_4A-12A | 4 | 0 |
|  | 12 | 0.5 |
| CA_4A-12B | 4 | 0 |
|  | 12 | 0.5 |
| CA_4A-4A-12A | 4 | 0 |
|  | 12 | 0.5 |
| CA_4A-13A | 4 | 0 |
|  | 13 | 0 |
| CA_4A-4A-13A | 4 | 0 |
|  | 13 | 0 |
| CA_4A-17A | 4 | 0 |
|  | 17 | 0.5 |
| CA_4A-27A | 4 | 0 |
|  | 27 | 0 |
| CA_4A-28A | 4 | 0 |
|  | 28 | 0.2 |
| CA_4A-30A | 4 | 0.4 |
|  | 30 | 0.5 |
| CA_5A-7A | 5 | 0 |
|  | 7 | 0 |
| CA_5A-12A | 5 | 0.5 |
|  | 12 | 0.3 |
| CA_5A-12B | 5 | 0.5 |
|  | 12 | 0.3 |
| CA_5A-13A | 5 | 0 |
|  | 13 | 0 |
| CA_5A-17A | 5 | 0.5 |
|  | 17 | 0.3 |
| CA_5A-25A | 5 | 0 |
|  | 25 | 0 |
| CA_5A-30A | 5 | 0 |
|  | 30 | 0 |
| CA_5A-38A | 5 | 0 |
|  | 38 | 0 |
| CA_5A-40A | 5 | 0 |
|  | 40 | 0 |
| CA_5A-40C | 5 | 0 |
|  | 40 | 0 |
| CA_7A-8A | 7 | 0 |
|  | 8 | 0.2 |
| CA_7A-12A | 7 | 0 |
|  | 12 | 0 |
| CA_7A-20A | 7 | 0 |
|  | 20 | 0 |
| CA_7A-22A | 7 | 0 |
|  | 22 | 0.5 |
| CA_7A-28A | 7 | 0 |
|  | 28 | 0 |
| CA_7B-28A | 7 | 0 |
|  | 28 | 0 |
| CA_7C-28A | 7 | 0 |
|  | 28 | 0 |
| CA_7A-40A | 7 | 0 |
|  | 40 | 0.5 |
| CA_7A-40C | 7 | 0 |
|  | 40 | 0.5 |
| CA_7A-42A | 7 | 0 |
|  | 42 | 0.5 |
| CA_7A-42A-42A | 7 | 0 |
|  | 42 | 0.5 |
| CA_8A-11A | 8 | 0 |
|  | 11 | 0 |
| CA_8A-20A | 8 | 0 |
|  | 20 | 0 |
| CA_8A-40A | 8 | 0 |
|  | 40 | 0 |
| CA_8A-41A | 8 | 0 |
|  | 41 | 0 |
| CA_8A-41C | 8 | 0 |
|  | 41 | 0 |
| CA_11A-18A | 11 | 0 |
|  | 18 | 0 |
| CA_12A-25A | 12 | 0 |
|  | 25 | 0 |
| CA_12A-30A | 12 | 0 |
|  | 30 | 0 |
| CA_18A-28A9 | 18 | 0 |
|  | 28 | 0 |
| CA_19A-21A | 19 | 0 |
|  | 21 | 0 |
| CA_19A-28A9 | 19 | 0 |
|  | 28 | 0 |
| CA_19A-42A | 19 | 0 |
|  | 42 | 0.5 |
| CA_19A-42C | 19 | 0 |
|  | 42 | 0.5 |
| CA_20A-31A | 20 | 0 |
|  | 31 | 0 |
| CA_20A-38A | 20 | 0 |
|  | 38 | 0 |
| CA_20A-40A | 20 | 0 |
|  | 40 | 0 |
| CA_20A-42A | 20 | 0 |
|  | 42 | 0.5 |
| CA_20A-42A-42A | 20 | 0 |
|  | 42 | 0.5 |
| CA_21A-42A | 21 | 0.2 |
|  | 42 | 0.5 |
| CA_21A-42C | 21 | 0.2 |
|  | 42 | 0.5 |
| CA_25A-26A | 25 | 0 |
|  | 26 | 0 |
| CA_25A-41A8 | 25 | 0 |
|  | 41 | 0 |
| CA_25A-41C8 | 25 | 0 |
|  | 41 | 0 |
| CA_25A-41D8 | 25 | 0 |
|  | 41 | 0 |
| CA_26A-41A | 26 | 0 |
|  | 41 | 0 |
| CA_26A-41C | 26 | 0 |
|  | 41 | 0 |
| CA_28A-40A | 28 | 0 |
|  | 40 | 0 |
| CA_28A-40C | 28 | 0 |
|  | 40 | 0 |
| CA_28A-40D | 28 | 0 |
|  | 40 | 0 |
| CA_28A-41A | 28 | 0 |
|  | 41 | 0 |
| CA_28A-41C | 28 | 0 |
|  | 41 | 0 |
| CA_28A-42A | 28 | 0.2 |
|  | 42 | 0.5 |
| CA_28A-42C | 28 | 0.2 |
|  | 42 | 0.5 |
| CA_38A-40A | 38 | 0.54 |
|  | 40 | 0.54 |
| CA_38A-40A-40A | 38 | 0.54 |
|  | 40 | 0.54 |
| CA_38A-40C | 38 | 0.54 |
|  | 40 | 0.54 |
| CA_39A-41A | 39 | 0.24 |
|  | 41 | 0.24 |
| CA_39A-41A | 39 | 0.27 |
|  | 41 | 0.27 |
| CA_39A-41C | 39 | 0.24 |
|  | 41 | 0.24 |
| CA_39A-41C | 39 | 0.27 |
|  | 41 | 0.27 |
| CA_39A-41D | 39 | 0.24 |
|  | 41 | 0.24 |
| CA_39C-41A | 39 | 0.24 |
|  | 41 | 0.24 |
| CA_39C-41A | 39 | 0.27 |
|  | 41 | 0.27 |
| CA_39C-41C | 39 | 0.24 |
|  | 41 | 0.24 |
| CA_41A-42A | 41 | 0.44 |
|  | 42 | 0.54 |
| CA_41A-42C | 41 | 0.44 |
|  | 42 | 0.54 |
| CA_41C-42A | 41 | 0.44 |
|  | 42 | 0.54 |
| CA_41C-42C | 41 | 0.44 |
|  | 42 | 0.54 |
| CA_42A-46A | 42 | [0] |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 2: The above additional tolerances also apply in intra-band and non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 3: In case the UE supports more than one of the above 2DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 2DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the 2DL tolerances in Table 7.3.1-1A, truncated to one decimal place that would apply for that operating band among the supported 2DL CA configurations. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 2DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional tolerance shall be the maximum 2DL tolerance in Table 7.3.1-1A that would apply for that operating band among the supported 2DL CA configurations NOTE 4: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 5: Unless otherwise specified, in case the UE supports more than one of the above 3DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 3DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz and the tolerances are the same, the value applies to the band. If the tolerances are different, the applicable additional 3DL tolerance is FFS. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 3DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional 3DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 3DL CA configurations. NOTE 6: The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration. NOTE 7: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 8: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in the FDD band. NOTE 9: For Band 28, the requirements only apply for the restricted frequency range specified for this CA configuration (Table 5.5A-2). NOTE 10: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 11: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

<Text omitted>

### 7.3.1A	Minimum requirements (QPSK) for CA

<Text omitted>

For band combinations including operating band 46 (Table 5.5-1), the requirements are specified in Table 7.3.1A-0eA and Table 7.3.1A-0eB.

Table 7.3.1A-0eA: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-46A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_2A-46A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_3A-46A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_4A-46A | 4 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_7A-46A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_41A-46A | 41 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_42A-46A | 42 |  |  | -99 | -96 | -94.2 | -93 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is TBD NOTE 3: The signal power is specified per port NOTE 4: The requirements do not apply when there is at least one individual RE within the uplink transmission bandwidth of the lower band for which the 2nd/3rd/4th transmitter harmonic is within the downlink transmission bandwidth of the higher band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = FFS MHz for the configuations listed. |  |  |  |  |  |  |  |  |

<Text omitted>

### 7.5.1A	Minimum requirements for CA

Table 7.5.1A-1: Adjacent channel selectivity

|  |  | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | B | C | D | E | F |
| ACS | dB | 27 | 24 | 22.2 | 21 |  |

Table 7.5.1A-2: Test parameters for Adjacent channel selectivity, Case 1

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC |  | REFSENS + 14 dB | REFSENS + 14 dB | REFSENS + 14 dB | REFSENS + 14 dB |  |
| PInterferer | dBm | Aggregated power + 25.5 dB | Aggregated power + 22.5 dB | Aggregated power + 20.7 dB | Aggregated power + 19.5 dB |  |
| BWInterferer | MHz | 5 | 5 | 5 | 5 |  |
| FInterferer (offset) | MHz | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 NOTE 3: The Finterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the adjacent channel interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |  |  |

Table 7.5.1A-3: Test parameters for Adjacent channel selectivity, Case 2

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -50.5 +10log10(NRB,c/ NRB agg) | -47.5 +10log10(NRB,c/NRB agg) | -43.9 +10log10(NRB,c/NRB agg) | -44.5 +10log10(NRB,c/NRB agg) |  |
| PInterferer | dBm | -25 |  |  |  |  |
| BWInterferer | MHz | 5 | 5 | 5 | 5 |  |
| FInterferer (offset) | MHz | 2.5+ Foffset / -2.5- Foffset | 2.5+ Foffset / -2.5- Foffset | 2.5+ Foffset / -2.5- Foffset | 2.5+ Foffset / -2.5- Foffset |  |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex 3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 NOTE 3: The Finterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the adjacent channel interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |  |  |

For combinations of intra-band and inter-band carrier aggregation with up to five downlink carriers (up to two non-contiguously aggregated carriers per band and up to four contiguously aggregated carriers per band) and one uplink carrier assigned to one E-UTRA band, the requirement is defined with the uplink active in each band other than that supporting the downlink(s) under test. The uplink configuration shall be in accordance with Table 7.3.1A-3 when the uplink is active in the band supporting two non-contiguous component carriers, Table 7.3.1A-1 when the uplink is active in a band supporting two contiguous component carriers and in accordance with Table 7.3.1-2 when the uplink is active in a band supporting one carrier per band. The downlink PCC shall be configured closer to the uplink operating band than the downlink SCC(s) when the uplink is active in band(s) supporting contiguous aggregation of up to three component carriers. For these uplink configurations, the UE shall meet the adjacent channel selectivity requirements for intra-band non-contiguous carrier aggregation of two downlink carriers with RIBNC = 0 dB for all sub-block gaps (Table 7.3.1A-3) for the two non-contiguous downlink carriers, the requirements for intra-band contiguous carrier aggregation for the contiguously aggregated downlink carriers and for any remaining component carrier(s) the requirements specified in subclause 7.5.1. For contiguously aggregated component carriers configured in Band 46, the said requirements for intra-band contiguous carrier aggregation of downlink carriers are replaced by requirements in Table 7.5.1A-4 with test parameters in Table 7.5.1A-5 and Table 7.5.1A-6. All downlink carriers shall be active throughout the tests and the requirements for downlinks shall be met with the single uplink carrier active in each band capable of UL operation.

Table 7.5.1A-4: Adjacent channel selectivity

| E-UTRA band | Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | C | D | E | F |
| 46 | ACS | dB |  | 24 | 22.2 | 21 |  |

Table 7.5.1A-5: Test parameters for Adjacent channel selectivity, Case 1

| E-UTRA Band | Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | C | D | E | F |
| 46 | Pw in Transmission Bandwidth Configuration, per CC |  |  | REFSENS + 14 dB | REFSENS + 14 dB | REFSENS + 14 dB |  |
|  | PInterferer | dBm |  | Aggregated power + 22.5 dB | Aggregated power + 20.7 dB | Aggregated power + 19.5 dB |  |
|  | BWInterferer | MHz |  | 20 | 20 | 20 |  |
|  | FInterferer (offset) | MHz |  | 10 + Foffset / -10 - Foffset | 10 + Foffset / -10 - Foffset | 10 + Foffset / -10 - Foffset |  |
| NOTE 1*: In a band capable of uplink operation, the transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 (TBD) NOTE 3: The Finterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the adjacent channel interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |  |  |  |

Table 7.5.1A-6: Test parameters for Adjacent channel selectivity, Case 2

| E-UTRA band | Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | C | D | E | F |
| 46 | Pw in Transmission Bandwidth Configuration, per CC | dBm |  | -47.5 +10log10(NRB,c/NRB agg) | -45.7 +10log10(NRB,c/NRB agg) | -44.5 +10log10(NRB,c/NRB agg) |  |
|  | PInterferer | dBm | -25 |  |  |  |  |
|  | BWInterferer | MHz |  | 20 | 20 | 20 |  |
|  | FInterferer (offset) | MHz |  | 10 + Foffset / -10 - Foffset | 10 + Foffset / -10 - Foffset | 10 + Foffset / -10 - Foffset |  |
| NOTE 1: In a band capable of uplink operation, the transmitter shall be set to 24dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex 3.2 (TBD) NOTE 3: The Finterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the adjacent channel interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |  |  |  |

<Text omitted>

#### 7.6.1.1A	Minimum requirements for CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band the in-band blocking requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. The UE shall meet the requirements specified in subclause 7.6.1.1 for each component carrier while all downlink carriers are active. For the UE which supports inter band CA configuration in Table 7.3.1-1A, PInterferer power defined in Table 7.6.1.1-2 is increased by the amount given by ΔRIB,c in Table 7.3.1-1A. For E-UTRA CA configurations including an operating band without uplink operation or an operating band with an unpaired DL part (as noted in Table 5.5-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. The requirements for the component carrier configured in the operating band without uplink operation are specified in Table 7.6.1.1A-0, Table 7.6.1.1A-0a and Table 7.6.1.1A-0b.

Table 7.6.1.1A-0: In-band blocking for additional operating bands for carrier aggregation

| E-UTRA band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 29, 32, 67 | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies |  |  |  |  |

Table 7.6.1.1A-0a: In band blocking parameters for additional operating bands for carrier aggregation

| E-UTRA band | Rx parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 46 (NOTE 3) | Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 9 |
|  | BWInterferer | MHz |  |  |  |  |  | 20 |
|  | FIoffset, case 1 | MHz |  |  |  |  |  | 30+0.0125 |
|  | FIoffset, case 2 | MHz |  |  |  |  |  | 50+0.0075 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 NOTE 3: The interferer consists of the Reference measurement channel specified in Annex A.3.2 (TBD) |  |  |  |  |  |  |  |  |

Table 7.6.1.1A-0b: In-band blocking for additional operating bands for carrier aggregation

| E-UTRA band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -50 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 46 | FInterferer | MHz | (Note 2) | FDL_low – 60 to FDL_high + 60 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz or 60 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies |  |  |  |  |

For E-UTRA CA configurations listed in Table 7.3.1A-0a under conditions for which reference sensitivity for the operating band being tested is N/A, the in-band blocking requirements of subclause 7.6.1.1A do not apply.

For intra-band contiguous carrier aggregation the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. For FDD, the PCC shall be configured closest to the uplink band. All downlink carriers shall be active throughout the test. The uplink output power shall be set as specified in Table 7.6.1.1A-1 with the uplink configuration set according to Table 7.3.1A-1 for the applicable carrier aggregation configuration. For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.1-2. The UE shall fulfil the minimum requirement in presence of an interfering signal specified in Tables 7.6.1.1A-1 and Tables 7.6.1.1A-2 being on either side of the aggregated signal. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.6.1.1A-1 and 7.6.1.1A-2. For operating bands with an unpaired DL part (as noted in Table 5.5-1), the requirements also apply for an SCC assigned in the unpaired part with parameters specified in Tables 7.6.2.1A-1 and 7.6.2.1A-2.

For intra-band non-contiguous carrier aggregation with one uplink carrier and two downlink sub-blocks, each larger than or equal to 5 MHz, the in-band blocking requirements are defined with the uplink configuration in accordance with Table 7.3.1A-3. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in subclause 7.6.1.1 and in this subclause for one component carrier and two component carriers per sub-block, respectively. The requirements apply for in-gap and out-of-gap interferers while all downlink carriers are active.

Table 7.6.1.1A-1: In band blocking parameters

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  | 9 | 12 | 13.8 | 15 |  |
| BWInterferer | MHz | 5 | 5 | 5 | 5 |  |
| FIoffset, case 1 | MHz | 7.5 | 7.5 | 7.5 | 7.5 |  |
| FIoffset, case 2 | MHz | 12.5 | 12.5 | 12.5 | 12.5 |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |  |  |  |

Table 7.6.1.1A-2: In-band blocking

| CA configuration | Parameter |  | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- | --- |
|  | PInterferer |  | dBm | -56 | -44 |
|  | FInterferer (offset) |  | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |
| CA_1C, CA_2C, CA_3C, CA_5B, CA_7C, CA_8B, CA_12B, CA_23B, CA_27B, CA_38C, CA_39C, CA_40C, CA_41C, CA_40D, CA_41D, CA_42C, CA_42D, CA_42E, CA_66B, CA_66C | FInterferer (Range) |  | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -Foffset - FIoffset, case 1 and b. the carrier frequency +Foffset + FIoffset, case 1 NOTE 3: Foffset is the frequency offset from the center frequency of the CC being tested to the edge of aggregated channel bandwidth. NOTE 4: The Finterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |

For combinations of intra-band and inter-band carrier aggregation with up to four downlink carriers (up to two non-contiguously aggregated carriers per band and up to three contiguously aggregated carriers per band) and one uplink carrier assigned to one E-UTRA band, the requirement is defined with the uplink active in the band other than that supporting the downlink(s) under test. The uplink configuration shall be in accordance with Table 7.3.1A-3 when the uplink is active in the band supporting two non-contiguous component carriers, Table 7.3.1A-1 when the uplink is active in a band supporting two contiguous component carriers and in accordance with Table 7.3.1-2 when the uplink is active in a band supporting one carrier per band. The downlink PCC shall be configured closer to the uplink operating band than the downlink SCC(s) when the uplink is active in band(s) supporting contiguous aggregation of up to three component carriers. For these uplink configurations, the UE shall meet the in-band blocking requirements for intra-band non-contiguous carrier aggregation of two downlink carriers with RIBNC = 0 dB for all sub-block gaps (Table 7.3.1A-3) for the two non-contiguous downlink carriers, the requirements for intra-band contiguous carrier aggregation for the contiguously aggregated downlink carriers and for any remaining component carrier(s) the requirements specified in subclause 7.6.1. For contiguously aggregated component carriers configured in Band 46, the said requirements for intra-band contiguous carrier aggregation of downlink carriers are replaced by requirements in Table 7.6.1.1A-3 and 7.6.1.1A-4. All downlink carriers shall be active throughout the tests and the requirements for the downlinks shall be met with the single uplink carrier active in each band capable of uplink operation.

Table 7.6.1.1A-3: In band blocking parameters

| E-UTRA Band | Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | C | D | E | F |
| 46 | Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  |  |  | 12 | 13.8 | 15 |  |
|  | BWInterferer | MHz |  | 20 | 20 | 20 |  |
|  | FIoffset, case 1 | MHz |  | 30 | 30 | 30 |  |
|  | FIoffset, case 2 | MHz |  | 50 | 50 | 50 |  |
| NOTE 1: In a band capable of uplink operation, the transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |  |  |  |  |

Table 7.6.1.1A-4: In-band blocking

| E-UTRA Band | Parameter |  | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- | --- |
|  | PInterferer |  | dBm | -50 | -44 |
|  | FInterferer (offset) |  | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |
| 46 | FInterferer (Range) |  | MHz | (Note 2) | FDL_low – 60 to FDL_high + 60 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -Foffset - FIoffset, case 1 and b. the carrier frequency +Foffset + FIoffset, case 1 NOTE 3: Foffset is the frequency offset from the center frequency of the CC being tested to the edge of aggregated channel bandwidth. NOTE 4: The Finterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |

<Text omitted>

#### 7.6.2.1A	Minimum requirements for CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band, the out-of-band blocking requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. The throughput in the downlink measured shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.6.2.1-1 and 7.6.2.1A-0. For E-UTRA CA configurations including an operating band without uplink operation (as noted in Table 5.5-1), the requirements for all downlinks shall be met with the uplink active in the band(s) capable of UL operation. For the E-UTRA CA configurations listed in Table 7.6.2.1A-0a, the parameters specified in Table 7.6.2.1A-0 are replaced by those specified in Table 7.6.2.1A-0a. The UE shall meet these requirements for each component carrier while all downlink carriers are active.

For inter-band carrier aggregation with one component carrier per operating band and the uplink active in two E-UTRA bands, the out-of-band blocking requirements specified above shall be met with the transmitter power for the uplink set to 7 dB below PCMAX_L,c  for each serving cell c.

For E-UTRA CA configurations including an operating band without uplink band or an operating band with an unpaired DL part (as noted in Table 5.5-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. For E-UTRA CA configurations listed in Table 7.3.1A-0a under conditions for which reference sensitivity for the operating band being tested is N/A, the out-of-band blocking requirements of subclause 7.6.2.1A do not apply.

Table 7.6.2.1A-0: out-of-band blocking for inter-band carrier aggregation

| Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- |
| Pw | dBm | Table 7.6.2.1-1 for all component carriers |  |  |
| Pinterferer | dBm | -44 + RIB,c | -30 + RIB,c | -15 + RIB,c |
| Finterferer (CW) | MHz | -60 < f – FDL_Low(j) < -15 or 15 < f – FDL_High(j) < 60 | -85 < f – FDL_Low(j) ≤ -60 or 60 ≤ f – FDL_High(j) < 85 | 1 ≤ f ≤ FDL_Low(1) – 85 or FDL_High(j) + 85 ≤ f ≤ FDL_Low(j+1) – 85 with j < X or FDL_High(X) + 85 ≤ f ≤ 12750 |
| NOTE 1: FDL_Low(j) and FDL_High(j) denote the respective lower and upper frequency limits of the operating band containing carrier j, j = 1,…,X, with carriers numbered in increasing order of carrier frequency and X the number of component carriers in the band combination (X ≤ 4 for the present version of this specification). NOTE 2: For FDL_Low(j+1) – FDL_High(j) < 145 MHz and FInterferer in FDL_High(j) < f < FDL_Low(j+1) with j < X, FInterferer can be in both Range 1 and Range 2. Then the lower of the PInterferer applies. NOTE 3: For FDL_Low(j) – 15 MHz ≤ f ≤ FDL_High(j) + 15 MHz the appropriate adjacent channel selectivity and in-band blocking requirments in the respective subclauses 7.5.1A and 7.6.1.1A shall be applied for carrier j. NOTE 4: RIB,c according to Table 7.3.1-1A applies when serving cell c is measured. NOTE 5: For inter-band CA combinations containing Bands 42 or 43, the interferer with respect to Band 42 or Band 43 shall have power level (PInterferer) for Range 3 modified to -20 + RIB,c dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. NOTE 6: For inter-band CA combinations containing Bands 7 and 38 simultaneously, for FInterferer Bands 7 and 38 are considered as one single band as follows: FDL_Low = 2570 MHz and FDL_High = 2690 MHz. For Range 2, the following applies for FDL_Low: [-105] < f – FDL_Low ≤ -60 or 60 ≤ f – FDL_High < 85 . For Range 3 the following applies 1 ≤ f ≤ FDL_Low [-105] or FDL_High + 85 ≤ f ≤ 12750. |  |  |  |  |

Table 7.6.2.1A-0a: out-of-band blocking for inter-band carrier aggregation with one active uplink

| E-UTRA CA Configuration | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| CA_1A-46A, CA_2A-46A, CA_3A-46A, CA_4A-46A, CA_7A-46A, CA_41A-46A, CA_42A-46A | Pwanted | dBm | Table 7.6.2.1-1 for all component carriers |  |  |
|  | Pinterferer | dBm | -44 + RIB,c | -30 + RIB,c | -15 + RIB,c (NOTE 5) |
|  | Finterferer (CW) | MHz | -60 < f – FDL_Low(j) < -15 with j ≤ K or 15 < f – FDL_High(j) < 60 with j ≤ K | -85 < f – FDL_Low(j) ≤ -60 or 60 ≤ f – FDL_High(j) < 85 | 1 ≤ f ≤ FDL_Low(j) – 85 or FDL_High(j) + 85 ≤ f ≤ 12750 |
| NOTE 1: FDL_Low(j) and FDL_High(j), j = 1,…,K,…N, denote the respective lower and upper frequency limits of the (non-overlapping) operating bands of the CA configuration numbered in increasing order of frequency, with N the number of bands in the band combination and K the number of bands with FDL_High ≤ 3600 MHz (K = 1 and N = 2 in the present version of this specification). NOTE 2: For FDL_Low(j) – 15 MHz ≤ f ≤ FDL_High(j) + 15 MHz the appropriate adjacent channel selectivity and in-band blocking requirements in the respective subclauses 7.5.1A and 7.6.1.1A shall be applied for carrier j = 1. NOTE 3: For FDL_Low(N) – 60 MHz ≤ f ≤ FDL_High(N) + 60 MHz the appropriate adjacent channel selectivity and in-band blocking requirements in the respective subclauses 7.5.1A and 7.6.1.1A shall be applied for carrier N = 2. NOTE 4: RIB,c according to Table 7.3.1-1A applies when serving cell c is measured. NOTE 5: The power level (PInterferer) for Range 3 is modified to -20 dBm for FInterferer > 4400 MHz except for band combinations with Band 42 for which PInterferer for Range 3 is modified to -20 dBm for FInterferer > 2800 MHz. |  |  |  |  |  |

### <Text omitted>

### 7.8.1A	Minimum requirements for CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band the wide band intermodulation requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. The UE shall meet the requirements specified in subclause 7.8.1.1 for each component carrier while all downlink carriers are active. For E-UTRA CA configurations including an operating band without uplink band or an operating band with an unpaired DL part (as noted in Table 5.5-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. For a component carrier configured in Band 46, the requirements specified in subclause 7.8.1.1 are replaced by the requirements in Table 7.8.1-1A-0.

Table 7.8.1.1A-0: Wide band intermodulation

| E-UTRA band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz |  | 3 MHz |  | 5 MHz | 10 MHz |  | 15 MHz |  | 20 MHz |  |
| 46 | Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | 9 |  |
|  | PInterferer 1 (CW) | dBm | -46 |  |  |  |  |  |  |  |  |  |  |
|  | PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |  |  |  |  |  |  |
|  | BWInterferer 2 |  |  |  |  |  |  |  |  |  |  |  | 20 |
|  | FInterferer 1 (Offset) | MHz |  |  |  |  |  |  |  |  |  |  | -BW/2 – 30 / +BW/2 + 30 |
|  | FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 (TBD) |  |  |  |  |  |  |  |  |  |  |  |  |  |

For E-UTRA CA configurations listed in Table 7.3.1A-0a under conditions for which reference sensitivity for the operating band being tested is N/A, the wideband intermodulation requirements of subclause 7.8.1A do not apply.

For intra-band contiguous carrier aggegation the downlink SCC(s) shall be configured at nominal channel spacing to the PCC, For FDD, the PCC shall be configured closest to the uplink band. All downlink carriers shall be active throughout the test. The uplink output power shall be set as specified in Table 7.8.1A-1 with the uplink configuration set according to Table 7.3.1A-1 for the applicable carrier aggreagation configuration. For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.1-2. The UE shall fulfil the minimum requirement in presence of an interfering signal specified in Table 7.8.1A-1 being on either side of the aggregated signal. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.8.1A-1. For operating bands with an unpaired DL part (as noted in Table 5.5-1), the requirements also apply for an SCC assigned in the unpaired part with parameters specified in Tables 7.8.1A-1.

Table 7.8.1A-1: Wide band intermodulation

| Rx parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  | 9 | 12 | 13.8 | 15 |  |
| PInterferer 1 (CW) | dBm | -46 |  |  |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |
| BWInterferer 2 | MHz | 5 | 5 | 5 | 5 |  |
| FInterferer 1 (Offset) | MHz | –Foffset-7.5 / + Foffset+7.5 | –Foffset-7.5 / + Foffset+7.5 | –Foffset-7.5 / + Foffset+7.5 | –Foffset-7.5 / + Foffset+7.5 |  |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 with set-up according to Annex C.3.1. NOTE 4: The interfering modulated signal is 5MHz E-UTRA signal as described in Annex D for channel bandwidth 5MHz; NOTE 5: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |  |  |  |  |

For intra-band non-contiguous carrier aggregation with one uplink carrier and two downlink sub-blocks, the wide band intermodulation requirements are defined with the uplink configuration in accordance with Table 7.3.1A-3. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in subclauses 7.8.1.1 and in this subclause for one component carrier and two component carriers per sub-block, respectively. The requirements apply for out-of-gap interferers while all downlink carriers are active.

For combinations of intra-band and inter-band carrier aggregation with up to four downlink carriers (up to two non-contiguously aggregated carriers per band and up to three contiguously aggregated carriers per band) and one uplink carrier assigned to one E-UTRA band, the requirement is defined with the uplink active in a band other than that supporting the downlink(s) under test. The uplink configuration shall be in accordance with Table 7.3.1A-3 when the uplink is active in the band supporting two non-contiguous component carriers, Table 7.3.1A-1 when the uplink is active in a band supporting two contiguous component carriers and in accordance with Table 7.3.1-2 when the uplink is active in a band supporting one carrier per band. For these uplink configurations, the UE shall meet the wide-band intermodulation requirements for intra-band non-contiguous carrier aggregation of two downlink carriers with RIBNC = 0 dB for all sub-block gaps (Table 7.3.1A-3) for the two non-contiguous downlink carriers, the requirements for intra-band contiguous carrier aggregation for the contiguously aggregated downlink carriers and for any remaining component carrier(s) the requirements specified in subclause 7.8.1. For contiguously aggregated component carriers configured in Band 46, the said requirements for intra-band contiguous carrier aggregation of two downlink carriers are replaced by requirements in Table 7.8.1A-2. All downlink carriers shall be active throughout the tests and the requirements for the downlinks shall be met with the single uplink carrier active in each band capable of UL operation.

Table 7.8.1A-2: Wide band intermodulation

| E-UTRA Band | Rx parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | C | D | E | F |
| 46 | Power per CC in Aggregated Transmission Bandwidth Configuration | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  |  |  | 12 | 13.8 | 15 |  |
|  | PInterferer 1 (CW) | dBm | -46 |  |  |  |  |
|  | PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |
|  | BWInterferer 2 | MHz |  | 20 | 20 | 20 |  |
|  | FInterferer 1 (Offset) | MHz |  | –Foffset-30 / + Foffset+30 | –Foffset-30 / + Foffset+30 | –Foffset-30 / + Foffset+30 |  |
|  | FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 with set-up according to Annex C.3.1. NOTE 4: The interfering modulated signal is 20 MHz E-UTRA signal as described in Annex D interference setting 2; NOTE 5: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |  |  |  |  |  |

##### --- End of changes ---