3GPP TSG-RAN WG4 Meeting #80bis	R4-167942

# Ljubljana, Slovenia, 10th – 14th Oct 2016

Agenda item:	8.19.1

Source: 	Qualcomm Incorporated

Title: 	Remaining Issues in Region 1 Specific UE RF Requirements

Document for:	Approval

# Introduction

In this paper, we discuss the remaining RF requirements in Region 1 according to RAN4 agreements in WF [5].

# Discussion

Table 6.6.3.2-1-1: Requirements

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| … |  |  |  |  |  |  |  |
| 47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 38 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 38 |
|  | Frequency range | 5795 | - | 5815 | [-65] | 1 | 38 |
| NOTE 38: Applicable when NS_XX is configured by the pre-configured radio parameters. |  |  |  |  |  |  |  |

Spectral Emission Mask

Table 6.6.2.2.4-1: Additional requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5] | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0] | 100 kHz |

Spurious Emission

It’s agreed in RAN4 that a new NS_XX signal is defined to encapsulate the A-SEM and A-SE requirements that are specific to Region 1 [3].

The A-SE requirement in 5795-5815 MHz frequency range originated from the CEN DSRC coexistence requirement in [1].  However, we observe that those requirement are only temporally activated when the vehicle is in the proximity of tolling stations [2]. Moreover, the requirement is not a fixed EIPR limit, but a combination of location, EIPR limit, transmit power, and on/off radio time depending on different coexistence modes as shown in Table 5.3 in [2].

Thus, it does not fit into NS signaling framework that is only suitable to signal permanent requirements.  A more appropriate signaling framework is proposed and discussed in detail in our companion contribution [4].

Observation 1: CEN DSRC coexistence requirement in ETSI harmonize standards is only applied when a vehicle is near a tolling station, or even do not apply if some condition is met. When the requirement applies, it is more than just an A-SE requirement. This is more dynamic than what NS signaling framework can handle.

Proposal 1: Do not capture the A-SE requirement in 5795-5815 frequency range under the scope of NS-XX. An additional method (dynamic signaling) is required to indicate when the A-SE requirements apply.

Another issue in Region 1 is that the Spurious Emission range is extended to right next to the band edge. While it is necessary to meet the out of band emission requirement in ETSI harmonized standard for the 5915-5955 MHz frequency range, it is not the case for the frequency range 5925-5950 MHz.

#### 4.2.5.2	Transmitter spectrum mask within the 5 GHz ITS frequency band for 10 MHz channel

(Texts removed)

It has to be noted that the spectrum mask is only applicable within 5 855 MHz to 5 950 MHz. Below 5 855 MHz and above 5 950 MHz the requirements in Clause 4.2.5.1 apply.

In particular, the clause 4.2.5.2 in [1] states that:

This means that for frequency range 5925-5950 MHz, the A-SEM applies when if the 10MHz channel within 25 MHz from this range. On the other hand, if the channel is at the lowest range of band 47, the requirement in Table 6.13-1 applies right next to the channel bandwidth.

Based on the discussion above, we revise the containing of the two tables cited above to:

Spurious Emission

Table 6.6.3.2-1: Requirements

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| … |  |  |  |  |  |  |  |
| 47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 38 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 38 |
| NOTE 38: Applicable when NS_XX is configured by the pre-configured radio parameters. |  |  |  |  |  |  |  |

Spectral Emission Mask

Table 6.6.2.2.4-1: Additional requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5] | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0] | 100 kHz |
| NOTE 1: The requirement in this table does not apply when the absolute frequency fc + ΔfOOB < 5855 MHz. In this case the requirement in clause 6.6.3.2 apply. |  |  |

Proposal 2: RAN4 adopt the modification Table 6.6.3.2-1 in [3].

Proposal 3: RAN4 adopt the modification of Table 6.6.2.2.4-1 in [3].

Figure 1: Combined Emisson Mask for ITS Channels in Region 1.

In Figure 1, we plot the Combined Emission Mask (i.e. combining SEM and SE following the highlighted rule in clauses 4.2.5.2 in [1]) for each of the 7 ITS 10 MHz channels. We can see that the requirement is stricter for the 3 lower channels. It is then reasonable to define A-MPR separately for each channel. For each channel, different A-MPR values may also be defined for the two cases where SA and Data is adjacent or nonadjacent infrequency. This is dependence on the simulation results.

Proposa4: A-MPR is defined separately for each ITS channel.

Observation2: A-MPR for the case SA and Data adjacent in frequency may be different from the case SA and Data nonadjacent in frequency.

# Conclusions

Observation 1: CEN DSRC coexistence requirement in ETSI harmonize standards is only applied when a vehicle is near a tolling station, or even do not apply if some condition is met. When the requirement applies, it is more than just a SEM requirement. This is more dynamic than what NS signaling framework can handle.

Proposal 1: Do not capture the SEM requirement in 5795-5815 frequency range under the scope of NS-XX.

Proposal 2: RAN4 adopt the modification of Table 6.13-1 in [3] and change the corresponding values in Table 6.6.3.2-1 in [4] accordingly.

Proposal 3: RAN4 adopt the modification of Table 6.6.2.2.4-1 in [4].

Proposa4: A-MPR is defined separately for each ITS channel.

Observation2: A-MPR for the case SA and Data adjacent in frequency may be different from the case SA and Data nonadjacent in frequency.

# References

- ETSI EN 302 571

- ETSI TS 102 791

[3] TS 36.101

[4] R4-167947, Discussion on Coexistence with CEN DSRC Requirements in Region 1, Qualcomm Incorporated

[5] R4-167151, WF on V2X UE RF requirements, Qualcomm, Erricson