3GPP TSG RAN WG4 Meeting #84Bis	R4-1710108

Dubrovnik, Croatia, 09 - 13 October, 2017

3GPP TSG RAN WG1 Meeting #90	              				         R1-1715188

Prague, Czech Republic, 21th – 25th August 2017

Title:	LS on RAN1 recommendation to define test cases

Response to:	-

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN1

To:	RAN4

Cc:	-

Contact Person:

#### Name:	Ralf Bendlin

E-mail Address:	ralf.bendlin@att.com

1. Overall Description:

At the 3GPP TSG RAN WG1 Meeting #90 in Prague, Czech Republic, the following was agreed:

Agreements: - Remove the support for 7-symbol slots from NR - It is allowed to have more than one DL/UL switching points within a 14-symbol slot by using non-slot-based scheduling - Note: at least 14-symbol, 7-symbol, and 2-symbol CORESET monitoring periodicities are supported for non-slot-based scheduling - Removing 7-symbol slot does not imply to remove the agreed design of 4- to 7-symbol long PUCCH - Allow additional DMRS position with non-slot based scheduling - RAN1 recommends to define test cases for following cases: - Slot-based scheduling for downlink - The first DMRS position of the PDSCH is fixed on the 3rd or 4th symbol of the slot - Non-slot-based scheduling for downlink - The first DMRS position of the PDSCH is the 1st symbol of the scheduled data - At least PDSCH durations of 2, 4, and 7 OFDM symbols including DMRS are recommended to be specified - Note: the LS includes the motivations of selected values - Note: Final decision is up to RAN4

The PDSCH durations of 2, 4, and 7 OFDM symbols (including DMRS) are motivated by

- Support of PDSCH durations that align with SS block transmissions with the same or different numerology in order to allow FDM of SS block transmissions and PDSCH transmissions , especially if TxRP uses beam-sweeping (e.g., above 6GHz)

  - This use case includes unicast and broadcast PDSCH

  - This use case includes FDM across CC in case of intra-band CA where the same RF hardware is used for more than one CC

- Support of finer TDM granularity of scheduling for the same/different UEs within a slot, especially if TxRP uses beam-sweeping (e.g., above 6GHz)

  - This use case includes unicast and broadcast PDSCH (e.g., for RMSI)

- Support of very low latency

- Support of NR transmissions scheduled in LTE non-MBSFN subframes

  - PDSCH can be scheduled on OFDM symbols not carrying CRS

- Support of forward compatibility, e.g., multiplexing of Rel. 15 waveforms with future services, e.g., integrated access and backhaul

2. Actions:

To RAN4 WG:

RAN1 kindly requests RAN4 to take the above RAN1 recommendation to develop test cases for PDSCH durations of 2, 4, and 7 OFDM symbols into account.

3. Dates of next TSG-RAN WG1 Meetings:

RAN1 NR Ad-Hoc#3	18th – 21th September 2017	Nagoya, Japan

RAN1 #90bis	9th – 13th October 2017	Prague, Czech Republic