3GPP TSG-RAN WG4 #84bis Meeting                               R4-1710269

Dubrovnik, Croatia, 9 - 13 October, 2017

Agenda item:	9.3.3.1

Source: 	ZTE

Title: 	On NR spectrum utilization of single numerology case for sub-6GHz

Document for:	Approval

Introduction

In August RAN4#84 meeting R4-1709075 WF on spectrum utilization was approved for single carrier operation and single numerology case below 6GHz [1]:

The RB values in the table above are agreed based on the assumption of symmetric guard band, and they are only meant to indicate the minimum guard band achievable for a given combination of SCS and channel BW. The RAN1 agreements below for subcarrier and RB alignment between different SCSs in a channel BW [2][3][4] could result in asymmetric guard band for some combinations of channel BW and higher order SCS [5], and their actual maximum RB allocation should be 1 RB less than the values in the WF on spectrum utilization above.

In September RAN4 AH#3 meeting some open issues have been identified for finalizing the spectrum utilization RB values for single carrier operation and single numerology case [6] and made the following agreement:

Agreement:

Companies are encouraged to bring contributions to next RAN4 meeting with different RB alignment options to check which SCS and channel BW combinations’ RB values can be reduced and how much.

In this contribution, we give our views on the open issues and calculate the guard band of each SCS and channel BW combination according to the RAN1 agreements and our assumption on channel raster design to find out which combinations’ RB values should be reduced by 1 RB.

Identified open issues and our views

  - Open issue 1

- Open issue 1 is: Which SCS in a channel BW will be the reference SCS for subcarrier and RB alignment to finalize the RB values?

- Option 1: determined minimum SCS in a channel BW.

  - i.e. for sub 6GHz:  SCS 15kHz for channel BW ≤ 50MHz, and SCS 30KHz for channel BW > 50MHz;

- Option 2: choosing reference SCS among NW scheduling.

Our view is that option 1 should be used. In Release 15 NR specifications we expect that there will be a single RB value specified for each SCS and channel BW combination for single carrier operation and single numerology case for sub-6GHz. Using option 2 might will result in multiple RB values for each SCS and channel BW combination which will make the standard and real implementation complicated.

Proposal 1: In a channel BW the reference SCS for subcarrier and RB alignment is the determined minimum SCS in the channel BW, i.e. for sub 6GHz, SCS 15kHz for channel BW ≤ 50MHz, and SCS 30 kHz for channel BW > 50MHz.

  - Open issue 2

### Open issue 2 is: RAN1 agreements on subcarrier alignment and RB grid alignment among different SCSs in a channel BW are not clear. Here are the possible options understood in RAN4 discussions:

- Option 1: start from left channel edge

  - Option 1A: Align PRB #0 across different SCSs.

  - Option 1B: Do not put constraints on aligning PRB #0 across different SCSs.

- Option 2: start from channel center and do not put constraints on aligning PRB #0 across different SCSs

To maximize the spectrum utilization for a channel BW the goal is trying to make the guard band of the higher order SCSs as less asymmetric as possible meanwhile not violating RAN1 subcarrier and RB alignment agreements. Option 1A will make the guard band of the higher order SCSs more asymmetric so it’s not a good option. If the reference SCS is the determined minimum SCS in a channel BW, Option 1B might will have the same results as Option 2. We think Option 2 is more straight forward and easier to meet the goal so we will use Option 2 in this contribution to calculate the guard band of each SCS and channel BW combination. For a channel BW in sub-6GHz the procedure to do the calculation with option 2 could be:

- Put all RBs of the reference SCS in the middle of the channel BW. According to RAN4 future agreements such as channel raster, channel raster mapping to subcarrier, etc., left-shifting all RBs to the exact location, and calculating the guard band of the reference SCS;

- Put all RBs of 2*(reference SCS) in the middle of the channel BW;

- Left-shifting all RBs of 2*(reference SCS) until the subcarrier 0 is aligned with the subcarrier 0 of the reference SCS;

- Calculating the guard band of 2*(reference SCS);

- Put all RBs of 4*(reference SCS) in the middle of the channel BW;

- Left-shifting all RBs of 4*(reference SCS) until the subcarrier 0 is aligned with the subcarrier 0 of 2*(reference SCS);

- Calculating the guard band of 4*(reference SCS);

Observation 1: Option 2 (i.e. start from channel center and do not put constraints on aligning PRB #0 across different SCSs) is straight forward and easier to make the guard band of the higher order SCSs as less asymmetric as possible meanwhile not violating RAN1 subcarrier and RB alignment agreements.

Spectrum utilization considering RAN1 agreements

The center of all RBs of the reference SCS could be impacted by RAN4 future agreements such as 100kHz/subcarrier based channel raster, channel raster position, and channel raster mapping to subcarrier. Since RAN4 has not reached the agreements for the channel raster design, in this contribution we assume that the center of all RBs of the reference SCS in a channel BW will be left shifted Δ kHz away from the center of the channel BW. In last RAN4 meeting for frequency range 2.6GHz to 6GHz there are discussions that subcarrier based channel raster will be used and raster points to the center of the channel: SC#0 of RB# NRB/2 for even number of RBs, and SC#6 of RB# int(NRB/2) for odd number of RBs. In such a case, Δ will be half of the reference SCS. Figure 1 shows Δ = 7.5 for channel BW 5MHz. The raster (red line) is at the center of the channel BW, and the center (green line) of all 25 RBs of the reference SCS 15 kHz is left shifted 7.5 kHz so that subcarrier #6 of the center RB (#12) is aligned with the raster.

Figure 1: Δ = 7.5 for channel BW 5MHz

Table 1 summarizes the Δ value for each channel BW.

Table 1: Δ value for each channel BW

| Channel BW [MHz] |  | 5 | 10 | 15 | 20 | 25 | 40 | 50 | 60 | 80 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Reference SCS = 15kHz | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 | N.A | N.A | N.A |
|  | Reference SCS = 30kHz | N.A | N.A | N.A | N.A | N.A | N.A | N.A | 15 | 15 | 15 |

Table 2 summarizes the tentative agreed RB values (numbers in black in the first row of a cell) in the latest WF on spectrum utilization [1], the minimum left guard band and minimum right guard band (numbers in green in the 2nd row of a cell), and the left guard band and right guard band (numbers in red in the 3rd row of a cell) considering the RAN1 subcarrier 0 alignment agreement and Δ kHz left shift for the center of all RBs of the reference SCS of a channel BW. The highly asymmetric guard bands are highlighted with yellow color in Table 2, and the corresponding RB values of the SCS and channel BW combinations should be reduced with 1 RB since the left guard band is much smaller than the minimum left guard band. The asymmetric guard bands with guard band difference ≤ 15 kHz are highlighted with light green color in Table 2, and RAN4 should decide if the corresponding RB values of thees combinations of SCS and channel BW should be reduced with 1 RB or not.

Table 2: Tentative agreed RB values (numbers in black in the first row of a cell) in the latest WF on spectrum utilization [1], the minimum left guard band and minimum right guard band (numbers in green in the 2nd row of a cell), and the left guard band and right guard band (numbers in red in the 3rd row of a cell) considering the RAN1 agreements

| Channel BW [MHz] |  | 5 | 10 | 15 | 20 | 25 | 40 | 50 | 60 | 80 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SCS 15 | 25 250,250 242.5,257.5 | 52 320,320 312.5,327.5 | 79 390,390 382.5,397.5 | 106 460,460 452.5,467.5 | 133 530,530 522.5,537.5 | 216 560,560 552.5,567.5 | 270 700,700 692.5,707.5 | N.A | N.A | N.A |
|  | SCS 30 | 11 520,520 415,625 | 24 680,680 665,695 | 38 660,660 555,765 | 51 820,820 805,835 | 65 800,800 695,905 | 106 920,920 905,935 | 133 1060,1060 1045,1075 | 162 840,840 825,855 | 217 940,940 925,955 | 273 860,860 845,875 |
|  | SCS 60 | N.A | 11 1040,1040 1010,1070 | 18 1020,1020 900,1140 | 24 1360,1360 1150,1570 | 31 1340,1340 1040,1640 | 51 1640,1640 1610,1670 | 65 1600,1600 1390,1810 | 79 1560,1560 1530,1590 | 107 1480,1480 1270,1690 | 135 1400,1400 1190,1610 |

Table 3 shows the reduced RB values (highlighted with yellow colour), and the RB values in square bracket which RAN4 should make a decision if they should be reduced by 1 RB.

Table 3: The reduced RB values (highlighted with yellow colour), and the RB values in square bracket which RAN4 should make a decision if they should be reduced by 1 RB.

| Channel BW [MHz] |  | 5 | 10 | 15 | 20 | 25 | 40 | 50 | 60 | 80 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SCS 15 | [25] | [52] | [79] | [106] | [133] | [216] | [270] | N.A | N.A | N.A |
|  | SCS 30 | 10 | 23 | 37 | 50 | 64 | 105 | 132 | 161 | 216 | 272 |
|  | SCS 60 | N.A | 10 | 17 | 23 | 30 | 50 | 64 | 78 | 106 | 134 |

Based on the discussions above, we have the following proposals:

Proposal 2: For the NR spectrum utilization of single carrier operation and single numerology case for 2.6GHz to 6GHz, considering the RAN1 agreements in R1-1706756 LS on subcarrier grid in NR, the tentative agreed RB values in R4-1709075 of the following SCS and channel BW combinations (highlighted with yellow colour) should be reduced to the values shown in the table below in order to meet the minimum guard band requirement.

| Channel BW [MHz] |  | 5 | 10 | 15 | 20 | 25 | 40 | 50 | 60 | 80 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SCS 15 | [25] | [52] | [79] | [106] | [133] | [216] | [270] | N.A | N.A | N.A |
|  | SCS 30 | 10 | 23 | 37 | 50 | 64 | 105 | 132 | 161 | 216 | 272 |
|  | SCS 60 | N.A | 10 | 17 | 23 | 30 | 50 | 64 | 78 | 106 | 134 |

Proposal 3: For the NR spectrum utilization of single carrier operation and single numerology case for 2.6GHz to 6GHz, for the RB values in square bracket in the table above, RAN4 should make a decision if the RB values should be reduced by 1 RB.

Conclusions

Based on the discussions in this contribution, we have the following Observation and Proposal:

Proposal 1: In a channel BW the reference SCS for subcarrier and RB alignment is the determined minimum SCS in the channel BW, i.e. for sub 6GHz, SCS 15kHz for channel BW ≤ 50MHz, and SCS 30 kHz for channel BW > 50MHz.

Observation 1: Option 2 (i.e. start from channel center and do not put constraints on aligning PRB #0 across different SCSs) is straight forward and easier to make the guard band of the higher order SCSs as less asymmetric as possible meanwhile not violating RAN1 subcarrier and RB alignment agreements.

Proposal 2: For the NR spectrum utilization of single carrier operation and single numerology case for 2.6GHz to 6GHz, considering the RAN1 agreements in R1-1706756 LS on subcarrier grid in NR, the tentative agreed RB values in R4-1709075 of the following SCS and channel BW combinations (highlighted with yellow colour) should be reduced to the values shown in the table below in order to meet the minimum guard band requirement.

| Channel BW [MHz] |  | 5 | 10 | 15 | 20 | 25 | 40 | 50 | 60 | 80 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SCS 15 | [25] | [52] | [79] | [106] | [133] | [216] | [270] | N.A | N.A | N.A |
|  | SCS 30 | 10 | 23 | 37 | 50 | 64 | 105 | 132 | 161 | 216 | 272 |
|  | SCS 60 | N.A | 10 | 17 | 23 | 30 | 50 | 64 | 78 | 106 | 134 |

Proposal 3: For the NR spectrum utilization of single carrier operation and single numerology case for 2.6GHz to 6GHz, for the RB values in square bracket in the table above, RAN4 should make a decision if the RB values should be reduced by 1 RB.

# References

- R4-1709075 WF on spectrum utilization, August 2017.

- R1-1706173 WF on subcarrier grid in NR, April 2017.

- R1-1706756 LS on subcarrier grid in NR, April 2017.

- R1-1708890 Final Report of 3GPP TSG RAN WG1 #88bis v1.0.0, May 2017.

- R4-1707599 Sub-carrier and resource block alignment in NR, August 2017.

- R4-1709989 WF on NR spectrum utilization of single numerology case, September 2017.