3GPP TSG-RAN WG4 Meeting #84-bis	R4-1710271

# Dubrovnik, Croatia, 9 - 13 Oct, 2017

Source:	KDDI Corporation

Title:	TP for TR 36.715-02-02: UE RF requirements on CA_11A-18A

Agenda item:	8.6.2 - UE RF [LTE_CA_R15_2DL2UL-Core]

Document for:	Approval

Introduction

In this contribution, we provide the text proposal for TR 36.715-02-02 to capture necessary requirements for UL CA_11A-18A.

Proposal

It is proposed to approve TP described in section 4.

Reference

TP for TR36.715-02-02

<Start of TP>

## 6.X	CA_11A-18A

### 6.X.1	Operating bands for CA

Table 6.X.1-1: Inter-band CA operating bands for CA_11-18

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_11-18 | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |

### 6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_11A-18A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_11A-18A | 11 |  |  | Yes | Yes |  |  | 25 | 0 |
|  | 18 |  |  | Yes | Yes | Yes |  |  |  |

### 6.X.3	Co-existence studies

Table 6.X.3-1 lists B11+B18 2UL CA 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.X.3-1: Band 11 and Band 18 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1427.9 | 1447.9 |  | 815 | 830 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |  |
| 2nd harmonics frequency limits (MHz) | 2855.8 – 2895.8 |  |  | 1630 – 1660 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low |  | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4283.7 – 4343.7 |  |  | 2445 – 2490 |  |  |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 597.9 – 632.9 |  |  | 2242.9 – 2277.9 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 2025.8 – 2080.8 |  |  | 182.1 – 232.1 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3670.8 – 3725.8 |  |  | 3057.9 – 3107.9 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3453.7 – 3528.7 |  |  | 997.1 – 1062.1 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 1195.8 – 1265.8 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5098.7 – 5173.7 |  |  | 3872.9 – 3937.9 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |  |
| IMD frequency limits (MHz) | 4485.8 – 4555.8 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1812.1 – 1892.1 |  |  | 4881.6 – 4976.6 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 365.8 – 450.8 |  |  | 2623.7 – 2713.7 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4687.9 – 4767.9 |  |  | 6526.6 – 6621.6 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5300.8 – 5385.8 |  |  | 5913.7 – 6003.7 |  |  |

Based on the table above, no desense is expected due to harmonic and IM falldown to Rx bands of Band 11 and Band 18.

When 2UL inter-band CA UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.X.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.X.3-2: 2UL B11+ B18 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 |  |  |  |
| Galileo | 1559 | - | 1591 |  |  |  |
| GLONASS | 1591 | - | 1610 |  |  |  |
| GPS | 1563 | - | 1587 |  |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 |  | US/Europe | 3rd harmonic |
|  | 2400 | - | 2494 |  | Asia | 3rd harmonic |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | IMD5 |
|  | 5150 | - | 5350 | YES | Europe | IMD5 |
|  | 5470 | - | 5725 | YES |  | IMD5 |
|  | 5150 | - | 5825 | YES | Asia | IMD5 |

### 6.X.4	∆TIB,c and ∆RIB,c values

For inter-band CA_11-18, same requirements on values for ∆TIB,c and ∆RIB,c can be applied as per 1UL-2DL case. Therefore, for two simultaneous UL, the TIB,c and RIB,c values are shown in table 6.X.4-1 and in table 6.X.4-2:

Table 6.X.4-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_11A-18A | 11 | 0.3 |
|  | 18 | 0.3 |

Table 6.X.4-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_11A-18A | 11 | 0 |
|  | 18 | 0 |

<End of TP>