3GPP TSG RAN WG4 Meeting #84bis	R4-1711450

Dubrovnik, Croatia, October 9 – 13, 2017

Agenda Item:	7.9.5.2

Source: 	Ericsson

Title:	Inter-frequency RSRP and RSRQ measurement accuracies for UE Cat M1

Document for:	Discussion

# Introduction

Placeholders for measurement accuracy requirements for UE cat M1 were introduced to TS 36.133 at the RAN4#83 meeting in Hangzhou. Measurement requirements for CE Mode A were agreed at the RAN4#84 meeting in Berlin [2]. The requirement placeholders that remain to be populated are the following:

9.1.21.7			Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B

9.1.21.11			Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B

9.1.21.12			Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B

9.1.21.15			Inter-frequency Absolute RSRQ Accuracy of RSRQ for UE category M1 in CE mode B

9.1.21.16			Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B

In this contribution we are proposing CE Mode B accuracy requirements based on RAN4#84 contributions from Ericsson [4] and Huawei [5].

# Proposed Requirements

The measurement accuracy requirements are based on physical layer measurement results derived under the assumption of perfect calibration, and then for RSRP absolute accuracy, and RSRP inter-frequency relative accuracy, RF margin is added. The reason for adding RF margins is that there is a calibration tolerance in the manufacturing of the UE (outline of procedure is provided in [3]). Following earlier agreements, for UE Cat M1 we are assuming an RF margin of ±4dB instead of the ±2.5dB assumed for legacy UEs.

For measurement accuracy requirements on RSRQ, the calibration errors have equal impacts on the numerator (RSRP) and the denominator (RSSI). Hence, for RSRQ there is no need to consider an RF margin since it is cancelled out.

9.1.21.7			Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B

Based on simulation results ([4], and [5]) which after RF margin adjustments differ by 1 dB (Ericsson 5dB, Huawei 4dB), and applying the same pattern as for CE Mode A regarding difference between normal and extreme conditions, we propose the following requirements to apply for Io ≤ -50dBm:

For -12 ≥ Ês/Iot > -6dB normal conditions: ±5dB, and extreme conditions: ±6.5dB

For -15 ≥ Ês/Iot > -12dB normal conditions: ±6dB, and extreme conditions: ±6.5dB

9.1.21.11			Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B

Based on simulation results ([4], and [5]) which are in agreement with each other, applying the 1dB relaxation for Ês/Iot < -12dB, and the same pattern as for CE Mode A regarding difference between normal and extreme conditions, we propose the following requirement to apply:

For Io < -70dBm;

For -12 ≤ Ês/Iot < -6dB normal conditions: ±7dB, and extreme conditions: ±10dB

For -15 ≤ Ês/Iot < -12dB normal conditions: ±8dB, and extreme conditions: ±11dB

For  -70 ≤ Io < -50dBm;

For -12 ≤ Ês/Iot < -6dB normal conditions: ±9dB, and extreme conditions: ±12dB

For -15 ≤ Ês/Iot < -12dB normal conditions: ±10dB, and extreme conditions: ±13dB

9.1.21.12			Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B

Following the methodology for normal condition in legacy (see [1] and compare 9.1.3.1 and 9.1.3.2), we propose the requirements to be identical to the inter-frequency absolute tolerance (9.1.21.11).

9.1.21.15			Inter-frequency Absolute RSRQ Accuracy of RSRQ for UE category M1 in CE mode B

Based on simulation results ([4], [5]) which after RF margin adjustments differ by 1 dB (Ericsson 5dB, Huawei 4dB), and applying the same pattern as for CE Mode A regarding difference between normal and extreme conditions, we propose the following requirements to apply for Io ≤ -50dBm:

For -12 ≥ Ês/Iot > -6dB normal conditions: ±5dB, and extreme conditions: ±6.5dB

For -15 ≥ Ês/Iot > -12dB normal conditions: ±6dB, and extreme conditions: ±6.5dB

9.1.21.16			Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B

Following the methodology in legacy (see [1] and compare 9.1.6.1 and 9.1.6.2), we propose the requirements to be based on the inter-frequency absolute tolerance plus an extra margin of 0.5dB for normal conditions. Hence we propose the following requirements to apply for Io ≤ -50dBm:

For -12 ≥ Ês/Iot > -6dB normal conditions: ±5.5dB, and extreme conditions: ±6.5dB

For -15 ≥ Ês/Iot > -12dB normal conditions: ±6.5dB, and extreme conditions: ±6.5dB

# Summary and Conclusion

We have proposed values for inter-frequency absolute and relative RSRP measurement accuracy requirements, and intra- and inter-frequency absolute and relative RSRQ measurement accuracy requirements, for UE cat M1 operating in CE Mode B. The proposed values are based on simulation input from Ericsson and Huawei, and the same alignment regarding RF margin as when agreeing on CE Mode A requirements has been applied.

A related CR where the values are incorporated within brackets in 36.133 is provided in [6].

# References

- 36.133

- R4-1708703 “CR on 36.133 Intra and inter-frequency RSRP and RSRQ measurement accuracies for UE cat M1”, Ericsson

- R4-144891 “Tightening RSRP accuracy requirements", Ericsson

- R4-1708313 “Inter-frequency RSRP and RSRQ measurement accuracies for UE Cat M1”, Ericsson

- R4-1708263 “CR on intra and inter-frequency RSRP and RSRQ measurement accuracies for FeMTC”, Huawei, HiSilicon

- R4-1711451 “CR on 36.133 Intra and inter-frequency RSRP and RSRQ measurement accuracies for UE cat M1”, Ericsson

# Appendix

## RSRP and RSRQ Measurement approach

The legacy requirements on measurement accuracy in static (AWGN) conditions can be met by coherently averaging 8 CRSs, calculate the power, and then average such power estimates non-coherently over 2x5 subframes. The coherent averaging reduces the bias and the non-coherent averaging the variance of the RSRP estimate. The increase in SINR achieved by the coherent averaging is 3dB for every doubling of the number of coherently averaged CRSs. Hence when deriving the legacy requirements an SINR increase of 9dB was achieved, allowing measurement accuracy requirements to be fulfilled at least down to Ês/Iot -6dB.

To be able to accurately support enhanced coverage down to Ês/Iot -15dB, the coherent averaging needs to include more samples to provide sufficient number of REs containing CRS compared to that of an individual CRS. Using all CRSs in two adjacent subframes provides 96 CRSs, which allows for an SINR increase of about 10.8dB and thus would allow Ês/Iot down to -16.8 dB to be supported. However, increasing the time and frequency window over which the REs are coherently averaged makes the result sensitive to time- and frequency characteristics of the propagation channel, as well as to carrier frequency offset. Therefore, the performance gain by increasing the number of REs in general is smaller than in theory.

The two sizes of coherent averages are shown in Figure 1.

Figure 1: Illustration of (left) coherent averaging based on 8 CRSs and (right) on 96 CRSs. The former can increase the SINR by 9dB and the latter by 19.8 dB.

## Simulation assumptions

Table 3: Simulation parameters

| Parameters | Value | Comments |
| --- | --- | --- |
| Measurement bandwidth | 6 RBs |  |
| L1 measurement period | 960 ms (CE mode A) 1600 ms (CE mode B) |  |
| Measurement sampling rate | 80ms |  |
| Consecutive subframes used | 2 | Coherent averaging over 96 REs |
| L3 filtering | Disabled |  |
| Transmit antennas | 1 | One TX port only |
| Receive antennas | 1 | One RX branch |
| Propagation conditions CE mode B | AWGN, ETU 1, EPA 1Hz | Range: -15 ≤ Ês/Iot < -6 dB |
| CP length | Normal |  |
| Carrier frequency | 2 GHz |  |
| Residual CFO | 50Hz |  |

## Simulation results, CE mode B