3GPP TSG-RAN WG4 Meeting #84bis	R4-1711487

Dubrovnik, Croatia, 9th-13th October 2017

Source: 	Ericsson

Title:  	TP to TR 38.817-02 v0.2.0: Methodology for mmWave NR BS receiver blocking investigation

Agenda Item:	9.5.4.3.2

Document for:	Approval

# Background

A skeleton for TR 38.817-02 has been approved in [1].

In the past in RAN4, extensive discussions have been done in RAN4 with respect to inband receiver blocking for FR2 NR BS. We have proposed a methodology which considers joint probability of wanted and interferer while considering the receiver blocking for FR2 NR BS. We presented our analysis in [2] and [3].

# Proposal

We propose to adopt the text in this contribution to TR 38.817-02.

# References

R4-1710085, TR 38.817-02 v0.2.0: General aspects for BS RF for NR

R4-1706811, The importance for considering joint probability for receiver blocking in NR BS, Ericsson

R4-1708145, Consideration of joint probability for wanted and interference signals in mmWave NR BS receiver blocking investigation, Ericsson

# Text proposal for TR 38.817-02

# <Start of Text proposal>

### 9.7.3	OTA Adjacent Channel Leakage Power Ratio (ACLR)

#### 9.7.3.1	General

The following is agreed for ACLR requirement for FR2:

- For FR2 ACLR definition, the same CBW with wanted signal can be assumed as adjacent CBW.

- Transmission BW configuration of wanted channel (it would differ depend on SCS and/or CBW) should be used as a measurement BW for wanted signal power measurement.

- Absolute ACLR limits for mmWave NR BS shall be equal to in-band spurious emission limit.

- No ACLR2 requirement is needed since emission level at ACLR2 range will be close to noise floor.

- Consider the adjacent carrier bandwidth which corresponds to largest transmission BW configuration when more than one subcarrier spacing is considered.

- Consider only the test which corresponds to smallest guard band between wanted and adjacent carrier.

Based on the agreements mentioned above, the ACLR requirement is specified in Table 9.7.3.1-1.

# Table 9.7.3.1-1: Base Station ACLR for FR2

| Channel bandwidth of E-UTRA lowest/highest carrier transmitted BWChannel [MHz] | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Subcarrier spacing (kHz) at wanted carrier | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- | --- |
| [50, 100, 200, 400, …] | BWChannel | 30, 60, 120 | NR of same BW with the SCS that provides largest transmission BW configuration | Square (BWConfig) | 28 (Note 1), 26 (Note 2), 24 (Note 3) |
| Note 1: Applicable to bands defined within the spectrum range of 24.24 – 33.4 GHz Note 2: Applicable to bands defined within the spectrum range of 37 – 52.6 GHz Note 3: Applicable to bands defined within the spectrum range of 66 – 86 GHz |  |  |  |  |  |

#### 9.7.3.2	AAS radiated ACLR requirement

Minimum radiated requirement for 1-O NR BS in FR1 is defined based on basic limit with the emissions scaling applied. Refer to sub-clause 6.6.3.2 for basic limit definition.

Minimum radiated requirement for 2-O NR BS in FR2 is FFS.

# <End of Text proposal>