3GPP TSG-RAN WG4 Meeting #86bis                     	R4-1803691

Melbourne, Australia, April 16th –20th, 2018

Agenda item:            7.2.2.1

Source:	MediaTek Inc.

Title:	Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective

Document for:          Discussion

1	Introduction

In this paper, we discuss the feature “Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective”

2	Discussion

In the feature list [1], the condition for supporting “Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective” is not clear as shown Table1.

Table 1 Feature description of Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective

| WI | # | Feature group | Components | Prerequisite feature groups | Need for gNB t | Consequences if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | RAN5 implication | Remarks | Responsible WG | Recommendation for TSG-RAN | TSG-RAN decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1-11 | Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective | Support of switching type between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective. Type 1: ~0us Type 2: <20us | 1-10 | Yes | UE does not support UL subcarrier alignment between LTE and NR for EN-DC with LTE-NR coexistence in UL | Type 3 (per band combination) | No Need | Applicable only to FR1 |  | This feature is the switching time between LTE UL and NR UL in the same carrier Per band combination signalling (Type 3) UE Capability signalling elements. 1: ~0us switching type. 2: <20us switching type. | RAN4 |  |  |

In the last meeting, additional condition of same carrier between LTE and NR is added to clarify the  requirement:

Agreements Keep the name of feature “Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective”. Add the clarifications in the remark column to clarify this feature is the switching time between LTE UL and NR UL in the same carrier

However, from UE implementation viewpoint, we think there are other factors affecting the actual switching time, including:

- NR and LTE may have different numerology and slot structure

- NR and LTE may have different RF bandwidth

- Extra timing alignment may be needed between NR and LTE

  - Note: to our understanding, there is only one agreement that normal UL and SUL are in the same TAG.

- NR and LTE may have TX performance requirement.

Considering the evaluation efforts and limited meeting time to finalize SUL in Release 15, we suggest to limit the combination of NR and LTE numerology. For more complicated scenarios, they can be considered in future releases.

Proposal 1: For LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective, LTE and NR UL have the same bandwidth, center frequency and subcarrier spacing, and the timing must be aligned.

Proposal 2: For capability of “Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective”, it is clarified as follow:

- The capability is only applied to the scenario that LTE and NR UL have the same RF bandwidth, center frequency and subcarrier spacing, and the timing must be aligned

3	Conclusions

In the contribution, we provide our view on the Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective. We have the following proposals:

Proposal 1: For LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective, LTE and NR UL have the same bandwidth, center frequency and subcarrier spacing, and the timing must be aligned

Proposal 2: For capability of “Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective”, it is clarified as follow:

- The capability is only applied to the scenario that LTE and NR UL have the same RF bandwidth, center frequency and subcarrier spacing, and the timing must be aligned

4	Reference

[1] R4-1803564, “Updated UE feature list”