3GPP TSG-RAN WG4 Meeting #86Bis                                   R4-1803849

Melbourne, Australia, 16th – 20th April, 2018

Agenda item:			7.9.9.2

Source:			Samsung

Title:	Clarification on Sync/Async EN-DC scenarios and Correction to Timing Requirement

Document for:		Discussion

# 1	Introduction

In last Oct. RAN4 meeting, there was WF approved for synchronous and asynchronous definitions for LTE-NR DC in Rel-15, with the following agreement for inter-band and intra-band EN-DC respectively [1]:

Furthermore, discussion on UE feature list made conclusion on the capability for Simultaneous reception and transmission for inter-band CA or EN-DC (TDD-TDD or TDD-FDD), and also Asynchronous FDD-FDD intra-band LTE-NR DC [2].

With the above conclusion to be considered, we would like to clarify the scenarios for EN-DC and also the text proposal to TS38.133’s MTTD/MRTD section.

# Scenarios of EN-DC and Capability Related

With the above agreement taken into account, our understanding for EN-DC scenarios and corresponding capability can be summarized as below

Table 1. EN-DC Scenarios and Corresponding Capability

|  | FDD-FDD | TDD-TDD | TDD-FDD |
| --- | --- | --- | --- |
| Intra-band | Sync (Mandatory), Async(Optional) Async support depends on UE capability (only support collocation for Rel-15) | Sync (only support Sync for Rel-15) (only support collocation for Rel-15) | N.A. |
| Inter-band | Sync/Async (Both supported) | Sync/Async (Both supported) (UE capability for "simultaneous TXRX") | Sync/Async (Both supported) (UE capability for "simultaneous TXRX") |

# Text Proposal for MTTD/MRTD

With the above understanding, we believe the corresponding core requirement for MTTD/MRTD needs to be revised.

Begin of Change 1

## 7.5	Maximum Transmission Timing Difference

### 7.5.1	Introduction

A UE shall be capable of handling a relative transmission timing difference between subframe timing boundary of E-UTRA PCell and slot timing boundaries of PSCell to be aggregated EN-DC

### 7.5.2	Minimum Requirements for inter-band EN-DC

For inter-band The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2-1. The requirements for asynchronous EN-DC are applicable for E-UTRA TDD- NR TDD, E-UTRA FDD- NR FDD, E-UTRA FDD-NR TDD and E-UTRA TDD-NR FDD inter-band asynchronous EN-DC.

Table 7.5.2-1 Maximum uplink transmission timing difference requirement for asynchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | UL Sub-carrier spacing for data in PSCell (kHz) | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 500 |
| 15 | 30 | 250 |
| 15 | 60 | 125 |
| 15 | 120Note1 | 62.5 |
| Note1 : For E-UTRA FDD- NR FDD and E-UTRA TDD- NR TDD intra-band EN-DC, 120kHz is not applied. |  |  |

The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2-2. The requirements for synchronous EN-DC are applicable for E-UTRA FDD-NR FDD, E-UTRA TDD-NR TDD, E-UTRA TDD-NR FDD and E-UTRA FDD-NR TDD inter-band EN-DC.

Table 7.5.2-2 Maximum uplink transmission timing difference requirement for inter-band synchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | UL Sub-carrier spacing for data in PSCell (kHz) | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 35.21 |
| 15 | 30 | 35.21 |
| 15 | 60 | 35.21 |
| 15 | 120 Note1 | 35.21 |
| Note 1: For E-UTRA FDD- NR FDD and E-UTRA TDD- NR TDD intra-band EN-DC, 120kHz is not applied. |  |  |

### 7.5.3	Minimum Requirements for intra-band EN-DC

For intra-band EN-DC, only collocated deployment is applied.

The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2-1 for E-UTRA FDD-NR FDD intra-band EN-DC provided the UE indicates that it is capable of asynchronous EN-DC operation [16]. The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2-1 for E-UTRA TDD-NR TDD intra-band EN-DC.

No uplink transmission timing difference is applicable for synchronous EN-DC.

## 7.6   Maximum Receive Timing Difference

### 7.6.1	Introduction

A UE shall be capable of handling a relative receive timing difference between subframe timing boundary of E-UTRA PCell and slot timing boundaries of PSCell to be aggregated for EN-DC.

A UE shall be capable of handling a relative receive timing difference between slot timing boundary of different carriers to be aggregated NR carrier aggregation.

### 7.6.2	Minimum Requirements for inter-band EN-DC

For inter-band The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from E-UTRA PCell and slot timing of signal from PSCell at the UE receiver as shown in Table 7.6.2-1. The requirements for asynchronous EN-DC are applicable for E-UTRA TDD- NR TDD, E-UTRA FDD- NR FDD, E-UTRA FDD- NR TDD and E-UTRA TDD- NR FDD inter-band EN-DC.

Table 7.6.2-1 Maximum receive timing difference requirement for asynchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | DL Sub-carrier spacing in PSCell (kHz)Note1 | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 500 |
| 15 | 30 | 250 |
| 15 | 60 | 125 |
| 15 | 120 | 62.5 |
| Note1 : DL Sub-carrier spacing is min{SCSSS, SCSDATA}. Note2 : For E-UTRA FDD- NR FDD and E-UTRA TDD- NR TDD intra-band EN-DC, 120kHz is not applied. |  |  |

The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from E-UTRA PCell and slot timing of signal from PSCell at the UE receiver as shown in Table 7.6.2-2. The requirements for synchronous EN-DC are applicable for E-UTRA FDD-NR FDD, E-UTRA TDD- NR TDD, E-UTRA TDD- NR FDD and E-UTRA FDD- NR TDD inter-band EN-DC.

Table 7.6.2-2 Maximum receive timing difference requirement for inter-band synchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | DL Sub-carrier spacing in PSCell (kHz)Note1 | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 33 |
| 15 | 30 |  |
| 15 | 60 |  |
| 15 | 120 |  |
| Note1: DL Sub-carrier spacing is min{SCSSS, SCSDATA}. Note2: For E-UTRA FDD- NR FDD and E-UTRA TDD- NR TDD intra-band EN-DC, 120kHz is not applied. |  |  |

### 7.6.3	Minimum Requirements for intra-band EN-DC

For intra-band EN-DC, only collocated deployment is applied.

The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from E-UTRA PCell and slot timing of signal from PSCell as shown in Table 7.6.2-1 for E-UTRA FDD-NR FDD intra-band EN-DC provided the UE indicates that it is capable of asynchronous EN-DC operation [16].

The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from E-UTRA PCell and slot timing of signal from PSCell as shown in Table 7.6.3-1. The requirements for synchronous EN-DC are applicable for E-UTRA TDD- NR TDD and E-UTRA FDD- NR FDD intra-band EN-DC.

Table 7.6.3-1 Maximum receive timing difference requirement for intra-band synchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | DL Sub-carrier spacing in PSCell (kHz) Note1 | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 3 |
| 15 | 30 | 3 |
| 15 | 60 | 3 |
| Note1: DL Sub-carrier spacing is min{SCSSS, SCSDATA}. |  |  |

End of Change 1

# Conclusion

In this paper, we provided our clarification on the understanding for the scenarios for EN-DC and corresponding capability signaling considering previous RAN4 discussion as below, and correspondingly the text proposal for MTTD/MRTD in TS38.133.

Table 1. EN-DC Scenarios and Corresponding Capability

|  | FDD-FDD | TDD-TDD | TDD-FDD |
| --- | --- | --- | --- |
| Intra-band | Sync (Mandatory), Async(Optional) Async support depends on UE capability (only support collocation for Rel-15) | Sync (only support Sync for Rel-15) (only support collocation for Rel-15) | N.A. |
| Inter-band | Sync/Async (Both supported) | Sync/Async (Both supported) (UE capability for "simultaneous TXRX") | Sync/Async (Both supported) (UE capability for "simultaneous TXRX") |

# Reference

[1] R4-1711964, “WF on synchronous and asynchronous definitions for LTE-NR DC in Rel-15”, Ericsson, MediaTek.

[2] R4-1803551, “WF on UE feature list for CA/EN-DC, other capability and LTE-NR co-existence”, Intel.