3GPP TSG-RAN WG4 Meeting #86bis	R4-1804030

Melbourne, Australia, 16th – 20th Apr 2018

Source: 	Nokia

Title: 					TP to TR 36.715-05-01: CA_5DL_5A-12A-48D_1UL_BCS0

Agenda Item: 			6.5

Document for:	Approval

# 1	Introduction

This contribution is a TP for introduction of CA_5DL_5A-12A-48D_1UL_BCS0 into 5DL TR.

******************************** Start of the TP *******************************************

## 5.X	CA_5A-12A-48D_BCS0

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.2-1: Supported E-UTRA bandwidths per CA configuration for 5DL inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_5A-12A-48D | - | 5 |  |  | Yes | Yes |  |  | 80 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 48 | See the CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |

### 5.X.2	UE Co-existence studies

Table 5.X.2-1 summarizes frequency ranges where harmonics occur due to Band 5, Band 12 and Band 48 CA with 1 UL.

Table 5.X.2-1: Impact of 1 UL Harmonic Interference

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 5 | 824 | 849 | 1648 | 1698 | 2472 | 2547 |
| 12 | 699 | 716 | 1398 | 1432 | 2097 | 2148 |
| 48 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 |

It can be seen from Table 5.X.2-1 that there is no harmonic interference for CA_5A-12A-48D towards own receive bands.

### 5.X.3	∆TIB,c and ∆RIB,c values

For five simultaneous DLs and one UL of Band 5, band 12 and Band 46 for 1UL/5DL CA_5A-12A-48D, the TIB,c and RIB,c  values are shown in table 5.X.3-1 and  table 5.X.3-2, respectively.

dTib values for bands 5 and 12 are taken from existing CA_5A-12A and the band 48 value following the general H-L priciple.

Table 5.X.3-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_5A-12A-48D | 5 | 0.8 |
|  | 12 | 0.4 |
|  | 48 | 0.3 |

dRib values for bands 5 and 12 are taken from existing CA_5A-12A and the band 48 value following the general H-L priciple.

Table 5.X.3-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_5A-12A-48D | 5 | 0.5 |
|  | 12 | 0.3 |
|  | 48 | 0 |

### 5.X.4	REFSENS requirements

No MSD is required.

******************************** End of the TP *******************************************