3GPP TSG-RAN WG4 Meeting #86bis	R4-1804033

Melbourne, Australia, 16th – 20th Apr 2018

Source: 	Nokia

Title: 					TP to TR 36.715-02-01: CA_2DL_12A-48A_1UL_BCS0

Agenda Item: 			6.2

Document for:	Approval

# 1	Introduction

This contribution is a TP for introduction of CA_2DL_12A-48A_1UL_BCS0 into 3DL TR.

******************************** Start of the TP *******************************************

## 5.X	CA_12A-48A_BCS0

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.2-1: Supported E-UTRA bandwidths per CA configuration for 5DL inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 [MHz] | 3 [MHz] | 5 [MHz] | 10 [MHz] | 15 [MHz] | 20 [MHz] | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_12A-48A |  | 12 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.X.2	UE Co-existence studies

Table 5.X.2-1 summarizes frequency ranges where harmonics occur due to Band 12 and Band 48 CA with 1 UL.

Table 5.X.2-1: Impact of 1 UL Harmonic Interference

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 12 | 699 | 716 | 1398 | 1432 | 2097 | 2148 |
| 48 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 |

It can be seen from Table 5.X.2-1 that there is no harmonic interference for CA_12A-48A towards own receive bands.

### 5.X.3	∆TIB,c and ∆RIB,c values

For two simultaneous DLs and one UL of band 12 and Band 48 for 1UL/2DL CA_12A-48A, the TIB,c and RIB,c  values are shown in table 5.X.3-1 and  table 5.X.3-2, respectively.

dTib values for bands 12 and band 48 follow the general L-H priciple.

Table 5.X.3-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_12A-48A | 12 | 0.3 |
|  | 48 | 0.3 |

dRib values for bands 12 and band 48 follow the general L-H priciple.

Table 5.X.3-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_12A-48A | 12 | 0 |
|  | 48 | 0 |

### 5.X.4	REFSENS requirements

No MSD is required.

******************************** End of the TP *******************************************