3GPP TSG-RAN WG4 Meeting #86bis	R4-1804055

# Melbourne, Australia, 16th April-20th April 2018

Agenda item:	5.7.1

Source: 	Qualcomm Incorporated

Title: 	Discussion on A-MPR Requirements for V2X

Document for:	Agreement

# Introduction

In RAN4 meeting #86, the following working assumptions are reached regarding the A-SE, A-SEM and A-MPR for v2X service at band 47 [3]:

  - The UE is required to declare (at least 1) expected post-antenna connector gain (external antenna gain – cable loss between the UE antenna connector and external antenna) for external antenna case.

  - The existing A-SE and A-SEM is in eirp. Conducted emission mask is obtained by subtracting the expected post-antenna connector gain.

  - Need A-MPR analysis for post-antenna connector gain  0, 1, …, 7 dB for external antenna case.

  - [information] A-MPR for larger post antenna connector gain should accommodate more stringent conducted emission requirement

We provide in this contribution the simulation results regarding the last 2 bullet points. Those results will be used in our companion draft CR.

# Analysis

## A-MPR Simulation Result

We present in this section the A-MPR simulation result for different declared post connector gain. The simulation assumption is the same as in [4]. The only difference is that when check against the A-SEM and A-SE requirement, we take the gain value into account. As the trend result is similar to A-MPR result for 0dB margin, only the actual A-MPR value is higher for higher gain. Thus, it makes sense to keep the current A-MPR specification structure, and add new A-MPR value entries for new declared post connector gains. We capture the current A-MPR structure in Table 1 with a small adjustment. The adjustment is to accommodate the sub-channel size of 6RBs, which was originally missed out in the specification. The A-MPR values for QPSK, 16QAM and 64 QAM are provided in Tables 2, 3 and 4.

As it is agreed that a single total backoff power will be used for all modulation order, we provide in Table 5 the max total backoff based on our simulation result. The way this table is formulated is that the simulated A-MPR are added with the MPR for each modulation (table 5) order and the max over each cell is retained. Furthermore, for the case where the retained value is 3 (equal to MPR of 64QAM), we check on a case by case if lower value can be used. Then the A-MPR value can be specified as:

A-MPR = max(total-MPR – MPR, 0),

Where total-MPR is taken value from table 6 and MPR is taken value from table 5.

Table 1. A-MPR Structure

| Resource pool | Carrier frequency(MHz) | Resources Blocks (NRB) | Start Resource Block | A-MPR Index |
| --- | --- | --- | --- | --- |
| Adjacent | 5860 | [≤ 10] | [0] | 1 |
|  |  |  | [5, 6] | 2 |
|  |  |  | [10, 12] | 3 |
|  |  |  | [≥ 15] | 4 |
|  |  | [>10 & ≤22] | [0] | 5 |
|  |  |  | [5, 6] | 6 |
|  |  |  | [10, 12] | 7 |
|  |  |  | [15, 18] | 8 |
|  |  |  | [20, 24 25] | 9 |
|  |  |  | [≥ 30] | 10 |
|  |  | [>22] | [0, 5, 6] | 11 |
|  |  |  | [10, 12] | 12 |
|  |  |  | [15, 18] | 13 |
|  |  |  | [20, 24, 25] | 14 |
|  | 5870, 5910, 5920 | [<20] | ≥ 0 | 15 |
|  |  | [≥ 20 and ≤45] |  | 16 |
|  |  | [>45] |  | 17 |
|  | 5880, 5890, 5900 | [<10] | ≥ 0 | 18 |
|  |  | [≥ 10 and ≤38] |  | 19 |
|  |  | [>38] |  | 20 |

Table 2: A-MPR for QPSK

| QPSK |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Waveform Index | 0 dBi | 1 dBi | 2 dBi | 3 dBi | 4 dBi | 5 dBi | 6 dBi | 7 dBi |
| 1 | 14 | 14.5 | 15 | 16 | 17 | 17.5 | 18.5 | 19.5 |
| 2 | 6.5 | 7.5 | 8 | 8.5 | 9 | 10 | 10.5 | 11 |
| 3 | 4.5 | 5 | 5 | 5.5 | 6.5 | 7 | 7.5 | 8 |
| 4 | 3.5 | 4.5 | 5.5 | 6.5 | 7.5 | 8.5 | 9 | 10 |
| 5 | 13 | 14 | 14.5 | 15 | 16 | 17 | 17 | 18.5 |
| 6 | 10 | 10.5 | 11 | 12 | 12.5 | 13 | 13.5 | 14 |
| 7 | 8.5 | 9 | 9.5 | 10 | 10.5 | 11.5 | 12 | 12.5 |
| 8 | 4.5 | 5 | 5.5 | 6 | 6.5 | 7.5 | 8 | 8.5 |
| 9 | 3 | 3.5 | 4 | 4.5 | 5 | 6 | 6.5 | 7.5 |
| 10 | 2.5 | 3 | 3.5 | 4.5 | 5 | 5.5 | 6.5 | 7.5 |
| 11 | 10.5 | 11 | 12 | 12.5 | 13.5 | 14 | 14.5 | 15.5 |
| 12 | 8.5 | 9 | 10 | 10.5 | 11 | 11.5 | 12 | 13 |
| 13 | 8 | 8.5 | 9 | 9.5 | 10.5 | 11 | 11.5 | 12 |
| 14 | 5 | 6 | 6.5 | 7.5 | 8 | 8.5 | 9 | 10 |
| 15 | 1 | 2 | 2.5 | 2.5 | 3 | 4 | 5 | 6 |
| 16 | 0 | 0 | 0.5 | 1 | 1.5 | 2 | 2.5 | 3 |
| 17 | 0.5 | 1 | 1.5 | 2 | 2 | 2.5 | 3 | 3.5 |
| 18 | 1.5 | 1.5 | 2.5 | 2.5 | 3 | 3.5 | 4 | 4.5 |
| 19 | 0 | 0 | 0.5 | 1 | 1.5 | 2 | 3 | 3.5 |
| 20 | 0.5 | 1 | 1.5 | 1.5 | 2 | 2.5 | 3 | 3.5 |

Table 3 A-MPR for 16QAM

| 16 QAM |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Waveform Index | 0 dBi | 1 dBi | 2 dBi | 3 dBi | 4 dBi | 5 dBi | 6 dBi | 7 dBi |
| 1 | 13 | 14 | 14.5 | 15 | 16 | 17 | 18 | 19 |
| 2 | 6 | 7 | 7.5 | 8 | 8.5 | 9.5 | 10 | 10.5 |
| 3 | 4 | 4.5 | 5 | 5 | 6 | 6.5 | 7 | 7.5 |
| 4 | 3 | 4 | 5 | 6 | 7 | 8 | 8.5 | 9.5 |
| 5 | 13 | 13.5 | 14.5 | 14.5 | 15.5 | 16.5 | 17 | 18 |
| 6 | 9.5 | 10.5 | 11 | 11.5 | 12 | 12.5 | 13 | 14 |
| 7 | 7.5 | 8.5 | 9 | 9.5 | 10 | 10.5 | 11.5 | 12 |
| 8 | 4 | 4.5 | 5 | 5.5 | 6 | 6.5 | 7.5 | 8 |
| 9 | 3 | 3.5 | 4 | 4.5 | 5 | 5.5 | 6.5 | 7 |
| 10 | 2 | 3 | 3.5 | 4 | 4.5 | 5.5 | 6.5 | 7 |
| 11 | 10.5 | 11.5 | 12 | 13 | 13.5 | 14 | 14.5 | 15.5 |
| 12 | 8 | 9 | 9.5 | 10 | 11 | 11.5 | 12 | 12.5 |
| 13 | 7.5 | 8 | 9 | 9.5 | 10 | 10.5 | 11.5 | 12 |
| 14 | 4.5 | 5 | 6 | 6.5 | 7 | 8 | 8.5 | 9 |
| 15 | 0.5 | 1 | 1.5 | 2.5 | 2.5 | 3.5 | 4.5 | 5.5 |
| 16 | 0 | 0 | 0 | 0.5 | 1 | 1.5 | 2 | 2.5 |
| 17 | 0 | 0.5 | 1 | 1 | 1.5 | 2 | 2.5 | 2.5 |
| 18 | 1 | 1 | 1.5 | 2 | 2.5 | 3 | 3.5 | 4 |
| 19 | 0 | 0 | 0 | 1 | 1 | 1.5 | 2.5 | 2.5 |
| 20 | 0 | 0 | 0 | 0.5 | 1 | 1.5 | 2 | 2.5 |

Table 4 A-MPR for 64QAM

| QPSK |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Waveform Index | 0 dBi | 1 dBi | 2 dBi | 3 dBi | 4 dBi | 5 dBi | 6 dBi | 7 dBi |
| 1 | 12 | 8.3 | 8.8 | 8.8 | 8.7 | 9.3 | 9.4 | 9.1 |
| 2 | 5 | 15.4 | 15.9 | 16.2 | 16.6 | 17.0 | 17.3 | 17.7 |
| 3 | 3 | 18.0 | 18.6 | 19.0 | 19.5 | 19.9 | 20.2 | 20.7 |
| 4 | 2 | 18.1 | 18.2 | 18.3 | 18.4 | 18.5 | 18.6 | 18.6 |
| 5 | 11.5 | 8.9 | 9.2 | 9.5 | 9.8 | 9.9 | 10.5 | 10.5 |
| 6 | 8.5 | 12.1 | 12.5 | 13.0 | 13.4 | 13.7 | 14.2 | 14.6 |
| 7 | 6.5 | 13.9 | 14.2 | 14.6 | 15.0 | 15.4 | 15.8 | 16.3 |
| 8 | 3 | 17.8 | 18.4 | 18.6 | 19.0 | 19.3 | 19.7 | 20.1 |
| 9 | 2 | 19.2 | 19.7 | 20.2 | 20.7 | 21.0 | 21.2 | 21.3 |
| 10 | 1.5 | 19.8 | 20.2 | 20.5 | 20.8 | 21.0 | 21.1 | 21.2 |
| 11 | 9.5 | 11.6 | 11.8 | 12.3 | 12.4 | 12.7 | 13.0 | 13.4 |
| 12 | 7.5 | 13.6 | 13.9 | 14.4 | 14.7 | 15.2 | 15.6 | 16.0 |
| 13 | 6.5 | 14.3 | 14.6 | 15.1 | 15.3 | 15.7 | 16.2 | 16.6 |
| 14 | 3.5 | 16.9 | 17.3 | 17.5 | 17.9 | 18.2 | 18.5 | 18.7 |
| 15 | 0 | 20.9 | 21.3 | 22.0 | 22.6 | 22.6 | 22.7 | 22.7 |
| 16 | 0 | 22.5 | 23.4 | 23.9 | 24.3 | 24.8 | 25.4 | 25.9 |
| 17 | 0 | 21.9 | 22.5 | 22.9 | 23.5 | 24.1 | 24.7 | 25.2 |
| 18 | 0 | 21.2 | 21.4 | 22.2 | 22.5 | 23.3 | 23.9 | 24.3 |
| 19 | 0 | 22.5 | 23.1 | 23.6 | 24.2 | 24.7 | 24.9 | 25.2 |
| 20 | 0 | 21.9 | 22.4 | 23.0 | 23.6 | 24.1 | 24.6 | 25.2 |

Table 5 MPR for all modulations

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) |  |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| QPSK |  |  |  | - |  | - | ≤ 1.5 |
| 16 QAM |  |  |  | - |  | - | ≤ 2 |
| 64 QAM |  |  |  | - |  | - | ≤ 31 |
| Note 1: This requirement applies for both PSD offset 0dB and 3dB between PSCCH and PSSCH. |  |  |  |  |  |  |  |

Table 6 Total MPR for all modulations

| Waveform Index | 0 dBi | 1 dBi | 2 dBi | 3 dBi | 4 dBi | 5 dBi | 6 dBi | 7 dBi |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15.5 | 16 | 16.5 | 17.5 | 18.5 | 19.5 | 20.5 | 21.5 |
| 2 | 8 | 9 | 9.5 | 10 | 10.5 | 11.5 | 12 | 12.5 |
| 3 | 6 | 6.5 | 7 | 7 | 8 | 8.5 | 9 | 9.5 |
| 4 | 5 | 6 | 7 | 8 | 9 | 10 | 10.5 | 11.5 |
| 5 | 15 | 15.5 | 16.5 | 17 | 17.5 | 18.5 | 19.5 | 20 |
| 6 | 11.5 | 12.5 | 13 | 13.5 | 14 | 14.5 | 15 | 16 |
| 7 | 10 | 10.5 | 11 | 11.5 | 12 | 13 | 13.5 | 14 |
| 8 | 6 | 6.5 | 7 | 7.5 | 8 | 9 | 9.5 | 10 |
| 9 | 5 | 5.5 | 6 | 6.5 | 7 | 7.5 | 8.5 | 9 |
| 10 | 4.5 | 5 | 5.5 | 6 | 6.5 | 7.5 | 8.5 | 9 |
| 11 | 12.5 | 13.5 | 14 | 15 | 15.5 | 16 | 17 | 17.5 |
| 12 | 10.5 | 11 | 11.5 | 12 | 13 | 13.5 | 14 | 14.5 |
| 13 | 9.5 | 10 | 11 | 11.5 | 12 | 12.5 | 13.5 | 14 |
| 14 | 6.5 | 7.5 | 8 | 9 | 9.5 | 10 | 10.5 | 11.5 |
| 15 | 3 | 3.5 | 4 | 4.5 | 4.5 | 5.5 | 6.5 | 7.5 |
| 16 | 1.5 | 1.5 | 2.5 | 3 | 3.5 | 3.5 | 4 | 4.5 |
| 17 | 2.5 | 2.5 | 3 | 3.5 | 3.5 | 4 | 4.5 | 5 |
| 18 | 3 | 3.5 | 4 | 4 | 4.5 | 5 | 5.5 | 6 |
| 19 | 1.5 | 1.5 | 2.5 | 3 | 3.5 | 4 | 4.5 | 5 |
| 20 | 2 | 2.5 | 3 | 3 | 3.5 | 4 | 4.5 | 5 |

# Conclusions

Simulation result for A-MPR of power class 3 with adjacent control and data are provided.

# References

[1] TS 36.101	  Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception

[2] TS 36.104	  Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception

[3] R4-1803355  Way forward on V2X RF Maintenance. Qualcomm Inc., Huawei, LGE, Harman Samsung

[4] R4-1610955  Simulation Assumption for A-MPR of V2V Waveforms. Qualcomm Inc., LGE