3GPP TSG-RAN WG4#86Bis Meeting	R4-1804132

Melbourne, Australia, 16 – 20 April 2018

Agenda item:	6.18.6

Title: 	Discussion on eFeMTC UE demodulation performance requirements

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

From RAN4#86Bis Meeting, the discussion on eFeMTC UE demodulation performance requirements starts in RAN4. In the scope of the Rel.15 eFeMTC WID [1], a number of enhancements were introduced to enable eFeMTC UE with higher velocity, improved latency, improved power consumption, improved spectral efficiency and improved load control, compared to previous Rel.14 FeMTC and/or Rel.13 eMTC.

In this contribution, we share our views on the impact on demodulation/CSI feedback performance requirements caused by these Rel.15 eFeMTC enhancements.

# Discussion

In our view, one of the major impact on the demodulation/CSI feedback performance requirements caused by the Rel.15 eFeMTC enhancements is the newly designed CQI table. For instance, RAN1#92 Meeting (Feb. 2018) discussed downlink 64QAM support and made the following agreements [2]:

- Confirm working assumption:

  - When a UE is configured both with 64QAM and csi-NumRepetitionCE-r13 > 1,

    - A single CQI table covers the range from QPSK with 32 times repetition up to 64QAM without repetition

- The following CQI table is adopted:

| CQI | modulation | code rate x 1024 | repetition |
| --- | --- | --- | --- |
| 0 | Out of range |  |  |
| 1 | QPSK | 56 | 32 |
| 2 | QPSK | 207 | 16 |
| 3 | QPSK | 266 | 4 |
| 4 | QPSK | 195 | 2 |
| 5 | QPSK | 142 | 1 |
| 6 | QPSK | 266 | 1 |
| 7 | QPSK | 453 | 1 |
| 8 | QPSK | 637 | 1 |
| 9 | 16QAM | 423 | 1 |
| 10 | 16QAM | 557 | 1 |
| 11 | 16QAM | 696 | 1 |
| 12 | 16QAM | 845 | 1 |
| 13 | 64QAM | 651 | 1 |
| 14 | 64QAM | 780 | 1 |
| 15 | 64QAM | 888 | 1 |

Obviously, the new CQI table would have impact on the UE demodulation/CSI feedback performance requirements. Moreover, it is stated in the eFeMTC WID [1] that:

“The objective is to specify the following improvements for machine-type communications for BL/CE UEs.”

Due to the difference between BL/CE UE and non-BL/CE UE, and based on the clear definition in TS36.133 about non-BL/CE UE as: “The requirements for non-BL/CE UE are derived assuming: DL and UL category other than Category 0/M1/M2/NB1/NB2, operation in any LTE system bandwidth but with a channel bandwidth not exceeding 20MHz, transmission bandwidth not exceeding 96RB in downlink and 5MHz in uplink, and dual antenna receiver, when in RRC_IDLE mode camped on a cell acquired using SIB1-BR, or in RRC_CONNECTED configured with CE mode A/B.”, we reckon that the scope of the demodulation/CSI feedback requirements for eFeMTC will only apply to BL/CE UE. Therefore, we propose that:

Proposal 1: Due to the scope of eFeMTC WI, the eFeMTC demodulation/CSI feedback requirements will be applied to BL/CE UE only.

Proposal 2: Due to the scope of eFeMTC WI, no new demodulation/CSI feedback requirements will be introduced to non-BL/CE UE in eFeMTC WI.

# Conclusion

In this contribution, we share our views on the impact on demodulation/CSI feedback performance requirements caused by the Rel.15 eFeMTC enhancements.

Proposal 1: Due to the scope of eFeMTC WI, the eFeMTC demodulation/CSI feedback requirements will be applied to BL/CE UE only.

Proposal 2: Due to the scope of eFeMTC WI, no new demodulation/CSI feedback requirements will be introduced to non-BL/CE UE in eFeMTC WI.

# Reference

[1] RP-172811, “Revised WID on Even further enhanced MTC for LTE”, Ericsson, Qualcomm, Dec. 2017

[2] RP-180124, “Status Report: Even further enhanced MTC for LTE”, Ericsson, Qualcomm, ZTE, Intel, Samsung, March 2018