3GPP TSG-RAN WG4 Meeting #86bis	                                                      R4-1804163

Melbourne, AU, 16th – 20th April, 2018

Agenda item:	6.19.4.2

Source:	Intel Corporation

Title:	Discussion on Sustained Data Rate requirements with 1024QAM

Document for:	Discussion

# Introduction

In RAN4#86, the way forward on demodulation and CSI requirements for high capacity stationary wireless link and 1024QAM was agreed upon.

The test purposes were defined as:

- Demodulation performance requirements

  - To verify the demodulation performance using 1024QAM reference channel under the fading channels;

  - To verify the support of peak data rate for the new UE categories, i.e., by defining sustained data rate tests.

  - To verify the functionality and performance for reduced DMRS ports.

- CSI reporting requirement

  - To verify the link adaptation performance following the new CQI/MCS/TBS tables, e.g., CQI definition test.

For the SDR tests, the following test parameters were agreed:

- Common parameters

  - TM3

  - 2 x 2 (for 2-layer transmission per CC, and 4 x 4 (for 4-layer transmission per CC)

    - For 4x4, the definition of 1024QAM capable UE category  should be considered (to address the concern on high SNR, the MCS can be further study)

  - CFI=1

- Tx EVM assumption :

  - Option 1: 2%

  - Option 2: 1.5%

  - RAN4 should consider the feasibility of test equipment

- 85% throughput

- Companies to provide input on per-CC MCS selection for

  - Rank 2 and rank 4

  - Different bandwidth

    - 10MHz, 15MHz, 20MHz

In this contribution we propose test parameters for SDR tests with 1024QAM.

# SDR Test Parameters

## Tx EVM

Figure 1: TX EVM at Max MCS in AWGN

In the agreed WF [1], options for Tx EVM of 1.5% and 2% were to be evaluated for SDR tests. In Figure 1 the effect of Tx EVM for different Tx EVM and practical Rx EVM at max MCS are shown. In 4x4 with 4 layers, the performance degradation is 2dB with 2% Tx EVM compared to 1.5%. Since the 1024QAM capable UE would typically use higher number of Rx, the degradation observed with 4x4 with 4 layers is more relevant. Hence we propose that for SDR requirements with 1024QAM Tx EVM no larger than 1.5% be used.

Proposal #1: Define SDR requirements for 1024QAM with Tx EVM limited to 1.5%

## MCS

The simulation parameters to evaluate performance for SDR tests with 1024QAM are given below:

System Bandwidth: 20MHz

Number of Control Symbols: 1

Transmission Mode: 3

Tx EVM: 1.5%

Rx EVM: Practical value

Channel: AWGN

MCS: 26 [TBS: 125808 for 1 layer; 251640 for 2layers; Reduced TBS in SF#0, 5]

Performance with 2x2, 2 layers with 1024QAM is shown in Figure 2 below:

Figure 2: AWGN performance at Max MCS-2x2

With 2x2, 2 layer and practical EVM, 85% success rate is achieved at around 31.5dB SNR with no impairments considered. For 2x2 2 layer SDR test, max MCS could be used.

Proposal #2: Define 2x2, 2 layer SDR test for 1024QAM with MCS [26]

Performance with 4x4, 4 layers with 1024QAM is shown in Figure 3 below:

Figure 3: AWGN performance at Max MCS-4x4

With practical EVM and Max MCS, 85% success rate is achieved at SNR > 35dB, which is not practical. We recommend using lower MCS for SDR test with 4x4, 4 layers.

Proposal#3: Define 4x4, 4 layer SDR test for 1024QAM with MCS [25]

# Conclusions

In this paper we provide simulation results and propose test parameters for SDR tests with 1024QAM. We have the following proposals:

Proposal #1: Define SDR requirements for 1024QAM with Tx EVM limited to 1.5%

Proposal #2: Define 2x2, 2 layer SDR test for 1024QAM with MCS [26]

Proposal#3: Define 4x4, 4 layer SDR test for 1024QAM with MCS [25]

# References

- R4-1803106 “Way forward on demodulation and CSI requirements for high capacity stationary wireless link and 1024QAM”, Huawei, HiSilicon