3GPP TSG-RAN WG4 Meeting #86bis	R4-1804223

Melbourne, Australia, 16-20 April, 2018

Source: 	Sprint

Title: 					NR CA, EN-DC and NR DC Notation

Agenda Item: 			7.1.3

Document for:	Approval

# 1		Introduction

In previous RAN4 meetings, potential notation has been discussed for CA bandwidth class and intra-band EN-DC notation. A way forward on CA bandwidth class was agreed at RAN4#86. The notation we have been using so far for EN-DC has some drawbacks as the number of CCs increases. This contribution proposes a modified notation scheme that overcomes some of the current shortcomings.

# 2	Discussion

For intra-band contiguous EN-DC, so far the notation DC_(n)71B and DC_(n)41C, DC_(n)41D and DC_(n)41E have been used in the NR TRs. The problem with this notation is that for DC_(n)41D and DC_(n)41E it is not clear how many LTE CCs there are vs. how many NR CCs. This is problematic for several reasons. For one, the total bandwidth could vary widely. If we ignore the fact that the band covers 196 MHz, DC_(n)41E could be three LTE CCs and one NR CC (160 MHz) or one LTE CC and 3 NR CCs (320 MHz). Also, the UE may have the ability to process two LTE CCs but only one NR CC,  but it would have no way to indicate that it does not support one LTE CC and two NR CCs.

In order to improve this situation, the following contiguous EN-DC band classes are proposed:

Table 1: Contiguous EN-DC bandwidth classes

| Contiguous EN-DC bandwidth class | Number of contiguous CC |  |
| --- | --- | --- |
|  | LTE | NR |
| AA | 1 | 1 |
| B | 1 | 1 |
| CA | 2 | 1 |
| AC | 1 | 2 |
| JK | J | K |

Note: The order of the letters is always the same, regardless of deployment of the LTE CCs relative to the NR CCs in frequency. The letter corresponding to the number of LTE CCs is always first.

Proposal 1: Intra-band contiguous EN-DC shall use the bandwidth class notation in Table 1 to indicate the number of contiguous LTE and NR CCs.

Except for Class B, the maximum bandwidth would be equal to the number of LTE CCs times the maximum LTE bandwidth for the band plus the number of NR CCs times the maximum NR bandwidth for the band. For Class B, the maximum aggregated bandwidth would be 20 MHz, as it is in LTE.

The full notation for intra and inter band NR CA, intra and inter band EN-DC and intra and inter band NR DC is shown in Table 2.

Table 2: NR CA, EN-DC  and NR DC band combination notation

| Type |  | Notation | Note |
| --- | --- | --- | --- |
| NR CA | Intraband contiguous | CA_nXC | Already used and follows LTE |
|  | Intraband non-contiguous example 1 | CA_nX(8xA) | new way compared to LTE notation which would be CA_n257A-n257A-n257A-n257A-n257A-n257A-n257A-n257A using band n257 as an example |
|  | Intraband non-contiguous example 2 | CA_nX(2xA-C-A) | new way compared to LTE notation which would be CA_n257A-n257A-n257C-n257A using band n257 as an example |
|  | Interband CA | CA_nXA-nYA | Already used and follows LTE |
| EN-DC | Intraband contiguous | DC_(n)XAA | 1 LTE, 1 NR, max BW ≤ max LTE BW + max NR BW |
|  |  | DC_(n)XB | Already used 1 LTE, 1 NR, max BW ≤ 20 MHz |
|  |  | DC_(n)XJK | J LTE, K NR |
|  | Intraband non-contiguous | DC_XA_nXA | Non-contiguous EN-DC with 2 sub-blocks |
|  |  | DC_XC_nXC | Non-contiguous EN-DC with 2 contiguous LTE sub blocks and 2 contiguous NR sub-blocks |
|  |  | DC_X(2xA)_nX(3xA) | Non-contiguous EN-DC with 2 non-contiguous LTE sub blocks and 3 non-contiguous NR sub-blocks |
| NR DC | Interband | DC_nXA-nYA | Follows LTE |

Proposal 2: Intra-band contiguous NR CA, EN-DC and NR DC shall use the band combination notation in Table 2.

# 3	Conclusion

In this contribution we propose notation for bandwidth class and

Proposal 1: Intra-band contiguous EN-DC shall use the bandwidth class notation in Table 1 to indicate the number of contiguous LTE and NR CCs.

Proposal 2: Intra-band contiguous NR CA, EN-DC and NR DC shall use the band combination notation in Table 2.

# 4	References

[1] R4-1803374, WF for NR BW Class, Nokia, Nokia Shanghai Bell