3GPP TSG-RAN WG4 Meeting #86-Bis 				R4-1804409

Melbourne, AU, 16th – 20th April, 2018

Source:	Verizon, Ericsson, Qualcomm, Samsung, Nokia

Title:	TP for TR 37.863-01-01 NR Intra-band n261 CA

Agenda item:			7.1.3.7

Document for:	Approval

# 1. Introduction

Rel-15 LTE-NR CA combinations are listed in the latest WID [1]. This contribution is a text proposal for TR 37.863-01-01 to add NR intra-band n261 CA.

# 2. Text Proposal

----- Unchanged sections omitted -----

## 6.X	NR intra-band contiguous n261 CA

### 6.X.1	Operating bands for NR

Table 6.X.1-1: NR band combination of n261 band

| NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |

### 6.X.2	Channel bandwidths per operating band for DC

### Table 6.X.2-1: NR CA configurations and bandwidth combination fallback group defined for non-contiguous intra-band CA

|  |  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR configuration | Uplink CA configurations | SCS | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Fallback group |
|  |  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_n261A-n261A | - | 60 | 50, 100, 200 | 50, 100, 200 |  |  |  | 400 | 0 |
|  |  | 120 | 50, 100, 200, 400 | 50, 100, 200, 400 |  |  |  | 800 | 0 |
| CA_n261A-n261A-n261A | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  | 600 | 0 |
|  |  | 120 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 |  |  | 1200 | 0 |
| CA_n261A-n261A-n261A-n261A | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  | 800 | 0 |
|  |  | 120 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 |  | 1600 | 0 |

### Table 6.X.2-2: NR CA configurations and bandwidth combination fallback group defined for non-contiguous intra-band CA

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations (NOTE 1) | Component carriers in order of increasing carrier frequency |  |  |  |  |  | Maximum aggregated bandwidth [MHz] |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |
| CA_n261D-n261G | - | See CA_n261D Bandwidth Combination Fallback group 2 in Table 6.X.2-1 |  | See CA_n261G Bandwidth Combination Fallback group 3 in Table 6.X.2-2 |  |  |  | 600 |
|  |  | See CA_n261G Bandwidth Combination Fallback group 3 in Table 6.X.2-2 |  |  |  | See CA_n261D Bandwidth Combination Fallback group 2 in Table 6.X.2-1 |  |  |
| CA_n261D-n261H | - | See CA_n261D Bandwidth Combination Fallback group 2 in Table 6.X.2-1 |  | See CA_n261H Bandwidth Combination Fallback group 3 in Table 6.X.2-2 |  |  |  | 700 |
|  |  | See CA_n261H Bandwidth Combination Fallback group 3 in Table 6.X.2-2 |  |  |  | See CA_n261D Bandwidth Combination Fallback group 2 in Table 6.X.2-1 |  |  |
| CA_n261D-n261I | - | See CA_n261D Bandwidth Combination Fallback group 2 in Table 6.X.2-1 |  | See CA_n261I Bandwidth Combination Fallback group 3 in Table 6.X.2-2 |  |  |  | 800 |
|  |  | See CA_n261I Bandwidth Combination Fallback group 3 in Table 6.X.2-2 |  |  |  | See CA_n261D Bandwidth Combination Fallback group 2 in Table 6.X.2-1 |  |  |
| CA_n261D-n261O | - | See CA_n261D Bandwidth Combination Fallback group 2 in Table 6.X.2-1 |  | See CA_n261O Bandwidth Combination Fallback group 4 in Table 6.X.2-1 |  |  |  | 600 |
|  |  | See CA_n261O Bandwidth Combination Fallback group 4 in Table 6.X.2-1 |  |  |  | See CA_n261D Bandwidth Combination Fallback group 2 in Table 6.X.2-1 |  |  |
| CA_n261D-n261P | - | See CA_n261D Bandwidth Combination Fallback group 2 in Table 6.X.2-1 |  | See CA_n261P Bandwidth Combination Fallback group 4 in Table 6.X.2-1 |  |  |  | 700 |
|  |  | See CA_n261P Bandwidth Combination Fallback group 4 in Table 6.X.2-1 |  |  |  | See CA_n261D Bandwidth Combination Fallback group 2 in Table 6.X.2-1 |  |  |
| CA_n261D-n261Q | - | See CA_n261D Bandwidth Combination Fallback group 2 in Table 6.X.2-1 |  | See CA_n261Q Bandwidth Combination Fallback group 4 in Table 6.X.2-1 |  |  |  | 800 |
|  |  | See CA_n261Q Bandwidth Combination Fallback group 4 in Table 6.X.2-1 |  |  |  | See CA_n261D Bandwidth Combination Fallback group 2 in Table 6.X.2-1 |  |  |
| CA_n261E-n261O | - | See CA_n261E Bandwidth Combination Fallback group 2 in Table 6.X.2-1 |  | See CA_n261O Bandwidth Combination Fallback group 4 in Table 6.X.2-1 |  |  |  | 800 |
|  |  | See CA_n261O Bandwidth Combination Fallback group 4 in Table 6.X.2-1 |  |  |  | See CA_n261E Bandwidth Combination Fallback group 2 in Table 6.X.2-1 |  |  |
| CA_n261E-n261P | - | See CA_n261E Bandwidth Combination Fallback group 2 in Table 6.X.2-1 |  | See CA_n261P Bandwidth Combination Fallback group 4 in Table 6.X.2-1 |  |  |  | 900 |
|  |  | See CA_n261P Bandwidth Combination Fallback group 4 in Table 6.X.2-1 |  |  |  | See CA_n261E Bandwidth Combination Fallback group 2 in Table 6.X.2-1 |  |  |
| CA_n261E-n261Q | - | See CA_n261E Bandwidth Combination Fallback group 2 in Table 6.X.2-1 |  | See CA_n261Q Bandwidth Combination Fallback group 4 in Table 6.X.2-1 |  |  |  | 1000 |
|  |  | See CA_n261Q Bandwidth Combination Fallback group 4 in Table 6.X.2-1 |  |  |  | See CA_n261E Bandwidth Combination Fallback group 2 in Table 6.X.2-1 |  |  |

----- Unchanged sections omitted -----

# Reference

[1]	RP-180536, “Revision of WI: New Radio Access Technology”, NTT DOCOMO, INC.