3GPP TSG-RAN WG4 #86bis	R4-1804659

Melbourne, AU, April 16th – 20th, 2018

Agenda item:	7.4.8.9

Source: 	Qualcomm Incorporated

Title: 	UE power back-off for protection of passive services

Document for:	Approval

# Introduction

In this contribution we focus on the additional spurious emission requirements needed to protect passive services operating in millimiter (mmW) frequency ranges. In this paper we refine our analysis presented in [1] and we estimate the amount of power reduction needed to meet -8dBm/200MHz emission level over the passive bands.

# Discussion

Regulators around the world are defining emission requirements to protect satellite passive services (e.g. EESS passive). The following list shows the bands of operation of passive services and corresponding reference bandwidth to be studies:

23.6-24 GHz -> 200MHz

31.3-31.8 GHz -> 200MHz

50.2-50.4 GHz -> 200MHz

52.6-54.25 GHz -> 100MHz

86-92 GHz -> 100MHz

CEPT PT1 finalized an ECC decision on the 26GHz band which is not under public consultation [2]. As shown in Table 1, the draft decision specifies a protection level in the range of -38/-40 dBW/200MHz.

MFCN terminal station maximum emissions into the 23.6-24.0 GHz band [2].

| Frequency range | Maximum emissions (see note) | Measurement bandwidth |
| --- | --- | --- |
| 23.6-24.0 GHz | [-38/-40] dBW | 200 MHz |

Note: This level requirement applies for terminal station for all foreseen modes of operation (i.e. maximum in-band power, electrical pointing, carrier configurations)

[Editor’s note: depending on information from industry about second harmonic, similar level might be necessary in the range 50.2-54.25 GHz]

A final value of within the proposed range will be decided in next PT1 meeting, which is happening the same week as RAN4 #86bis.

Several RAN4 contributions already addressed this issue [2][4][5][6][7], in the following we will run a more detailed analysis considering the upper limit of the range discussed by CEPT, i.e. -8dBW/200MHz. This analysis will be further refined once a final decision about the requirement is made in CEPT PT1.

# Impact of additional spurious emission requirement: methodology and results

Based on the recent Way Forward on MPR for FR2 [9], we assume the following waveform as reference (0dB MPR waveform): BW = 100MHz, SCS=60KHz, DFT-S-OFDM QPSK, 128RB0.

In our analysis, we considered different waveforms in terms of aggregated bandwidth spanning from 50MHz to 800MHz. The channel allocation always starts at band n258 lower edge, i.e. 24.25GHz. We assumed DFT-S-OFDM since this is the waveform requiring less MPR according to our analysis in [10]. Table 1 summarized the total amount of back-off needed compared to the 0dB MPR waveform. The table shows both contiguous and non-contiguous allocation cases. As we described in [10], for non-contiguous RB allocation, if the allocation ratio A is less than 2%, the total back-off is waveform and bandwidth agnostic as far as IM3 or Triple Beat products fall into passive services frequency region.

Table 1. Total back-off needed to meet -8dBm/200MHz in 23.6-24GHz.

|  |  |  |  | Contiguous RB allocation | Non-Contiguous RB allocation, A<=0.02 |
| --- | --- | --- | --- | --- | --- |
| Modulation | Component Carriers [MHz] | Aggregated BW [MHz] | SCS [KHz] | Total Back-off needed [dB] |  |
| QPSK | 50 | 50 | 60 | 0 | NA |
|  | 200 | 200 | 60 | 0 | NA |
|  | 200+200 | 400 | 60 | 2 | 4.5 |
|  | 400 | 400 | 120 | 2 | NA |
|  | 400+400 | 800 | 120 | 3.5 | 4.5 |

Observation: for band n258 the amount of power back-off needed to meet the -8dBm/200MHz requirement to protect 23.6-24GHz passive band is up to 4.5dB. For band n257 and band n261 no power back-off is needed to meet the same requirement.

# Conclusions

In this contribution, we analyze the additional requirements of -8dBm/200MHz needed to protect passive bands. Based on the outcome of our analysis, we made the following observation:

Observation: for band n258 the amount of power back-off needed to meet the -8dBm/200MHz requirement to protect 23.6-24GHz passive band is up to 4.5dB. For band n257 and band n261 no power back-off is needed to meet the same requirement.

This analysis will be further refined once a final decision about the requirement is made in CEPT PT1.

# Reference

- R4-1714048, “On the additional spurious emission limit to protect passive services”.

- Draft ECC Decision on “Harmonised technical conditions for Mobile/Fixed Communications Networks (MFCN) in the band 24.25-27.5 GHz”.

- R4-1710084, "LS on Unwanted emissions of IMT-2020" to ITU-R WP5D.

- R4-1707701, “Feasibility of improved spurious emissions in NR mmWave UE”, Skyworks Solutions Inc.

- R4-1708125, “On spurious emission limits for mm-wave bands”, Ericsson.

- R4-1708450, “General observations on UE emissions in mmW”, Qualcomm Incorporated.

- R4-1709330, “NR UE above 24GHz Spurious Emissions and Protected Bands”, Skyworks Solutions Inc.

- R4-1709408, “On spurious emissions requirement for mmWave NR”, Intel Corporation.

- R4-1803263, “WF on MPR for FR2”, Intel, Qualcomm Incorporated.

- R4-1805181, “FR2 CA MPR”, Qualcomm Incorporated.