3GPP TSG-RAN WG4 #86bis	R4-1804991

Melbourne, Australia 16th April –20th April 2018

Source:	Ericsson

Title:	TP to draft CR 37.843 CATR Measurement uncertainty budget for ACLR

Agenda item:	6.26.3.1.4

Document for:	Approval

# Introduction

This contribution presents an uncertainty budget for ACLR measurement in a Compact Antenna Test Range which is a method for conformance testing in [2].

# Discussion

Description of uncertainty contribution can be found in [3].

From framework on uncertainty budget for ACLR in [4] and measurement uncertainty budget [5] is the basis for following contribution.

Note that the measurement uncertainty proposed below is for one measurement point, and the TRP method comes with a separate uncertainty which is not captured in this proposal.

# Conclusion

This contribution presents measurement uncertainty budget for ACLR measurement in a Compact Antenna Test Range.

# References

[1] 	TS 37.105, Active Antenna System (AAS) Base Station (BS) transmission and reception

[2]	TS 37.145-2, Active Antenna System (AAS) Base Station (BS) conformance testing; Part 2: radiated conformance testing

[3] 	TR 37.842, Evolved Universal Terrestrial Radio Access (E-UTRA) and Universal Terrestrial Radio Access (UTRA; Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)

[4]	R4-1705386, “Framework on Uncertainty Budget for ACLR”, Ericsson

[5]	R4-1713564, “Measurement uncertainty budget for ACLR for CATR”, Ericsson

[Start of Text Proposal]

10.2.3.2.1.4	Measurement Budget Format

Table 10.2.3.2.1.4-1: CATR uncertainty budget format for ACLR measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Misalignment DUT & pointing error | B2-1 |
| 5 | QZ ripple DUT | B2-5 |
| Stage 1: Calibration measurement |  |  |
| 6 | Uncertainty of network analyser | E |
| 7 | Mismatch of receiver chain | B2-7 |
| 8 | Insertion loss variation of receiver chain | B2-8 |

### 10.2.3.2.1.5	Uncertainty Assessment

Table 10.2.3.2.1.5-1: CATR uncertainty budget format for ACLR measurement

| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3 GHz < f ≦ 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3 GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |
| 5 | QZ ripple with DUT | 0.0928 | 0.0928 | Normal | 1 | 1 | 0.0928 | 0.0928 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |
| 6 | Network Analyzer | 0.13 | 0.20 | Normal | 1 | 1 | 0.13 | 0.20 |
| 7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.127 | 0.325 | U-shaped | √2 | 1 | 0.09 | 0.23 |
| 8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.10 | 0.10 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  |  |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  |  |

[End of Text Proposal]