3GPP TSG-RAN WG4 #86-bis	R4-1805087

Melbourne, Australia, 16th – 20th April 2018

Source:	MVG Industries

Title:	TP to Draft CR 37.843 - OTA ACLR measurement in a Near Field Test Range

Agenda item:	6.26.3.1.4

Document for:	Approval

# Introduction

During 3GPP RAN4 #86, a contribution highlighting the test procedures for ACLR type of measurements in Near Field Test Range was provided [1]. ACLR is based on in-band TRP measurement.

This contribution is providing the test procedure and MU for ACLR measurement in Near Field test Range.

# Discussion

Near Field is a testing methodology which can be used for radiated transmit power. Basically, ACLR is defined OTA as the ratio between the power measured in the channel bandwidth and the power measured in the adjacent channel bandwidth. It means ACLR is based on TRP type of measurement. The number of discrete points to be measured around the sphere surrounding the DUT has been defined as TRP requirement and highlighted in clause 10.4 [2].

It has to be noted that the TRP has been calculated from the power density measured in Near Field. None NF2FF transform will be applied to the power density measured in Near Field. This is because TRP is obtained from integration of a power density over a closed spherical surface based on the following formula:

For different measurement distances, the radiated power is integrated over different size spherical surfaces, but since there are no losses in free space, the same amount of total power will be “collected” independent of the distance (conservation of power). The exact shape of the radiation pattern would be different at shorter distances, but the integral over the entire spherical surface will be the same. The above is to say that, TRP is independent of the measurement distance.

TRP accuracy will be impacted by the sampling grid so that an uncertainty term for TRP measurement must be added to the EIRP MU (TR37.842 Table 10.3.1.1.5.3-1).

# Proposal

<< Unchanged sections omitted >>

<< Start of proposal >>

## 10.3.1.1 OTA ACLR

## 10.3.1.1.3 Near Field

### 10.3.1.1.3.1 Description

The near field measurement techniques consist in measuring amplitude and phase of the modulated signal at the AAS BS under test, on some specific surfaces such as planar, cylindrical, and spherical. Each of the near field test methods can be implemented by one or more mechanical rotations of the probe and/or AAS BS under test as shown in figure 10.3.2.1.4.1-1. One or more mechanical movement can be substitute by a probe array. All the scanning methods will need an RF transmit and receive system equipped with an automated scanning, a data collection and control system, and computerized analysis ability.

# Figure 10.2.2.3.3.1-1: Probe/scanner near field systems: spherical, cylindrical and planar

NOTE:	Although there are three methods available for obtaining the near field data, the spherical method is used as a working example.

In case the radiated field is sampled on a sphere surrounding completely the AAS BS under test, the 3D full sphere EIS values can be measured in near field when the AAS BS is receiving a defined modulated signal. The near field measurement techniques would imply the use of mathematical artefact, NF to FF transform in order to have the EIS value the in far field. The near field to far field transform is based on the well-known Huygens-Fresnel principle. The spherical modal wave expansion is the implicit application of the Huygens principle. A direct solution of the Helmholtz equations is found by applying boundary conditions on the surface S at infinity. From the tangential fields over the surface, the modal coefficients can be determined using the orthogonality of the modal expansion.  Based on this formulation, the near field sampling criteria does play an important role.

### 10.3.1.1.3.2 Procedure

Stage 1 – Calibration:

Calibration accounts for the various factors affecting the measurements of the EIRP. These factors include components such as range length path loss, cable losses, gain of the receiving antenna, etc. Each measured data point for both radiated power and radiated sensitivity is transformed from a relative value in dB to an absolute value in dBm. For doing that the total path loss from the DUT to the measurement receiver, named L path loss is calibrated out. The calibration measurement is usually done by using a reference antenna with known gain. This approach is based on the so called gain-comparison method [3]. Figure 10.3.1.1.3.2-1 shows the typical configuration for measuring path loss.

# Figure 10.3.1.1.3.2-1: Typical Lpath loss measurement configuration

The Lpath loss can be determined from the power into the reference antenna by adding the gain of the reference antenna:

# Piso = Pref + GrefPIsotropic Radiator=Preference antenna+Greference antenna

so that:

# Lpath loss = Pref + Gref - PtestLpath loss=Preference antenna+ Greference antenna- Ptest equipment

In order to determine Pref, a cable reference measurement is performed in order to calibrate out the A, and B pathsLtest equipment-receiver. Assuming that the power at the source is fixed, it can be showed that:

# Pref-Ptes=Prec'-Prec

Where Prec Preceiver and Prec' are the power measured at the receiver during the calibration measurement with the reference antenna and the power measured at the receiver during the cable reference measurement respectively. Lpath loss Lpath loss is then given by:

Lpath loss = Gref + Prec' - Prec

Stage 2 - Measurement:

The testing procedure consists of the following steps:

- Measure the Near Field pattern of the power density within the signal bandwidth – wanted signal

  - Power is measured Near Field – no NF to FF transform is applied to

  - Full sphere power is measured with using a defined sampling grid (5deg in both azimuth and elevation)

- Calculate TRP by using the formula in [2]

- Measure the Near Field emission power density in the neighbouring channel bandwidth (+/- 5MHz, +/-10MHz)

  - Power is measured Near Field – no NF to FF transform is applied to

  - Full sphere power is measured with using a defined sampling grid (5deg in both azimuth and elevation)

- Calculate TRP by using the formula in [2]

- Calculate ACLR as TRP signal power / TRP emission power

### 10.3.1.1.3.3 Uncertainty budget format

Table 10.3.1.1.3.3-1: Near Field Test Range Uncertainty contributors for eAAS OTA ACLR measurement

| UID | Description of uncertainty contribution | Details in paragraph |
| --- | --- | --- |
| Stage 2: OTA ACLR measurement |  |  |
| 1 | Axes Intersection | 10.3.1.1.3.3-1 |
| 2 | Axes Orthogonality | 10.3.1.1.3.3-2 |
| 3 | Horizontal Pointing | 10.3.1.1.3.3-3 |
| 4 | Probe Vertical Position | 10.3.1.1.3.3-4 |
| 5 | Probe H/V pointing | 10.3.1.1.3.3-5 |
| 6 | Measurement Distance | 10.3.1.1.3.3-6 |
| 7 | Amplitude and Phase Drift | 10.3.1.1.3.3-7 |
| 8 | Amplitude and Phase Noise | 10.3.1.1.3.3-8 |
| 9 | Leakage and Crosstalk | 10.3.1.1.3.3-9 |
| 10 | Amplitude Non-Linearity | 10.3.1.1.3.3-10 |
| 11 | Amplitude and Phase Shift in rotary joints | 10.3.1.1.3.3-11 |
| 12 | Channel Balance Amplitude and Phase | 10.3.1.1.3.3-12 |
| 13 | Probe Polarization Amplitude and Phase | 10.3.1.1.3.3-13 |
| 14 | Probe Pattern Knowledge | 10.3.1.1.3.3-14 |
| 15 | Multiple Reflections | 10.3.1.1.3.3-15 |
| 16 | Room Scattering | 10.3.1.1.3.3-16 |
| 17 | DUT support Scattering | 10.3.1.1.3.3-17 |
| 18 | Scan Area Truncation | 10.3.1.1.3.3-18 |
| 19 | Sampling Point Offset | 10.3.1.1.3.3-19 |
| 20 | Spherical Mode Truncation | 10.3.1.1.3.3-20 |
| 21 | Positioning | 10.3.1.1.3.3-21 |
| 22 | Probe Array Uniformity | 10.3.1.1.3.3-22 |
| 23 | Mismatch of receiver chain (i.e. between receiving antenna and measurement receiver) | 10.3.1.1.3.3-23 |
| 24 | Insertion loss of receiver chain | 10.3.1.1.3.3-24 |
| 25 | Uncertainty of the absolute gain of the probe antenna | 10.3.1.1.3.3-25 |
| 26 | RF Measurement Equipment | 10.3.1.1.3.3-26 |
| 27 | Measurement repeatability - Positioning Repeatability | 10.3.1.1.3.3-27 |
| 28 | Sampling Grid | 10.3.1.1.3.3-28 |
| Stage 1: Calibration Measurement |  |  |
| 29 | Uncertainty of Vector Network Analyzer | 10.3.1.1.3.3-29 |
| 30 | Mismatch of receiver chain | 10.3.1.1.3.3-30 |
| 31 | Insertion Loss of receiver chain | 10.3.1.1.3.3-31 |
| 32 | Mismatch in the connection of the calibration antenna | 10.3.1.1.3.3-32 |
| 33 | Influence of the calibration antenna feed cable | 10.3.1.1.3.3-33 |
| 34 | Influence of the probe antenna cable | 10.3.1.1.3.3-34 |
| 35 | Uncertainty of the absolute gain of the reference antenna | 10.3.1.1.3.3-35 |
| 36 | Short term repeatability | 10.3.1.1.3.3-36 |

# 10.3.1.1.3.3-1	Axes Intersection

This is a mechanical uncertainty term and aim to find the uncertainty related with the lateral displacement between the horizontal and vertical axes of the AAS BS positioner. This can result in sampling the field on a non-ideal sphere. This uncertainty is assumed to have a Gaussian distribution.

# 10.3.1.1.3.3-2	Axes Orthogonality

The difference from 90° of the angle between the horizontal and vertical axes also results in sampling the field on a non-ideal sphere. This uncertainty is assumed to have a Gaussian distribution.

# 10.3.1.1.3.3-3	Horizontal Pointing

The horizontal mispointing of the horizontal axis to the probe reference point for Theta=0° also results in sampling the field on a non-ideal sphere. This uncertainty is assumed to have a Gaussian distribution.

# 10.3.1.1.3.3-4	Probe Vertical position

The vertical displacement of the probe reference point from the horizontal axis results in sampling the field on a non-ideal sphere. This uncertainty is assumed to have a Gaussian distribution.

# 10.3.1.1.3.3-5	Probe Horizontal/Vertical pointing

The horizontal or vertical mispointing of the probe z-axis from the intersection point of the horizontal/vertical axis. This uncertainty is assumed to have a Gaussian distribution.

# 10.3.1.1.3.3-6	Measurement distance

This is the knowledge of the distance between the intersection point of the horizontal and vertical axis and probe reference point. This uncertainty is assumed to have a Gaussian distribution.

# 10.3.1.1.3.3-7	Amplitude and phase drift

The system drift due to temperature variations the signal at AAS BS location to drift in amplitude and phase. This uncertainty is assumed to have a Gaussian distribution.

# 10.3.1.1.3.3-8	Amplitude and phase noise

This uncertainty is due to the noise level of the test range so that the S/N ratio should be determined or measured at the AAS BS location. The noise level is usually measured with a Spectrum Analyzer. This uncertainty is assumed to have a Gaussian distribution.

# 10.3.1.1.3.3-9	Leakage and Crosstalk

This uncertainty can be addressed by measurements on the actual system setup. The leakage and crosstalk cannot be separated from the random amplitude and phase errors so that the relative importance should be determined. This uncertainty is assumed to have a Gaussian distribution.

# 10.3.1.1.3.3-10	Amplitude non-Linearity

This uncertainty is the linearity of the receiver used for the measurement. It can be taken from the data sheet of the receiver.

# 10.3.1.1.3.3-11	Amplitude and phase shift in rotary joint

This uncertainty is due to the variation of the rotary joint. It can be measured and is assumed to have a Gaussian distribution.

# 10.3.1.1.3.3-12	Channel balance amplitude and phase

This uncertainty is relevant for systems which are using dual polarized probes and polarization switches. The amplitude and phase difference between two signal channels of the receiver includes the difference between the probe ports, difference between the channels of the polarization switch, connecting cables and reflection coefficients. This uncertainty is assumed to have a Gaussian distribution.

# 10.3.1.1.3.3-13	Probe polarization amplitude and phase

The amplitude and phase of the probe polarization coefficients should be measured. This uncertainty is assumed to have a Gaussian distribution.

# 10.3.1.1.3.3-14	Probe pattern knowledge

The probe(s) pattern(s) is assumed to be known so that the AAS BS measurement in near field can be corrected when performing the near field to far field transform. There is no direct dependence between the AAS BS pattern and the probe pattern in near field measurements. This uncertainty is assumed to have a Gaussian distribution.

# 10.3.1.1.3.3-15	Multiple reflections

The multiple reflections occur when a portion of the transmitted signal is reflected form the receiving antenna back to the transmitting antenna and re-reflected by the transmitting antenna back to the receiving antenna. This uncertainty can be determined by multiple measurements of the AAS BS when at different distance from the probes. This uncertainty is assumed to have a Gaussian distribution.

# 10.3.1.1.3.3-16	Room scattering

As for the multiple reflections, a portion of the transmitted signal is reflected by either the absorbers or other structures in the measurement anechoic chamber before being received by the receiving antenna. This effect can be isolated from the multiple reflections by testing the AAS BS in different positions, separated by λ/4 with respect to the anechoic chamber and comparing these measurements with the reference. This uncertainty is assumed to have a Gaussian distribution.

# 10.3.1.1.3.3-17	DUT support scattering

This is the uncertainty due to the AAS BS supporting structure on the signal level. This uncertainty is assumed to have a Gaussian distribution.

# 10.3.1.1.3.3-18	Scan area truncation

This uncertainty does affect this near field measurement. It can be addressed by comparing the measurement result when scanning the full area. This uncertainty is assumed to have a Gaussian distribution.

# 10.3.1.1.3.3-19	Sampling point offset

This uncertainty has an influence in near field and far field. It is assumed to have a Gaussian distribution.

# 10.3.1.1.3.3-20	Mode truncation

The measured near field is expanded using a finite set of spherical modes. The number of modes is linked to number of samples. The filtering effect generated by the finite number of modes can improve measurement results by removing signals from outside the physical area of the AAS BS. Care should be taken in order to make sure the removed signals are not from the AAS BSitself. This uncertainty is usually negligible.

# 10.3.1.1.3.3-21	Positioning

The relative position of the probe array is not ideal. This uncertainty is assumed to have a rectangular distribution.

# 10.3.1.1.3.3-22	Probe array uniformity

This is the uncertainty due to the fact that different probes are used for each physical position. Different probes have different radiation patterns. Generally, the probe array is calibrated so that the uniformity of the probes is achieved.

# 10.3.1.1.3.3-23	Mismatch of receiver chain

If the same chain configuration (including the measurement receiver; the probe antenna and other elements) is used in both stages, the uncertainty is considered systematic and constant  0.00 dB value.

If it is not the case, this uncertainty contribution has to be taken into account and should be measured or determined by the method described in [14]. This uncertainty is assumed to have a U-shaped distribution

# 10.3.1.1.3.3-24	Insertion loss of receiver chain

It is composed of the following:

-	Insertion loss of the probe antenna cable.

-	Insertion loss of the probe antenna attenuator (if used).

-	Insertion loss of RF relays (if used).

If the same chain configuration is used for measurement and calibration, the uncertainty due to the above components is considered systematic and constant  0.00 dB value. This uncertainty is assumed to have a Gaussian distribution.

# 10.3.1.1.3.3-25	Uncertainty of the absolute gain of the probe antenna

This uncertainty appears in the both stages and it is thus considered systematic and constant 0.00 dB value.

# 10.3.1.1.3.3-26	RF Measurement Equipment

# This uncertainty is related to the test equipment which receives the signal.  All uncertainties that pertain to the measurement being performed shall be included.

# 10.3.1.1.3.3-27	Measurement Repeatability - Positioning Repeatability

This uncertainty is due to the repositioning of the AAS BS in the test setup. It can be addressed by repeating the corresponding measurement 5 times. Calculate the standard deviation of the metric obtained and use that as the measurement uncertainty. For tests that require multiple setups, the worst-case standard deviation is used. This uncertainty is assumed to have a Gaussian distribution.

# 10.3.1.1.3.3-28	Sampling Grid

This uncertainty is due to using a finite angular sampling when measuring the 3D pattern.  This uncertainty is an estimation of the difference in the TRP result when performing the measurement with a finite angular spacing vs an infinitely small angular spacing. In figure 10.3.1.1.3.3-28-1, the TRP uncertainty versus sampling grid is reported for a typical AAS BS beam pattern:

# Figure 10.3.1.1.3.3-28-1: TRP uncertainty versus sampling grid for a typical AAS BS beam pattern

# 10.3.1.1.3.3-29	Uncertainty of Vector Network Analyzer

This uncertainty includes the all uncertainties involved in the S21 measurement with a vector network analyzer, and will be calculated from the manufacturer's data sheet. This uncertainty is assumed to have a uniform distribution. This uncertainty value can be found in Annex E and was a result of compromised value in order to align all test methods having this uncertainty contribution.

# 10.3.1.1.3.3-30	Mismatch of receiver chain

If the same chain configuration (including the measurement receiver; the probe antenna and other elements) is used in both stages, the uncertainty is considered systematic and constant  0.00 dB value.

If it is not the case, each uncertainty contribution has to be taken into account and should be measured or determined  and then taking the total of all non-zero mismatch uncertainty contribution from all parts by root-sum-squares (RSS) method. This uncertainty is assumed to have a Gaussian distribution.

# 10.3.1.1.3.3-31	Insertion loss of receiver chain

If the same chain configuration is used for measurement and calibration, the uncertainty due to the above components is considered systematic and constant  0.00 dB value. This uncertainty is assumed to have a Gaussian distribution.

# 10.3.1.1.3.3-32	Mismatch in the connection of the calibration antenna

This is the uncertainty from the mismatch in the connection between the system coax cable and the calibration antenna. This uncertainty is from the mismatch between the cable and the reference antenna that is used for calibration. It is determined by the S11 of the reference antenna and the S11 of the cable to which the antenna is connected i.e if using an SGH antenna for calibration and 10dB pad is inserted on the cable connecting to the antenna this uncertainty contribution can be considered negligible. This uncertainty is assumed to have a U-shaped distribution.

# 10.3.1.1.3.3-33	Influence of the calibration antenna feed cable

This uncertainty is due to the impact of the feeding cable on the radiation properties of the calibration antenna. In case of using either a standard horn or standard gain horn, the impact of the cable is to be considered negligible thus the uncertainty  0.00 dB value. In case of using a dipole-like antenna, the uncertainty should be addressed by measuring this impact. This uncertainty is assumed to have a Gaussian distribution.

# 10.3.1.1.3.3-34	Influence of the probe antenna cable

If the same chain configuration is used for measurement and calibration, the uncertainty due to the above components is considered systematic and constant  0.00 dB value. This uncertainty is assumed to have a Gaussian distribution.

# 10.3.1.1.3.3-35	Uncertainty of the absolute gain of the calibration antenna

The calibration antenna only appears in Stage 2. Therefore, the gain/efficiency uncertainty has to be taken into account. This uncertainty will be calculated from the manufacturer's calibration certificate and is assumed to have a uniform distribution, unless otherwise informed  This uncertainty value can be found in Annex E and was a result of compromised value in order to align all test methods having this uncertainty contribution.

# 10.3.1.1.3.3-36	Short term repeatability

It can be addressed by performing a repeatability test of the calibration antenna. This uncertainty is assumed to have a Gaussian distribution.

### 10.3.1.1.3.4 Uncertainty assessment

Table 10.3.1.1.3.4-1: Near Field Test Range Uncertainty assessment for eAAS OTA ACLR measurement

| UID | Uncertainty source |  | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 1: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Axes Intersection |  | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 2 | Axes Orthogonality |  | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 3 | Horizontal Pointing |  | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 4 | Probe Vertical Position |  | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 5 | Probe H/V pointing |  | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 6 | Measurement Distance |  | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 7 | Amplitude and Phase Drift |  | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 8 | Amplitude and Phase Noise |  | 0.02 | 0.02 | Gaussian | 1.00 | 1 | 0.02 | 0.02 |
| 9 | Leakage and Crosstalk |  | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 10 | Amplitude Non-Linearity |  | 0.04 | 0.04 | Gaussian | 1.00 | 1 | 0.04 | 0.04 |
| 11 | Amplitude and Phase Shift in rotary joints |  | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 12 | Channel Balance Amplitude and Phase |  | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 13 | Probe Polarization Amplitude and Phase |  | 0.0001 | 0.0001 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 14 | Probe Pattern Knowledge |  | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 15 | Multiple Reflections |  | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 16 | Room Scattering |  | 0.09 | 0.09 | Gaussian | 1.00 | 1 | 0.09 | 0.09 |
| 17 | DUT support Scattering |  | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 18 | Scan Area Truncation |  | 0.003 | 0.003 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 19 | Sampling Point Offset |  | 0.0058 | 0.0058 | Gaussian | 1.00 | 1 | 0.01 | 0.01 |
| 20 | Spherical Mode Truncation |  | 0.015 | 0.015 | Gaussian | 1.00 | 1 | 0.02 | 0.02 |
| 21 | Positioning |  | 0.03 | 0.03 | Rectangular | 1.73 | 1 | 0.02 | 0.02 |
| 22 | Probe Array Uniformity |  | 0.055 | 0.055 | Gaussian | 1.00 | 1 | 0.06 | 0.06 |
| 23 | Mismatch of receiver chain |  | 0.284 | 0.284 | U-Shaped | 1.41 | 1 | 0.20 | 0.20 |
| 24 | Insertion loss of receiver chain |  | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 25 | Uncertainty of the absolute gain of the probe antenna |  | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 26 | RF measurement equipment |  | TBD | TBD | Rectangular | 1.73 | 1 | TBD | TBD |
| 27 | Measurement repeatability – positioning repeatability |  | 0.15 | 0.15 | Gaussian | 1.00 | 1 | 0.15 | 0.15 |
| 28 | Sampling Grid |  | 0.2 | 0.2 | Gaussian | 1.00 | 1 | 0.2 | 0.2 |
| Stage 2: Calibration Measurement |  |  |  |  |  |  |  |  |
| 29 | Vector Network Analyzer |  | 0.13 | 0.2 | Gaussian | 1.00 | 1 | 0.13 | 0.2 |
| 30 | Mismatch of Receiver Chain |  | 0 | 0 | Gaussian | 1.00 | 1 | 0 | 0 |
| 31 | Insertion Loss of Receiver Chain |  | 0 | 0 | Gaussian | 1.00 | 1 | 0 | 0 |
| 32 | Mismatch in the connection of the calibration antenna |  | 0.02 | 0.02 | U-shaped | 1.41 | 1 | 0.02 | 0.02 |
| 33 | Influence of the calibration antenna feed cable |  | 0 | 0 | Gaussian | 1.00 | 1 | 0 | 0 |
| 34 | Influence of the probe antenna feed cable |  | 0 | 0 | Gaussian | 1.00 | 1 | 0 | 0 |
| 35 | Reference antenna |  | 0.5 | 0.25 | Rectangular | 1.73 | 1 | 0.29 | 0.25 |
| 36 | Short term repeatability |  | 0.088 | 0.088 | Gaussian | 1.00 | 1 | 0.088 | 0.088 |
|  |  | Combined standard uncertainty (1σ) [dB] |  |  |  |  |  | TBD | TBD |
|  |  | Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  | TBD | TBD |

<< end of proposal >>

# Conclusion

Text proposal to TR 37.843 is presented in order to add the test procedure and MU for OTA occupied bandwidth measurement in a Near Field test range.

# Reference

[1] R4-1803043, “Test Procedure for OTA ACLR measurement in a Near Field Test Range”, Feb 2018

[2] 3GPP TR 37.842 v 13.2.0, March 2017

[3] ANSI/IEEE Std 149-1979: "IEEE Standard Test Procedures for Antennas".