3GPP TSG-RAN WG4 Meeting #86bis                 	R4-1805142

Melbourne, Australia, April 16th – 20th 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on NR SCell Activation Delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-04-06 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The NR SCell activation delay was left as TBD |  |  |  |  |  |  |  |  |
| Summary of change: |  | To give values for NR SCell activation delay |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements are not complete since many TBD values in NR SCell activation delay. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 8.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes>>

### 8.3.2	SCell Activation Delay Requirement for Deactivated SCell

The requirements in this section shall apply for the UE configured with one downlink SCell in SCG.

The delay within which the UE shall be able to activate the deactivated SCell in SCG depends upon the specified conditions.

Upon receiving SCG SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+ [THARQ + Tactivation_time + TCSI_Reporting ]

Where:

THARQ is the timing between DL data transmission and acknowledgement as specified in [7].

Tactivation_time is the SCell activation delay. If the SCell is known and belongs to FR1, Tactivation_time is 1 SSB. If the SCell is unknown and belongs to FR1, Tactivation_time is  [5 or 6] SSBs provided the SCell can be successfully detected on the first attempt. If the SCell is known and belongs to FR2,Tactivation_time is 1 SSB. If the SCell is unknown and belongs to FR2, Tactivation_time is  [5 or 6] * N1 SSBs provided the SCell can be successfully detected on the first attempt.

TCSI_reporting is the delay uncertainty in acquiring the first available CSI reporting resources as specified in [2].

SCell is known if it has been meeting the following conditions:

-	[During the period equal to max([5] measCycleSCell,  [5] DRX cycles) for FR1 before the reception of the SCell activation command (the condition for FR2 case is FFS):

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SCell being activated remains detectable according to the cell identification conditions specified in section TBD,]

-	SCell being activated also remains detectable during the SCell activation delay according to the cell identification conditions specified in section [TBD].

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in [2] for an SCG SCell at the first opportunities for the corresponding actions once the SCell is activated.

The PSCell interruption specified in section 8.2 shall not occur before slot n+[THARQ] and not occur after the timing as defined in section TBD of [3] (timing for secondary Cell activation/deactivation).

Starting from the slot specified in section [TBD] of [3] (timing for secondary Cell activation/deactivation) and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

<< End of changes>>