3GPP TSG-RAN WG4 Meeting #86bis	R4-1805272

Melbourne, Australia, 16 – 20 April 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR |  | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 36.133 on UE measurement capability requirements with same carrier configured by MN and SN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-04-06 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | UE measurement capability requirements for EN-DC do not account the situation where PCell and PSCell configure the same carrier to be monitored. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update UE measurement capability requirements to address that in case PCell and PSCell configure same carrier, this carrier shall only be counted once to the total number of monitored carriers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The total number of carriers is not clear in the described situation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1.1b.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

8.1.2.1.1b.1 Maximum allowed layers for multiple monitoring for UE in NSA operation

The UE configured with NR PSCell shall be capable of monitoring at least per RAT group:

-	Depending on UE capability, [6] FDD E-UTRA inter-frequency carriers configured by PCell, and

-	Depending on UE capability, [6] TDD E-UTRA inter-frequency carriers configured by PCell, and

-	Depending on UE capability, [7] NR inter-RAT carriers configured by PCell, and

-	Depending on UE capability, [7] NR inter-frequency carriers configured by NR PSCell [50].

-	Depending on UE capability, [3] FDD UTRA carriers, and

-	Depending on UE capability, [3] TDD UTRA carriers, and

-	Depending on UE capability, [32] GSM carriers (one GSM layer corresponds to 32 carriers), and

In addition to the requirements defined above, the UE shall be capable of monitoring a total of at least [13] effective carrier frequency layers comprising of any above defined combination of E-UTRA FDD, E-UTRA TDD, UTRA FDD, UTRA TDD, GSM (one GSM layer corresponds to 32 carriers) and NR layers. If E-UTRA PCell and PSCell configure the same NR carrier frequency layer to be monitored, this carrier frequency layer shall be counted only once to the total number of effective carrier frequency layers.

The UE shall be capable of monitoring a total of at least [7] effective NR carrier frequency layers configured by PCell and NR PSCell.

Note:	The EN-DC capable UE configured with NR PSCell shall fulfil the requirements defined in only one of Section 8.2.1.1b.1 and Section 9.1.3.2 of TS 38.133 [50].