3GPP TSG-RAN WG4 Meeting #86Bis		R4-1805321

Melbourne, Australia, 16 - 20 April 2018

Source:	Sony

Title:	UE Spherical coverage at mmWave 28GHz

Agenda item:	7.4.8.3

Document for:	Discussion

# Background

The impact on spherical coverage of a realistic design has been discussed for several meetings. In RAN4 #86 in Athens the focus moved to the NW impact on spherical coverage [4]. Even though there were a few contributions covering simulation on a realistic design [1], [2], [3]. In RAN4-AH-1801 in San Diego, detailed simulation assumptions were agreed in WF on spherical coverage [5]. No new simulation assumption was agreed in RAN4 #86.

This paper is an update of [1] but does also include measurement on a smartphone evaluation prototype. In [1] the impact of glass and plastic back cover on spherical coverage was studied for 9 different assumptions including the 7 assumptions listed in [4]. In this paper we have added measurement and we have updated the simulations based on the feedback in last meeting and therefore a simulation on decreased distance between antenna and glass layer is added.

# Simulation and measurement set-up

Spherical coverage is simulated for all 7 assumptions in [5]. These assumptions are copied in Table 1. In addition assumption 4*, 6* and assumption 7* including “Partial” displays are added. Partial display, in our simulation, means there is 5mm clearance in top (and bottom) and thus one antenna array is placed in the front and the two others in the back compared to assumption 6 and 7 where there are no room for antennas in the front and thus all 3 antennas are placed on the backside. This is schematically shown in Figure 1. The simulations are done for a single frequency but valid for the complete frequency band since only relative results ( EIRP @ %-tile) are discussed.

|  |  | Assumptions1 | Assumptions2 | Assumptions3 | Assumptions4 | Assumptions5 | Assumptions6 | Assumptions7 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency range |  | n257 | n257 | n257 | n257 | N257 | n257 | N257 |  |
| # of antenna in an antenna module/set (# of patches, # of dipoles, etc.) |  | - | - | - | - | - | - | - | Depends on the current implementation |
| # of antenna module/set in total |  | 1 | 1 | 2 | 2 | 2 | 3 | 3 |  |
| Finite UV test points | Y/N | Y | Y | Y | Y | Y | Y | Y | Finite test point shall be the baseline |
| Beam phase shifter controller | degree | 45 | 45 | 45 | 45 | 45 | 45 | 45 | Finite beam shall be the baseline |
| Antenna type (patch, dipole, or both) |  | - | - | - | - | - | - | - | Depends on the current implementation |
| Antenna module/set location (front, back, top-side, left-side, right-side, bottom-side) |  | Top / Bottom | Top / Bottom | Top & Bottom | Top & Bottom | Top & Bottom | Left & Right & Bottom | Left & Right & Bottom | Combination of the lists are not precluded. |
| Front cover (Plastic, Glass, Ceramic, Metal) |  | Glass | Glass | Glass | Glass | Glass | Glass | Glass | This information is meaningful only if it’s the same with the material which covers antennas. |
| Back cover (Plastic, Glass, Ceramic, Metal) |  | Glass | Plastic | Glass | Glass | Plastic | Glass | Plastic |  |
| Side cover / Frame (Plastic, Glass, Ceramic, Metal) |  | Metal | Plastic | Metal | Metal | Plastic | Metal | Plastic |  |
| Device size (WxHxD) | cm3 | 66.6 | 66.6 | 66.6 | 66.6 | 66.6 | 66.6 | 66.6 | This is for information |
| Display panel – Full (Y) or Partial (N) | Y/N | Y | Y | Y | N | Y | Y | Y |  |
| Bezel Margin | mm | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | Module can’t be placed outer edge of UE to secure mechanical reliability |

Table 1. Simulations assumption from “WF on EIRP CDF for spherical coverage” [5].

Figure 1. Schematic picture of Assumption 6* (and 7*)

## UE layout model

In [6] a simplified model of a handheld UE (smartphone) was used. In this simulation CAD-files of an existing smartphone have been extracted into the full wave electromagnetic simulator. The display size has been modified to accommodate the different assumptions. In all simulations the antennas are general patches on Rogers RO4003C™ laminate. A summary of the layer properties according to [5] is given in Table 2.

| Layer Properties |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Material | Thickness | Ɛr | tan δ |
| Antenna substrate Dielectrics | Rogers RO4003C™ | 0.3mm |  |  |
| Antenna Substrate Metals | copper | - | conductivity |  |
| Gap 1 | Air/Plastic/? | ~2-3mm |  |  |
| Gap 2 | Air/Plastic/? | 2.8mm |  |  |
| Gap 3 | Air/Plastic/? | G3 |  |  |
| Gap 4 | Air/Plastic/? | 1.5mm | Air |  |
| Edge material 1 | Glass/Plastic/Metal | W1 | Al |  |
| Edge material 2 | Glass/Plastic/Metal | 0.77mm | 6.98 | 0.03 |
| Edge material 3 | Glass/Plastic/Metal | W3 | Al |  |
| Edge material 4 | Glass/Plastic/Metal | 0.77mm | 6.98 | 0.03 |

Table 2. Summary of layer properties used in the simulations

## Measurement set-up

The measurement is done on an evaluation prototype in a handheld UE size (lager than 3GPP simulation assumption) with partial display and plastic back cover. A schematic outline is shown in Figure 2. Measurement is done with a CW signal and therefore only relative gain is reported. Antenna modules consists of patch elements in 2x2 matrices. A far field test method is used.

Figure 2. Schematic outline of measurement prototype.

# Result

## Measurements vs. Simulations

Measured versus simulated CDF EIRP spherical performance is shown in Figure 3 and result is listed in Table 3.

Observation 1: A similar trend is observed between simulated and measured data.

Figure 3. Measurements vs. simulation on case 4* (front glass + plastic back cover).

| Assumption | - | Measured 4* | Simulated 4* |
| --- | --- | --- | --- |
| Display | Partial / Full | Partial | Partial |
| No antenna arrays | Back side | 1 | 1 |
|  | Front side | 1 | 1 |
| Side cover (cover material near antennas) | Metal / Plastic | Plastic | Plastic |
| Back cover | Glass / Plastic | Plastic | Plastic |
| Front cover | Glass / Plastic | Glass | Glass |
| EIRP @ 100%%-tile point | dBm | Refer to separate discussion |  |
| EIRP @ 50% | dB | 9 | 8 |
| EIRP @ 40% | dB | 10 | 9 |
| EIRP @ 30% | dB | 11 | 10 |
| EIRP @ 20% | dB | 12 | 11 |

Table 3. Comparison between measured and simulated CDF spherical performance

## Impact of Back Cover Material – Glass vs. Plastic

Glass versus plastic back cover material is shown in Figure 4 in the 3-antenna array example (assumption 6 vs. assumption 7). Note that all 3 antennas are mounted in the back.

It can be seen that the peak value is only affected marginally but the lower percentiles are degraded substantially by the high loss glass material compared to plastic. The same trend is seen for other number of antennas (e.g. assumption 1 vs. 2 or assumption 3 vs. 5)

Observation 2: Implementation loss due to high-loss glass is 1 – 2 dB higher compared to plastic cover material.

Figure 4. Impact of Back Cover Material.

## Number of Antenna Arrays

The number of antenna arrays are studied in Figure 5 for both plastic and glass back cover. Note that all antenna arrays are in the back in this comparison. The following observations are done:

Observation 3: There is no significant difference in free space between 2 and 3 arrays when facing in the same direction. Single antenna has ~1dB worse spherical performance.

Observation 4: The same trend is seen regardless of plastic or glass cover.

Observation 5: Hand and body blockage could possibly motivate higher number of antenna arrays.

Figure 5. Number of Antenna Arrays. a. Plastic back cover. b. Glass back cover.

## Single-side Antennas Arrays vs. 2-side Arrays

Single-side antennas arrays (backside only) versus antenna arrays on both front and back is studied in Figure 6 leading to:

Observation 6: There is a significant improvement in spherical coverage with front + back antenna arrays vs. back-side arrays only.

Figure 6. Single-side antennas arrays versus 2-side arrays

## Antenna Cavity Size

In last meeting the influence of antenna cavity size was discussed, i.e. the distance between the antenna module and front and back cover. Therefore the antenna cavity was investigated in a simulation where the distances were reduced to 0.5mm compared to the original model (based on an existing smartphone) of 1.5mm (antenna to back cover) and 2.8mm (antenna to front). The simulation was based on 3GPP assumption 4 and the result is shown in Figure 7. In this case no significant difference was observed, but this observation may not cover all possible cases.

Figure 7. Impact of antenna cavity size.

## Summary

The simulated assumptions are summarized in Figure 8 and Table 4. Note that  EIRP @ %-tile in Table 4 refers to the 99.9%-tile point which is dependent on assumption (~1dB difference).

Figure 8. Spherical coverage as CDF simulated on a realistic handheld UE (smartphone) layout.

| Assumption | - | 3GPP 1 | 3GPP 2 | 3GPP 3 | 3GPP 4 | 4* | 4* measured | 3GPP 5 | 3GPP 6 | 6* | 3GPP 7 | 7* |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Display | Partial / Full | F | F | F | P | P | P | F | F | P | F | P |
| No antenna arrays | Back side | 1 | 1 | 2 | 1 | 1 | 1 | 2 | 3 | 2 | 3 | 2 |
|  | Front side |  |  |  | 1 | 1 | 1 |  |  | 1 |  | 1 |
| Side cover (cover material near antennas) | Metal / Plastic | M | M | M | M | P | P | M | M | M | M | M |
| Back cover | Glass / Plastic | G | P | G | G | P | P | P | G | G | P | P |
| Front cover | Glass / Plastic | G | G | G | G | G | G | G | G | G | G | G |
| EIRP @ 100%%-tile point | dBm |  |  |  |  |  |  |  |  |  |  |  |
| EIRP @ 50% | dB | 13 | 10 | 13 | 9 | 8 | 9 | 10 | 12 | 7 | 10 | 7 |
| EIRP @ 40% | dB | 15 | 12 | 14 | 10 | 9 | 10 | 11 | 14 | 9 | 11 | 8 |
| EIRP @ 30% | dB | 16 | 14 | 16 | 12 | 10 | 11 | 13 | 16 | 10 | 13 | 9 |
| EIRP @ 20% | dB | 18 | 17 | 18 | 14 | 11 | 12 | 15 | 17 | 11 | 14 | 10 |

Table 4. Summary of simulated spherical coverage for a realistic handheld UE (smartphone) layout.

# Conclusion

The spherical coverage of a handheld UE was simulated using CAD-files from an existing smartphone modified with patch antenna arrays for 28GHz. In addition spherical coverage was measured on an evaluation prototype in a handheld UE size. The following observations were made:

Observation 1: 	A similar trend is observed between simulated and measured data.

Observation 2: 	Implementation loss due to high-loss glass is 1 – 2 dB higher compared to plastic cover material.

Observation 3: 	There is no significant difference in free space between 2 and 3 arrays when facing in the same direction. Single antenna has ~1dB worse spherical performance.

Observation 4: 	The same trend is seen regardless of plastic or glass cover.

Observation 5: 	Hand and body blockage could possibly motivate higher number of antenna arrays.

Observation 6: 	There is a significant improvement in spherical coverage with front + back antenna arrays vs. back-side arrays only.

# References

- R4-1802868	“UE Spherical coverage at mmWave 28GHz”, Sony

- R4-1801594	“EIRP CDF Simulation Analysis for Spherical Coverage Topic_2”, MediaTek Inc

- R4-1802033	“Facts on UE performance and spherical EIRP in FR2”, Samsung

- R4-1803275	“WF on network performance analysis for spherical coverage”, Samsung

- R4-1801202	“WF on EIRP CDF for spherical coverage”, Samsung MediaTek

- R4-1800888 	“UE Spherical coverage at mmWave 28GHz”, Sony, Ericsson

Refer to separate discussion