3GPP TSG-RAN WG4 Meeting #86bis                                         R4-1805323

Melbourne, Australia, 16 - 20 April, 2018

Agenda Item:	7.1.3

Source:	Apple

Title:	Performance comparison of single Tx and dual Tx for intra-band EN-DC

Document for:	Discussion/Decision

# Introduction

A key enabler for early commercialization of 5G NR is the non-stand-alone (NSA) deployment, which is expected to be adopted in many of the initial deployment world-wide. The NSA mode, which is based on LTE-NR dual connectivity, requires UEs to support both LTE and NR uplink transmissions.

In EN-DC, based on whether UE is required to transmit LTE UL and NR UL simultaneously, UE can have two different UL operation modes

- Dual UL transmission: in this mode, UE is required to transmit LTE UL and NR UL simultaneously

- Single UL transmission: in this mode, at any given time, UE is required to transmit uplink in at most one RAT, i.e. either LTE or NR.

Compared to dual UL transmission, single UL transmission has many performance benefits especially for the UE in power limited scenario (i.e. cell edge) as well as implementation advantages.

- Single UL transmission solves the DL desense problem: as explained in [1, 2, 3], depending on the band combinations and resource allocation, sending two concurrent uplinks can produce undesirable intermodulation products that could fall right into the NR or LTE receiving bands, causing degradation of UE’s receiver sensitivity. This is especially problematic for band combinations for both LTE and NR bands below 6 GHz, the impacted receiver bands due to MSD can be virtually not usable in practice, and it creates extreme challenges in RF implementations.

- Single UL transmission removes the UE additional maximum transmit power backoff requirement i.e. A-MPR: As discussed in [4], simultaneous LTE and NR UL transmission can cause intermodulation products outside the desired transmission bandwidth, which can violate the regulatory emission requirement. To meet the regulatory emission requirement, UE is required to apply additional maximum transmit power backoff, i.e. A-MPR. A-MPR.

- Single UL transmission ensures LTE coverage for UE not capable of dynamic power sharing: For EN-DC, total maximum transmit power budget has to be shared between LTE and NR in case of dual UL transmission. However, the initial NR UE may not support dynamic power sharing between LTE and NR. As a result, maximum transmission power budget has to be split semi-statically between LTE and NR. For example, a 23dBm total maximum transmit power limit can be split such that both LTE and NR have 20dB maximum transmit power per RAT. Compared to single UL transmission where each RAT can transmit up to 23dBm, this reduces the coverage by 3dB.

- Single UL transmission reduces the impact on the RF frontend architecture: Compared to single UL transmission, supporting of dual UL transmission requires the device to completely change its RF frontend architecture while adds significantly more complexity. As results, mandating dual UL transmission will cause the UE to support much less number of band combinations. The detailed explanation can be found in [5]

In 3GPP RAN#77 [6], it was agreed to introduce the indication of whether UE supports 2 simultaneous UL transmission.

- Define in RAN4 specs for which band combinations and channel allocations (to the extent specified by RAN4) within that band combination the UE is allowed to indicate that it does not support 2 simultaneous UL tx (this is not implying the granularity of the signalling) - Signalling to be defined to support the 'red text' from RAN4 part of the single tx discussion (i.e. as in RP-172064): - UE capability indicates that the UE does not allow 2 simultaneous UL transmission for the RAN4 specified channel allocations in a given band combination. If the network chooses to operate the UE in a way that is not consistent with this capability indication then the UE behavior is not specified and the UE might not meet the performance criteria.

In 3GPP RAN1#90 in August [7], specific TDM patterns were agreed to enable the single UE transmission operation:

Agreements: - When the UE is configured with multiple UL carriers on different frequencies (where there is at least one LTE carrier and at least one NR carrier of a different carrier frequency), but the UE operates on only one of the carriers at a given time among a pair of LTE and NR carriers - For LTE carrier, UE can be configured with - Case 1: DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell - For scheduling/HARQ timing of LTE FDD carrier, DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell is applied - UE is allowed to transmit NR UL signals at least in the subframe(s) where LTE UL transmission is not allowed according to the DL-reference UL/DL configuration - FFS whether or not a UE-specific subframe offset for the DL-reference UL/DL configuration can be configured considering system resource utilization and potential spec impact - Case 2: Release 15 LTE-FDD HARQ timing - No impact on LTE RAN1 specifications - Note: it doesn’t necessarily imply that UE has to support both cases

In 3GPP RAN2#100 [8], a capability indication is agreed for UE to inform the NW whether it supports dual UL transmission per band combination.

Agreements - RAN2 to adopt per-BC capability bit for SUO. The bit applies to all fallback BCs that are defined in RAN4 as problematic BCs. The bit does not apply to fallback BCs that are defined in RAN4 as not problematic BCs. Hence the UE does not explicitly list the fallback BCs. - UE is only allowed to set the per-BC capability bit for those band combinations that are defined in RAN4 as problematic BCs

In 3GPP RAN4 NR-AH#180 [9], three band combinations are agreed in which UE is allowed to indicate to NW not supporting dual UL transmission.

EN-DC band; E-UTRA Band; NR Band; Single UL allowed. DC_1_n28; 1; n28; No. DC_1_n77; 1; n77; DC_1_n77. DC_1_n78; 1; n78; No. …. DC_1_n257; 1; n257; No. DC_3_n77; 3; n77; DC_3_n77. DC_3_n78; 3; n78; DC_3_n78. ….

Note that the current agreed three difficult band combinations in RAN4 are all for inter-band EN-DC deployment. We provide the performance comparison between single Tx and dual Tx for inter-band EN-DC in [10].

In this contribution, we consider the intra-band EN-DC deployment and provide the performance comparison between single Tx and dual Tx.

# A-MPR issue of dual Tx for intra-band EN-DC deployment

As discussed in contribution [4, 11], simultaneous LTE/NR UL transmission can cause unwanted intermodulation product that requires UE to have additional maximum transmit power backoff, A-MPR, to avoid performance degradation or meet emission requirement. For intra-band EN-DC, there are at least two major issues

- Intra-band EN-DC in FDD band (e.g. band 71) [11]: With a single PA reference architecture, but to lower extent also dual PA architectures, even for intra-band continuous EN-DC deployment, intermodulation products created by simultaneous LTE and NR UL transmission can still significantly desense the DL reception because LTE and NR resource allocation may not be contiguous. Without single UL transmission, either large transmit power backoff is needed (more than 20dB), which significantly impacts the UL coverage, or Rx performance has to suffer.

- Intra-band EN-DC in TDD or FDD bands (e.g. band 41) [4]: The simultaneous LTE and NR UL transmission can cause intermodulation that violates the emission requirement. To meet the emission requirement, without single UL transmission, large additional maximum transmit power backoff, A-MPR, is required which reduces the UL coverage significantly. Single UL transmission ensures

In summary, Table 1 provides the potential A-MPR that are needed for the intra-band EN-DC deployment based on contribution [4,11]. Note that the actual number of A-MPR may need further investigation depending on the band combinations and the resource allocations.

Table 1 A-MPR comparison between single Tx and dual Tx for intra-band EN-DC

|  |  | A-MPR |
| --- | --- | --- |
| Single Tx |  | 0 dB |
| Dual Tx | Intra-band EN-DC, FDD Band Single PA | Up to 24 dB |
|  | Intra-band EN-DC, TDD Band Dual PA | 9-14 dB |

From Table 1, it is clear that compared to single Tx, dual Tx requires much higher A-MPR for intra-band EN-DC.

Observation 1: Compared to single Tx, dual Tx requires much higher A-MPR for intra-band EN-DC

In the section 3, we discuss the mechanism that can be used to share the UL resource between LTE and NR for single Tx operation.

# UL TDM pattern for single Tx in intra-band EN-DC

Before we could provide the performance comparison between single Tx and dual Tx. We need to discuss the UL resources sharing between LTE and NR in single Tx operation. For single Tx operation, the UL resources have to be shared in a TDM manner between LTE and NR. Following are more details for the TDM pattern for single UL operation in intra-band EN-DC in FDD band and TDD band.

## Single Tx UL TDM pattern for intra-band EN-DC in FDD Band

For single Tx operation in intra-band EN-DC in FDD band, the UL resource sharing can follow the agreed pattern of case 1 in RAN1 [7]. More specifically

- UL subframes/slots are semi-statically split between LTE and NR

- For the UL subframes allocated to LTE, the HARQ feedback timing used for LTE TDD-FDD CA is used such that LTE can schedule DL without any restriction.

In this contribution, we assume UL resource allocation ratio of 4:6 between LTE and NR as illustrated in Figure 1.

Figure 1 Single Tx UL TDM pattern for intra-band EN-DC in FDD band

## Single Tx UL TDM pattern for intra-band EN-DC in TDD Band

For single Tx operation in intra-band EN-DC in TDD band, we list a few considerations as below for the TDM pattern design

- When LTE and NR are deployed in the same TDD band, it is very nature for them to be deployed synchronously in terms of TDD pattern, i.e. deployment uses one of the 7 LTE TDD configurations

- Given the LTE TDD pattern, if we do not want to limit the LTE DL scheduling, then we need to guarantee the PUCCH for DL ACK/NAK feedback. Therefore, all the U subframes in the TDD configuration need to be used for LTE.

- As all the U subframes are used for LTE, NR can use the special subframe for uplink. NR could transmit its uplink in either GAP or UpPTS or combination of both. NR supports very flexible PUCCH configurations, including short PUCCH (1-2 symbols) and long PUCCH (4-14 symbols). In addition, scalable numerology can be used as well.

Based on the above consideration, we use an example of LTE TDD configuration 2 (2 UL, 2 SSF and 6 DL), and special subframe configuration 7 to demonstrate the TDM pattern for LTE-NR EN-DC in TDD band. This is also the settings that we use for our performance evaluations, in summary

- UL subframe in the LTE TDD configuration is used for LTE in order not to restrict the LTE DL scheduling

- Special subframes are TDM’ed between LTE and NR as illustrated in Figure 2.

Figure 2 Single Tx UL TDM pattern for intra-band EN-DC in TDD band, for special subframe

# Link budget comparison between single Tx and dual Tx for intra-band EN-DC

In this session, we focus on the link budget analysis of the PUCCH (control) channel. The link budget of PUCCH depends on two major factors

- Maximum transmit power: Dual Tx incurs A-MPR which will negatively impact the link budget

- Duration of PUCCH: single Tx may require the PUCCH duration to be shorter especially for TDD band which may impact the PUCCH link budget as well.

For the A-MPR, we take a conservative value of 9dB from Table 1. Table 2 provides the setting for the link budget comparison

Table 2 Link budget analysis for single and dual UL transmission

|  | Dual Tx | Single Tx |
| --- | --- | --- |
| AMPR | 9dB | 0dB |
| LTE Effective Tx Power (Link Budget) | 20 - 9 = 11 | 23 |
| NR Effective Tx Power (Link Budget) | 20 - 9 = 11 | 23 + 10*log10(#NR_PUCCH Symbol/14) |

Figure 3 illustrates the link budget analysis of single Tx and dual Tx for intra-band EN-DC, for both LTE and NR respectively. Figure 3 clearly demonstrates the link budget improvement from single UL transmission, for both LTE and NR.

Observation 2: Compared to dual Tx, single Tx provides significantly better UL coverage

Figure 3 Link budget comparison between single Tx and dual Tx (top: LTE, bottom NR)

# UL Throughput comparison between single Tx and dual Tx for intra-band EN-DC

As discussed in section 3, the UL TDM resource sharing pattern could be different for EN-DC deployment depending on TDD band or FDD band. Therefore, we have two subsections for the performance comparison. Table 3 lists the evaluation assumptions

Table 3 Evaluation assumption for performance comparison for single Tx and Dual Tx

|  | LTE | NR |
| --- | --- | --- |
| Carrier Frequency | 2.5GHz | 2.5GHz |
| Aggregate System Bandwidth | 20MHz | 20MHz |
| Dual Tx A-MPR | 9 dB |  |
| Single Tx A-MPR | 0 dB |  |
| Path Loss (d in meter) | 36.873, Assume bore-side direction PL = 22.0log10(d) + 28.0 + 20log10(fc); 10m<d<400m PL = 40log10(d)+28.0+20log10(fc) -46.85; d>400m |  |
| Penetration Loss | 20dB |  |
| UE Antenna Gain | 0dBi |  |
| BS Antenna Gain | 17dBi |  |
| Thermal Noise Level | -174dBm/Hz |  |
| UL Power Control | Target = 25dB over Thermal, Alpha = 0.9 |  |
| UE Total Tx Power | 23dBm |  |
| BS Receiver Noise figure | 5dB |  |
| UE Antenna Config | 2Tx/4Rx | 2Tx/4Rx |

## UL Throughput comparison for intra-band EN-DC in FDD Band

Figure 4 shows UL throughput comparison between single Tx and dual Tx for intra-band EN-DC in FDD band. Comparing the blue (single Tx) and black (dual Tx) curve, it is easy to see what for a wide range of path loss (path loss greater than 100dB), single Tx actually performs better than dual Tx.

Moreover, it is also important to compare the performance from the RF architecture perspective. For single Tx operation, as we explained in [5], it allows the UE to use single PA solution to support EN-DC. For intra-band EN-DC, single PA solution also supports the dual Tx operation. For dual Tx operation, compared to dual PA solution, single PA solution suffers from increased A-MPR. To compare the performance of different RF architecture, we also provide the evaluation of the single PA, dual Tx operation assuming 15dB A-MPR (increased from 9dB) illustrated as the red curve in Figure 4. Clearly, single PA solution has minimal performance gap from the dual PA solution.

Figure 4 UL Throughput comparison for intra-band EN-DC in FDD Band

## UL Throughput comparison for intra-band EN-DC in TDD Band

Figure 5 shows UL throughput comparison between single Tx and dual Tx for intra-band EN-DC in TDD band. Comparing the blue (single Tx) and black (dual Tx) curve, it is easy to see what for a wide range of path loss (path loss greater than 100dB), single Tx actually performs better than dual Tx.

Similar as the discussion in section 5.1, we also compare the performance of single PA architecture and dual PA architecture. As we discussed before, for intra-band EN-DC, single PA solution also supports dual Tx operation. For dual Tx operation, compared to dual PA solution, single PA solution suffers from increased A-MPR. To compare the performance of different RF architecture, we assume 15dB A-MPR (increased from 9dB) for single PA, dual Tx operation, whose performance is illustrated as the red curve in Figure 5. Clearly, single PA solution has minimal performance gap from the dual PA solution.

To be more representative, we also present the performance comparison in terms of the coverage area in Figure 6. To show the coverage, each path loss point is converted to the distance, i.e. dPL. Assuming a cell has the coverage of 140dB, then for each path loss, the normalized coverage is computed as dPL/d140dB2. It is even more straightforward to see that single Tx performs as good as or even better than dual Tx for majority of the coverage

Therefore, we have the following observations

Observation 3: For majority of the coverage area, single Tx provides better UL throughput compared to dual Tx. for intra-band EN-DC

Observation 4: Single PA solution has minimal performance impact compared to dual PA solution for intra-band EN-DC

Figure 5 UL Throughput comparison for intra-band EN-DC in FDD Band, in terms of Path loss

Figure 6 Throughput comparison for intra-band EN-DC in FDD Band, in terms of coverage

In conclusion, based on all the discussions and observations, we have the following proposal

Proposal 1:  3GPP will expand the EN-DC band combination list allowing the UE to operate in single Tx operation in EN-DC band combinations additionally to the combinations already listed in 38.101-3. An example how the list can be specified is shown in contribution [5]

# Conclusions

In this contribution, we provide the numerical results comparing the performance of dual Tx and single Tx in terms of both link budget and UL throughput, we focus on the intra-band EN-DC deployment for both the TDD band and FDD band.

It is shown that single Tx offers many benefits compared to dual Tx with the following observations

Observation 1: Compared to single Tx, dual Tx requires much higher A-MPR for intra-band EN-DC

Observation 2: Compared to dual Tx, single Tx provides significantly better UL coverage

Observation 3: For majority of the coverage area, single Tx provides better UL throughput compared to dual Tx. for intra-band EN-DC

Observation 4: Single PA solution has minimal performance impact compared to dual PA solution for intra-band EN-DC

Based on our observations, we propose the following

Proposal 1:  3GPP will expand the EN-DC band combination list allowing the UE to operate in single Tx operation in EN-DC band combinations additionally to the combinations already listed in 38.101-3. An example how the list can be specified is shown in contribution [5]

# References

- R1-1708276, “Uplink sharing in NSA mode”, 3GPP RAN1#89, Hangzhou, China, May 15th-19th, 2017.

- R4-1704759, “Co-existence analysis on LTE-NR band combinations”, 3GPP RAN4 #83, Hangzhou, China, May 15th-19th, 2017.

- R4-1703999, “European LTE-NR band combinations, and coexistence analysis”, RAN4#82bis, Spokane, US, April 3-7, 2017

- R4-1803062, “A-MPR for DC_41A_n41A”, RAN4#86, Athens, Greece, Feb. 26-March 2, 2018

- R4-1805411, “The need of FR1 single-switched UL for EN-DC”, RAN4#86bis, Melbourne, Australia, April 16-20, 2018

- RP-172085, “Signalling for Single/Dual UL Tx”, RAN#77, Sapporo, Japan, Sept 11-15, 2017

- Chairman Note, RAN1#90, Prague, Czech Rep, 9th – 13th October 2017

- Chairman Note, RAN2#100, Reno, Nevada, USA Nov. 27 – Dec. 1, 2017

- R4-1800404, “38.101-3 Implementation of agreed draft CRs”, RAN4 NR-AH#1801, San Diego, CA, Jan. 18-21, 2018

- R4-1710727, “Performance Evaluation of LTE NR DC Dual UL and Single UL”, RAN4 #84bis, Dubrovnik, Croatia, 9 -13 October, 2017

- R4-1802243, “Measurements of power backoff requirements for DC_(n)71B with single PA”, Athens, Greece, Feb. 26th ‒ March 2nd, 2018