3GPP TSG-RAN WG4 Meeting #86bis															R4-1805354

# Melbourne, AU, 16th – 20th April, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TR 37.843: On MU of the indoor anechoic chamber test method for OTA reference sensitivity level |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-04-06 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For adding MU for an indoor anechoic chamber test method on OTA reference sensitivity level |  |  |  |  |  |  |  |  |
| Summary of change: |  | MU of the anechoic chamber test method for OTA reference sensitivity level requirement is described |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Appropriate MU for OTA reference sensitivity level is not determined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

10.2.[2.1]	OTA reference sensitivity level

10.2.[2.1.1]	Indoor anechoic chamber test method

10.2.[2.1.1.1]	Description

10.2.[2.1.1.2]	Limitations and scope

10.2.[2.1.1.3]	Procedure

10.2.[2.1.1.4] Measurement Budget Format

10.2.[2.2.1.5] Uncertainty Assessment

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | 1.73 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the transmitting antenna | 0.3 | 0.3 | Rectangular | 1.73 | 1 | 0.17 | 0.17 |
| 3 | Quality of quiet zone | 0.1 | 0.1 | Gaussian | 1.00 | 1 | 0.10 | 0.10 |
| 4 | Polarization mismatch between the AAS BS and the transmitting antenna | 0.01 | 0.01 | Rectangular | 1.73 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the transmitting antenna | 0 | 0 | Rectangular | 1.73 | 1 | 0.00 | 0.00 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1.00 | 1 | 0.05 | 0.05 |
| 7 | Uncertainty of the RF signal generator | 0.46 | 0.46 | Gaussian | 1.00 | 1 | 0.46 | 0.46 |
| 8 | Impedance mismatch in the transmitting chain | 0.14 | 0.23 | U-shaped | 1.41 | 1 | 0.10 | 0.16 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | 1.73 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the transmitting antenna and the network analyzer | 0.05 | 0.05 | U-shaped | 1.41 | 1 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the transmitting antenna | 0.01 | 0.01 | Rectangular | 1.73 | 1 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and network analyzer | 0.05 | 0.05 | U-shaped | 1.41 | 1 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.1 | 0.1 | Gaussian | 1.00 | 1 | 0.10 | 0.10 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | 1.73 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the transmitting antenna | 0 | 0 | Rectangular | 1.73 | 1 | 0.00 | 0.00 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1.00 | 1 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.20 | Gaussian | 1.00 | 1 | 0.13 | 0.20 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | 1.73 | 1 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1.00 | 1 | 0.06 | 0.06 |
| 20 | Influence of the transmitting antenna feed cable | 0.05 | 0.05 | Rectangular | 1.73 | 1 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.43 | Rectangular | 1.73 | 1 | 0.29 | 0.25 |
| 22 | Uncertainty of the absolute gain of the transmitting antenna | 0 | 0 | Rectangular | 1.73 | 1 | 0.00 | 0.00 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.62 | 0.64 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.22 | 1.25 |