3GPP TSG-RAN WG4 Meeting #86bis	                                                             R4-1805370

# Melbourne, AU, 16th – 20th April, 2018

Source:	NTT DOCOMO, INC.

Title:	Indoor anechoic chamber test method procedure and MU for Rx spurious emissions OTA

Agenda item:	6.26.3.1.5

Document for:	Approval

- Introduction

This contribution presents Rx spurious emission test procedure and MU for an indoor anechoic chamber test method for conformance testing.

- Spurious emission conformance requirement

It is stated that the derivation of the OTA conformance requirement for the RX spurious emissions, uses the same principles as for the TX spurious emissions requirement OTA, as in sub-clause 5.6.6.3. Definition of the emissions measurements for the conformance requirement is based on the core requirement, subject to the TRP MU, where the MU definition should capture the whole spurious frequency range.

NOTE:	The OTA RX spurious emission requirement for the AAS BS with the RX-only capabilities is FFS.

For OTA AAS BS operating in TDD: applicability the OTA RX spurious emissions conformance requirement when all transmitters are OFF, is FFS.

- Indoor anechoic chamber test method for spurious emissions

The method measures spurious emissions in an anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the receiving antenna of no less than 2D2/λ, where D is the largest dimension of the antenna of AAS BS and λ is the wavelength. One or some receiving antennas  cover the whole frequency region of spurious emissions. The measurement system setup is as depicted in figure 10.3.2.1-1.

The maximum size of the DUT is a chamber restriction that would affect the quality of the quiet zone. For larger DUT sizes larger size chambers should be considered such that the uncertainty of the quiet zone is taken into account. One or some reference antennas cover the whole frequency region of spurious emissions. The measurable frequency region of spurious emissions is limited by the frequency characteristics of receiving and reference antennas. The measurement is conducted during the transmitter off period for OTA AAS BS operating in TDD.

The test consists of two stages: the calibration and the measurement. The test procedure is as follows.

# Figure 10.3.2.1-1: Indoor Anechoic Chamber measurement system setup for RX spurious emissions OTA

# Figure 10.3.2.3-1: Indoor Anechoic Chamber calibration system setup for RX spurious emissions OTA

# Stage 1 - Calibration:[The following procedure is completed every measured frequency range]

1)	Connect the reference antenna and the receiving antenna for measuring the target RX spurious emission frequency range to the measurement RF out port and RF in port of the network analyzer, respectively, as shown in figure 10.3.2.3-1.

2)	Install the reference antenna with OTA REFSENS RoAoA/minSENS RoAoA and the height of its phase centre aligned with the receiving antenna.

3)	Set the frequency range of the network analyzer to the target RX spurious emission frequency range and measure LFEIRP, E→D which is equivalent to 20log|S21| (dB) obtained by the network analyzer:

-	LFEIRP, E→D: Pathloss between E and D in figure 10.3.2.3-1.

4)	Measure the cable loss, LFEIRP, E→F between the reference antenna connector and the network analyzer connector for the target RX spurious emission frequency range:

-	LFEIRP, E→F: Cable loss between E and F in figure 10.3.2.3-1 for the target RX spurious emission frequency range.

5)	Calculate the calibration value for the target RX spurious emission frequency range between A and D with the following formula:

-	LEIRP_cal, A→D = LFEIRP, E→D  + GREF_ANT_EIRP, A→F -LFEIRP, E→F.

-	LEIRP_cal, A→D:  Calibration value between  A and D in figure 10.3.2.3-1.

-	GREF_ANT_EIRP, A→F: Antenna gain of the reference antenna.

# Stage 2 - Measurement:

6)	Uninstall the reference antenna and install the AAS BS with the manufacturer declared coordinate system reference point in the same place as the phase centre of the reference antenna. The manufacturer declared coordinate system orientation of the AAS BS is set to be aligned with the testing system.

7)	Set the AAS BS to be satisfied with OTA REFSENS RoAoA/minSENS RoAoA covering conformance testing receiving direction.

8)	Rotate the AAS BS to make the testing direction aligned with the direction of the receiving antenna.

9)	Set the AAS BS to transmit the test signal at the maximum power according to E-TM1.1.

10)	Measure the mean powers of spurious emissions in the target RX spurious emission frequency range at the measurement equipment connector, denoted by PR_AAS_rxspurious, D, during the transmitter off period,

-	PR_AAS_rxspurious, D: Measured spurious mean power for each carrier at the measurement equipment connector at D in figure 10.3.2.3-1.

11)	Calculate the EIRP with the following formula:

# EIRPe = PR_AAS_rxspurious, D+ LEIRP_cal, A→D

12)	Repeat the above steps 10)~11) at a number of discrete directions around the sphere

13)	Calculate TRP following the equation for each frequency range of reference antenna

, where EIRPe is the filtered mean emission power in the spurious domain and p1 and p2 denotes two orthogonal polarizations.

- Indoor anechoic chamber Uncertainty Assessment for RX spurious emissions OTA

TRP requirement is defined as the sum of measured EIRP values around sphere. Therefore, the EIRP measurement uncertainties of spurious emissions for an indoor anechoic chamber are the same as EIRP measurement for radiated transmit power in Rel-13 AAS specification except for target frequency range. The uncertainties of the measurement equipment and the antenna in the extended frequency range (4.2 GHz <f < 12.75GHz) is TBD..

# Table 1: Indoor Anechoic Chamber uncertainty assessment for EIRP measurement of RX spurious emissions OTA

| UID | Uncertainty source | Uncertainty value |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz | 4.2GHz < f ≦ 12.75GHz |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz | 4.2GHz < f ≦ 12.75GHz |  |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 | 0.02 |  |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0.3 | 0.3 | 0.3 | Rectangular | √3 | 1 | 0.17 | 0.17 | 0.17 |  |
| 3 | Quality of quiet zone | 0.10 | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 | 0.10 |  |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 | 0.01 |  |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0.00 | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 | 0.00 |  |
| 6 | Phase curvature | 0.05 | 0.05 | 0.05 | Gaussian | 1 |  | 0.05 | 0.05 | 0.05 |  |
| 7 | Uncertainty of the RF Power Measurement Equipment | 0.14 | 0.26 | TBD | Gaussian | 1 | 1 | 0.14 | 0.26 | TBD |  |
| 8 | Impedance mismatch in the receiving chain | 0.14 | 0.33 | 0.33 | U-shaped | √2 | 1 | 0.10 | 0.23 | 0.23 |  |
| 9 | Random uncertainty | 0.1 | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 | 0.06 |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |  | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |  | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |  | 0.04 |
| 13 | Quality of quiet zone | 0.10 | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |  | 0.10 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |  | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0.00 | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |  | 0.00 |
| 16 | Phase curvature | 0.05 | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |  | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.20 | TBD | Gaussian | 1 | 1 | 0.13 | 0.20 |  | 0.20 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |  | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 |  | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |  | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.43 | TBD | Rectangular | √3 | 1 | 0.29 | 0.25 |  | TBD |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0.00 | 0.00 | TBD | Rectangular | √3 | 1 | 0.00 | 0.00 |  | TBD |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  | 0.44 | 0.54 |  | TBD |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  | 0.87 | 1.06 |  | TBD |

- Conclusion

This contribution presents:

- OTA RX spurious emissions measurement description and MU values

- Further investigation of the dynamic range of spurious emission measurement equipment may be necessary.