3GPP TSG-RAN4 Meeting #87	R4-1806360

# Busan, South Korea, 21 - 25 May 2018

Source: 	Qualcomm Incorporated

Title: 	MSD for CA_1-21-42

Agenda item:	6.7.2

Document for:	Discussion

# Introduction

MSD for CA_1-21-42 has already been studied before as a part of rel 14. Though IMD of B1 and B42 transmit falls in B21 receive, it was determined that no MSD needs to be specified for this combination as only the spectrum used by the carrier needs to be considered for the MSD analysis. Using the spectrum used, it was determined that no MSD is needed as IM doesn’t fall in B21.

# Discussion

This approach resulted in not adding any MSD in 36.101. Engineers reading 36.101 only are mistakenly assuming that no MSD is allowed for this CA combination for all frequencies in these bands. Similarly, such confusion is but natural for other such band combinations. It is unclear that only frequencies used by the carrier needs to be tested. Also, its added work for engineers to dig up spectrum holdings for each carrier and to determine which CA combination is associated with which carrier.

Hence, we think its reasonable to add a new table to 36.101 to list all such band combinations which do have IMD issues but do not need to be specified as the spectrum used by the concerned carrier does not need MSD.

Proposal 1: To add a new table to TS 36.101 to list all the CA combinations which do need MSD but MSD does not need to be specified as the carriers use only portions of the spectrum for each band and as a result for that used spectrum no MSD is needed.

Proposal 2: There is no need to test these CA combinations

# Conclusion

In this paper, we propose to add a new table to 36.101 to list the CA combinations which need MSD when full spectrum of each band is considered but do not need MSD when only the spectrum holdings of the respective carriers are considered.

# References