3GPP TSG-RAN WG4 Meeting #87	R4-1806851

Busan, South Korea, 21st – 25th May, 2018

Agenda item:	6.20.4.3

Source: 	Qualcomm Incorporated

Title: 	Simulation Result for CGI reading with cross-TTI combining for eMTC UE

Document for:	Discussion

# Introduction

In the last RAN4 #86bis meeting, RAN4 agreed to revise the SIB1-BR TBS to 208 bits in the simulation assumption for CGI reading with cross-TTI MIB/SIB1-BR combining [1].

In this contribution, we present the revised simulation result for CGI reading delay based on the test parameters in [1].

# Simulation Result

Table 1 captures the simulation assumption agreed in the RAN4 #86bis meeting [1].

Table 1	Simulation parameters for CGI reading for CE Mode B UE.

| Parameters | Values |
| --- | --- |
| Serving cell SCH Es/Iot | -6dB |
| Target cell SCH Es/Iot | -15dB |
| UE receive antennas | 1 |
| BS transmit antennas | 2 |
| Fading channel | ETU1, EPA1 |
| PBCH repetition | Configured as specified in TS36.211 6.6.4 |
| SIB1-BR repetition level | 16 (schedulingInfoSIB1-BR = 18) |
| TBS for SIB1-BR | 208 bits (schedulingInfoSIB1-BR = 18) |
| MIB assumption | The network does not change the MIB payload information across MIB TTI (40ms) except for the system frame number (SFN) |
| SIB1-BR assumption | The network does not change the SIB1-BR information across SIB1-BR TTI (80ms). |
| Evaluation | 90% of target cell SI decoding success rate. |

Figure 1 shows the MIB acquisition delay in EPA1 and ETU1 channel with and without cross-TTI MIB combining. It is shown that inter-TTI combining can shorten the 95-percentile MIB acquisition delay to 1500ms, a reduction of ~400ms compared to the case without inter-TTI combining.

(a) EPA1                                                              				(b) ETU1

Figure 1. MIB acquisition delay with and without inter-TTI combining: (a) EPA1, (b) ETU1

Figure 2 shows the SIB1-BR decoding performance for different accumulation period. It is shown that up to 1280ms of accumulation is required in order to achieve PER less than 5%.

(a) EPA1                                                              				(b) ETU1

Figure 2. SIB1-BR PER decoding performance with different accumulation period: (a) EPA1, (b) ETU1

Combining the MIB and SIB1-BR decoding performance, we can see that it takes up to 2780 ms for an eMTC UE to perform CGI reading with the target success rate of 90% at -15dB SNR. Actual requirement can be defined by considering additional implementation margin on top of the simulation result.

Proposal 1. For a given SIB1-BR TBS of 208, CGI reading delay requirement under cross-TTI MIB/SIB1-BR combining is defined as 3200ms.

# Conclusion

In this contribution, we presented the simulation result about CGI acquisition time for eMTC UE with cross-TTI combining. Observation in this contribution is summarized as follows:

Proposal 1. For a given SIB1-BR TBS of 208, CGI reading delay requirement under cross-TTI MIB/SIB1-BR combining is defined as 3200ms.

# Reference

[1] R4-1805954, “Simulation assumptions for CGI reading for eFeMTC”, Ericsson, RAN4 #86bis, April 2018