3GPP TSG-RAN WG4 Meeting #88	R4-1809698

Gothenburg, Sweden, 20 – 24 August 2018

Agenda Item:	7.9.4.2.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on MU for NR FR1 OTA receiver sensitivity and reference sensitivity requirements for operating bands above 4.2GHz

Document for:	Approval

# 1.	Introduction

The MU for eAAS and NR FR1 OTA receiver sensitivity and reference sensitivity requirements for operating bands below 4.2GHz have been agreed in RAN4. For operating bands above 4.2GHz, it was agreed in the way forward [1] approved in RAN4 AH-1807 that ‘If no further analysis on NR FR1 according to the agreed timeline, the same MU budget as eAAS (3GHz < f ≦ 4.2GHz) can be reused for NR FR1 (3GHz < f ≦ 6GHz).’

This contribution provides our proposal on the MU for NR FR1 OTA receiver sensitivity and reference sensitivity requirements for operating bands above 4.2GHz, according to the agreed way forward in [1] and our proposals in [2].

# 2.	Discussion

For operating bands above 4.2GHz, we propose in [2] that:

1.	For the MU variables independent of the radiated measurement methods, use the values specified in TS 36.141 [3] for 4.2GHz < f ≦ 6GHz.

2.	For the MU variables dependent of the radiated measurement methods, use the values included in [4] for 4.2GHz < f ≦ 6GHz, and reuse the 3GHz < f ≦ 4.2GHz values in TR 37.842 [5] for 4.2GHz < f ≦ 6GHz for those not included in [4], unless other values are proposed in this meeting.

Using the above proposals, the MU for NR FR1 OTA receiver sensitivity and reference sensitivity requirements for operating bands above 4.2GHz can be calculated from those for operating bands below 4.2GHz as:

MU=MUEIS<4.22-MUconductedwanted<4.22+MUconductedwanted>4.22-MUTestEquipment<4.22+MUTestEquipment>4.22

The resultant MU are shown in Tables 1 and 2 below for Indoor Anechoic Chamber and Compact Antenna Test Range, respectively.

Table 1: MU for Indoor Anechoic Chamber

| Test System Uncertainty | Standard uncertainty ui [dB] 4.2GHz < f ≦ 6GHz |
| --- | --- |
| MUEIS<4.2 (Combined standard uncertainty in [5]) | 0.64 |
| 1.96σ MUconductedwanted<4.2 (Wanted signal level error for 3GHz < f ≦ 4.2GHz in [3]) | 1 |
| 1.96σ MUconductedwanted>4.2 (Wanted signal level error for 4.2GHz < f ≦ 6GHz in [3]) | 1.5 |
| MUTestEquipment<4.2 (Uncertainty of the RF signal generator for 3GHz < f ≦ 4.2GHz in [5]) | 0.46 |
| MUTestEquipment>4.2 (Uncertainty of the RF signal generator for 4.2GHz < f ≦ 6GHz in [4]) | 0.58 |
| Combined standard uncertainty (1σ) | 0.93 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) | 1.82 |

Table 2: MU for Compact Antenna Test Range

| Test System Uncertainty | Standard uncertainty ui [dB] 4.2GHz < f ≦ 6GHz |
| --- | --- |
| MUEIS<4.2 (Combined standard uncertainty in [5]) | 0.71 |
| 1.96σ MUconductedwanted<4.2 (Wanted signal level error for 3GHz < f ≦ 4.2GHz in [3]) | 1 |
| 1.96σ MUconductedwanted>4.2 (Wanted signal level error for 4.2GHz < f ≦ 6GHz in [3]) | 1.5 |
| MUTestEquipment<4.2 (Uncertainty of the RF signal generator for 3GHz < f ≦ 4.2GHz in [5]) | 0.46 |
| MUTestEquipment>4.2 (Uncertainty of the RF signal generator for 4.2GHz < f ≦ 6GHz in [4]) | 0.58 |
| Combined standard uncertainty (1σ) | 0.98 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) | 1.91 |

Consequently, the test system specific measurement uncertainty values for NR FR1 OTA receiver sensitivity and reference sensitivity tests for operating bands above 4.2GHz can be obtained in Table 3 below.

# Table 3: Test system specific measurement uncertainty values for NR FR1 OTA receiver sensitivity and reference sensitivity tests for operating bands above 4.2GHz

|  | Expanded uncertainty ue [dB] |
| --- | --- |
|  | 4.2GHz < f ≦ 6GHz |
| Indoor Anechoic Chamber | 1.82 |
| Compact Antenna Test Range | 1.91 |
| Common maximum accepted test system uncertainty | 1.9 |

# 3.	Conclusion

This contribution has provided our proposal on the MU for NR FR1 OTA receiver sensitivity and reference sensitivity requirements for operating bands above 4.2GHz, according to the agreed way forward in [1] and our proposals in [2].

Proposal:

Adopt the following table as the MU for NR FR1 OTA receiver sensitivity and reference sensitivity tests.

# Table 3: Test system specific measurement uncertainty values for NR FR1 OTA receiver sensitivity and reference sensitivity tests for operating bands above 4.2GHz

|  | Expanded uncertainty ue [dB] |
| --- | --- |
|  | 4.2GHz < f ≦ 6GHz |
| Indoor Anechoic Chamber | 1.82 |
| Compact Antenna Test Range | 1.91 |
| Common maximum accepted test system uncertainty | 1.9 |

# References

[1]	R4-1809428, “WF on MU of receiver directional requirements”, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, NTT DOCOMO, INC.

[2]	R4-1809697, “Proposals on MU for NR FR1 OTA receiver requirement for operating bands above 4.2GHz”, Nokia, Nokia Shanghai Bell.

[3]	3GPP TS 36.141 v15.3.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing”.

[4]	R4-1805879, “Test Equipment uncertainty values for FR1”, Keysight Technologies.

[5]		3GPP TR 37.842 v13.2.0, “E-UTRA and UTRA; Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)”.