3GPP TSG-RAN WG4 Meeting #88	R4-1809843

# Gothenburg, Sweden, 20th – 24th, August 2018

Agenda item:	7.6.12

Source: 	Intel Corporation

Title: 	On Pulse Shaped Pi/2 BPSK in FR2

Document for:	Approval

# Introduction

Pulse Shaped Pi/2 BPK has been discussed for a few meetings for FR2. In latest RAN plenary meeting [1], the decision about Pi/2 BPSK was optional for FR1 and mandatory for FR2. But MPR requirement for pi/2 BPSK is not finalized yet. In this contribution, we provide our simulation result of MPR for pulse shaped pi/2 BPSK in FR2.

# Discussion

In RAN plenary #80 meeting, the approved WF on RAN4 UE feature list [1] shows pi/2 BPSK is mandatory with capability for FR2.

| WI | # | Feature group | Components | Prerequisite feature groups | Need for gNB to know whether the feature is supported by the UE (what happens if gNB does not know?) | Consequences if the feature is not supported by the UE | Type (See R4-17121 19) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | RAN5 implication | Remarks | Responsible WG | Recommendation for TSG-RAN | TSG-RAN decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1-6 | pi/2-BPSK for PUSCH | 1) pi/2-BPSK for PUSCH |  | Yes | pi/2-BPSK for PUSCH is not possible | Type 4 | No need | Yes |  | RAN4 will define the same minimum requirements for pulse-shaped pi/2 BPSK and non pulse-shaped pi/2 BPSK for FR2. | RAN4 | Optional for FR1 | Optional for FR1 For FR2, mandatory with capability |
|  | 1-7 | pi/2-BPSK for PUCCH format 3/4 | 1) pi/2-BPSK for PUCCH format ¾ |  | Yes | pi/2-BPSK for PUCCH format 3/4 is not possible | Type 4 | No need | Yes |  |  | RAN4 | Optional for FR1 | Optional for FR1 For FR2, mandatory with capability |

Since Tx requirements including EVM equalizer flatness requirement has been already defined for pi/2 PSK with spectrum shaping (aka. Pulse shaping) in FR2 although MPR value is not finalized. In the following session, we provide our simulation results for MPR and our MPR proposal. The simulation was performed based on the Tx requirements of EVM, IBE, ACLR and SEM defined in 38.101-2 for PC3 device.

There are three shaping filters with different roll-off being considered. [0.05, 1, 0.05], [0.1, 1, 0.1] and [0.2, 1, 0.2] and their amplitude roll-offs at the edge RE are -1.7dB, -3.5dB and -7.35dB comparing the peak at the center correspondingly. We select [0.2, 1, 0.2] as maximum roll-off filter by considering EVM equalizer flatness requirements for pi/2 BPSK with spectrum shaping from 38.101-2.

In DFT-S-OFDM waveform generation, DMRS symbols were generated according to 38.211, with maximum of 4 DMRS symbols which is allowed within each slot of 14 symbols. DMRS symbols have higher PAPR than pi/2 BPSK symbols. And also 4 symbols of DMRS within each slot give worst PAPR to the waveform since DMRS symbol has higher PAPR than pi/2 BPSK symbols.

Receiver had no knowledge of pulse shaping coefficients. EVM equalizer equalizing PA distortion was performed by using channel estimation and equalization process.

The resulting MPR values for pulse shaped pi/2 BPSK and QPSK are shown in the table below. Please note, MPR values derived here are based on assumption of PCG is granted. Without PCG, the values are further increased by 0.5dB for pi/2 BPSK and QPSK.

Table 1. MPR values with PCG for pi/2 BPSK w/wo spectrum shaping

| DFT-S-OFDM waveform | MPR with PCG |
| --- | --- |
|  | 50/100/200MHz |
| Pi/2 BPSK [0.2, 1, 0.2] | 0.5 |
| Pi/2 BPSK [0.1, 1, 0.1] | 1.0 |
| Pi/2 BPSK [0.05, 1, 0.05] | 1.5 |
| Pi/2 BPSK | 1.5 |
| QPSK | 1.5 |

The Rx SNR performance was also investigated with 4PRB and 8 PRB allocations with the following setups. The small PRB allocations were analyzed since at cell edge, with limited Tx power, using small PRB allocations to maintain the link SNR is possible.

Simulation setup:

- pi/2-BPSK with code rate 1/3 vs. QPSK with code rate 1/6 (LDPC code)

- 1Tx and 1 Rx

- AWGN channel

- 2 PRB bundling

The following figures show simulation results

Figure 1 BLER curves of pi/2 BPSK w/wo spectrum shaping compared with QPSK

The SNR loss of pulse shaped pi/2 BPSK comparing with QPSK at 10% BLER is summarized in the table below

Table 2 SNR loss of pi/2 BPSK w/wo spectrum shaping comparing with QPSK

|  | SNR loss @ 10% BLER (dB) |  |
| --- | --- | --- |
| DFT-S-OFDM | 4PRB | 8PRB |
| QPSK | 0 | 0 |
| Pi/2 BPSK with no shaping | 0.5 | 0.5 |
| Pi/2 BPSK with [0.05,1, 0.05] | 0.55 | 0.6 |
| Pi/2 BPSK with [0.1, 1, 0.1] | 0.7 | 0.9 |
| Pi/2 BPSK with [0.2, 1, 0.2] | 1.2 | 2.0 |

We also summary the MPR gains, link losses and overall net gains (MPR gain – Link loss gain) in table 3.

Table 3 Net gain of pi/2 BPSK w/wo spectrum shaping comparing with QPSK

| DFT-S-OFDM modulation | Shaping roll-off (dB) | MPR gain (dB) | Link loss (dB) |  | Max net gain (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 50/100/200MHz | 4 RB | 8 RB | 4 RB | 8 RB |
| QPSK | 0 | 0 | 0 | 0 | 0 | 0 |
| Pi/2 BPSK with no shaping | 0 | 0 | 0.5 | 0.5 | -0.5 | -0.5 |
| Pi/2 BPSK with [0.05 1 0.05] | 1.7 | 0 | 0.55 | 0.6 | -0.55 | -0.6 |
| Pi/2 BPSK with [0.1 1 0.1] | 3.5 | 0.5 | 0.7 | 0.9 | -0.2 | -0.4 |
| Pi/2 BPSK with [0.2 1 0.2] | 7.35 | 1 | 1.2 | 2 | -0.2 | -1 |

The net gain of pulse-shaping BPSK is strictly negative even for BPSK without shaping. This is because when QPSK is used with 1/6 coding rate in comparison, the mother LDPC Parity Check Matrix (PCM) shows better coding gain than PCM with 1/3 coding rate. We have following observation.

Observation:  Pi/2 BPSK has no benefit to enhance the cell coverage in FR2.

Since pi/2 BPSK is mandatary in FR2, we propose the MPR values for pi/2 BPSK regardless of being with or without pulse-shaping to be the same with MPR values for QPSK. This also aligns with the proposed MPR values in [2] where showed pi/2 BPSK (shaped or non-shaped) has the same MPR as QPSK and also follows the agreement that “RAN4 will define the same minimum requirements for pulse-shaped pi/2 BPSK and non pulse-shaped pi/2 BPSK for FR2.” in approved RAN4 feature list [1].

Proposal:  Adopt the DFT-S-OFDM QPSK MPR values for pi/2 BPSK which includes both pulse-shaped pi/2 BPSK and non pulse-shaped pi/2 BPSK.

# Conclusion

In this contribution, we provide our MPR simulation results for pi/2 BPSK with and without spectrum shaping and also Rx SNR @ 10% BLER for small RB allocations which are typically used by pi/2 BPSK at the cell edge. The overall performance gain (MPR gain – Rx SNR loss) is negative comparing with QPSK modulation.  Based on our results, we have the following proposal.

Proposal:  Adopt the DFT-S-OFDM QPSK MPR values for pi/2 BPSK which includes both pulse-shaped pi/2 BPSK and non pulse-shaped pi/2 BPSK.

# References

- RP-181472, “WF on remaining parts of RAN4 feature list”, NTT DOCOMO, 3 GPP TSG RAN Meeting #80, La Jolla, USA, June 2018.

- R4-1806609, “Draft CR to 38.101-2: On FR2 MPR single CC” Qualcomm, 3GPP TSG RAN WG4 #87, Busan, Korea, May, 2018.